|************************************************************************

| IBIS file .ibs created by s2ibis2 version 1.1

| North Carolina State University Electronics Research Laboratory  1995

|************************************************************************

|

[IBIS ver]       2.1

[File name]    163631_pf.ibs

[File Rev]       1.3

[Date]           Friday, November 26, 2010

[Source]         Alliance Memory, Inc.

[Notes]          1. Temperatures: typ=25C, min=100C, max=0C 2. Cemos11.5_2.5v

process. 3. Ramp rates based on 50 ohm load.

[Copyright]      Copyright 2010, Alliance Memory, Inc.

(Alliance Memory, Inc. hereby grants the user of this

IBIS model a non-exclusive, nontransferable license to use

this IBIS model under the following terms. The user is granted

this license only to use the IBIS model and is not granted

rights to sell, copy (except as needed to run the IBIS model),

rent, lease or sub-license the IBIS model in whole or in part,

or in modified form to anyone. The User may modify the IBIS

model to suit its specific applications, but rights to

derivative works and such modifications shall belong to Alliance Memory, Inc.

This IBIS model is provided on an AS IS" basis and Alliance Memory, Inc. makes

absolutely no warranty with respect to the information

contained herein. Alliance Memory, Inc. DISCLAIMS AND CUSTOMER WAIVES ALL

WARRANTIES, EXPRESS AND IMPLIED, INCLUDING WARRANTIES OF

MERCHANTABILITY AND FITNESS FOR A PARTICULAR PURPOSE. THE

ENTIRE RISK AS TO QUALITY AND PERFORMANCE IS WITH THE USER

ACCORDINGLY, IN NO EVENT SHALL Alliance Memory, Inc. BE LIABLE FOR ANY DIRECT OR

INDIRECT DAMAGES, WHETHER IN CONTRACT OR TORT, INCLUDING ANY

LOST PROFITS OR OTHER INCIDENTAL, CONSEQUENTIAL, EXEMPLARY, OR

PUNITIVE DAMAGES ARISING OUT OF THE USE OR APPLICATION OF THE

IBIS model. Further, Alliance Memory, Inc. reserves the right to make changes

without notice to any product herein to improve reliability,

function, or design. Alliance Memory, Inc. does not convey any license under

patent rights or any other intellectual property rights,

including those of third parties. Alliance Memory, Inc. is not obligated to

provide maintenance or support for the licensed IBIS model.

|

|************************************************************************

|                           Component AS8C163631_PIPELINE_ZBT_SRAM

|************************************************************************

|

[Component]      AS8C163631

[Manufacturer]   Alliance Memory, Inc.

|                IBIS model for  100-TQFP package

[Package]

| variable       typ                 min                 max

R_pkg            10.00m              9.00m               11.00m

L_pkg            5.50nH              5.00nH              5.60nH

C_pkg            0.40pF              0.30pF              0.47pF

|

[Pin]  signal_name          model_name           R_pin     L_pin     C_pin

1      I/OP3                IBIS_IO

2      I/O16                IBIS_IO

3      I/O17                IBIS_IO

4      VDDQ                 POWER

5      VSS                  GND

6      I/O18                IBIS_IO

7      I/O19                IBIS_IO

8      I/O20                IBIS_IO

9      I/O21                 IBIS_IO

10     VSS                  GND

11     VDDQ                 POWER

12     I/O22                IBIS_IO

13     I/O23                IBIS_IO

14     VDD                  POWER

15     VDD                  POWER

16     VDD                  POWER

17     VSS                  GND

18     I/O24                IBIS_IO

19     I/O25                IBIS_IO

20     VDDQ                 POWER

21     VSS                  GND

22     I/O26                IBIS_IO

23     I/O27                IBIS_IO

24     I/O28                IBIS_IO

25     I/O29                IBIS_IO

26     VSS                  GND

27     VDDQ                 POWER

28     I/O30                IBIS_IO

29     I/O31                IBIS_IO

30     I/OP4                IBIS_IO

31     LBO/                 IBIS_INBUF

32     A5                   IBIS_INBUF

33     A4                   IBIS_INBUF

34     A3                   IBIS_INBUF

35     A2                   IBIS_INBUF

36     A1                   IBIS_INBUF

37     A0                   IBIS_INBUF

38     TMS                  IBIS_INBUF

39     TDI                  IBIS_INBUF

40     VSS                  GND

41     VDD                  POWER

42     TDO                  IBIS_OUT

43     TCK                  IBIS_INBUF

44     A10                  IBIS_INBUF

45     A11                  IBIS_INBUF

46     A12                  IBIS_INBUF

47     A13                  IBIS_INBUF

48     A14                  IBIS_INBUF

49     A15                  IBIS_INBUF

50     A16                  IBIS_INBUF

51     I/OP1                IBIS_IO

52     I/O0                 IBIS_IO

53     I/O1                 IBIS_IO

54     VDDQ                 POWER

55     VSS                  GND

56     I/O2                 IBIS_IO

57     I/O3                 IBIS_IO

58     I/O4                 IBIS_IO

59     I/O5                 IBIS_IO

60     VSS                  GND

61     VDDQ                 POWER

62     I/O6                 IBIS_IO

63     I/O7                 IBIS_IO

64     ZZ                   IBIS_INBUF

65     VDD                  POWER

66     VDD                  POWER

67     VSS                  GND

68     I/O8                 IBIS_IO

69     I/O9                 IBIS_IO

70     VDDQ                 POWER

71     VSS                  GND

72     I/O10                IBIS_IO

73     I/O11                IBIS_IO

74     I/O12                IBIS_IO

75     I/O13                IBIS_IO

76     VSS                  GND

77     VDDQ                 POWER

78     I/O14                IBIS_IO

79     I/O15                IBIS_IO

80     I/OP2                IBIS_IO

81     A9                   IBIS_INBUF

82     A8                   IBIS_INBUF

83     A17                  IBIS_INBUF

84     A18                  IBIS_INBUF

85     ADV/LD/              IBIS_INBUF

86     OE/                  IBIS_INBUF

87     CEN/                 IBIS_INBUF

88     R/W/                 IBIS_INBUF

89     CLK                  IBIS_INBUF

90     VSS                  GND

91     VDD                  POWER

92     CE2/                 IBIS_INBUF

93     BW1/                 IBIS_INBUF

94     BW2/                 IBIS_INBUF

95     BW3/                 IBIS_INBUF

96     BW4/                 IBIS_INBUF

97     CE2                  IBIS_INBUF

98     CE1/                 IBIS_INBUF

99     A7                   IBIS_INBUF

100    A6                   IBIS_INBUF

| in   in                   dummy_model

| oel  oel                  dummy_model

| int  int                  dummy_model

| oelt oelt                 dummy_model

|

|************************************************************************

|                             Model IBIS_IO

|************************************************************************

|

[Model]          IBIS_IO

Model_type       I/O

Polarity         Non-Inverting

Enable           Active-Low

Vinl =  0.700V

Vinh =  1.700V

Vmeas =  1.250V

Cref =  0.000F

Rref = 50.000

Vref =  1.250V

C_comp           3.00pF              2.00pF              4.00pF

|

|

[Temperature Range]       25.00               100.00              0.000

[Voltage Range]           2.50V               2.37V               2.62V

[Pulldown]

| voltage     I(typ)              I(min)              I(max)

|

-2.50      -0.117A            -89.700mA            -0.133A

-2.30      -0.113A            -85.600mA            -0.128A

-2.10      -0.108A            -81.500mA            -0.124A

-1.90      -0.104A            -77.500mA            -0.119A

-1.70     -99.400mA           -73.200mA            -0.115A

-1.50     -94.300mA           -68.590mA            -0.109A

-1.30     -88.180mA           -63.420mA            -0.103A

-1.10     -81.030mA           -57.450mA           -94.920mA

-0.90     -72.130mA           -50.400mA           -85.160mA

-0.70     -60.877mA           -41.933mA           -72.530mA

-0.60     -54.071mA           -36.983mA           -64.784mA

-0.50     -46.260mA           -31.428mA           -55.780mA

-0.40     -37.635mA           -25.387mA           -45.603mA

-0.30     -28.607mA           -19.131mA           -34.816mA

-0.20     -19.300mA           -12.779mA           -23.600mA

-0.10      -9.754mA            -6.392mA           -11.980mA

0.00      20.940pA             0.000A             93.150pA

0.10       9.359mA             6.091mA            11.580mA

0.20      17.790mA            11.620mA            22.040mA

0.30      25.330mA            16.620mA            31.450mA

0.40      32.030mA            21.090mA            39.840mA

0.50      37.910mA            25.050mA            47.270mA

0.60      43.030mA            28.520mA            53.780mA

0.70      47.410mA            31.520mA            59.410mA

0.80      51.070mA            34.050mA            64.180mA

0.90      54.030mA            36.140mA            68.110mA

1.00      56.330mA            37.800mA            71.230mA

1.10      58.020mA            39.060mA            73.580mA

1.20      59.210mA            39.980mA            75.260mA

1.30      60.020mA            40.630mA            76.420mA

1.40      60.590mA            41.080mA            77.220mA

1.50      61.000mA            41.410mA            77.800mA

1.60      61.300mA            41.650mA            78.220mA

1.70      61.540mA            41.840mA            78.540mA

1.80      61.720mA            41.990mA            78.790mA

1.90      61.880mA            42.110mA            79.000mA

2.00      62.010mA            42.210mA            79.170mA

2.10      62.120mA            42.290mA            79.320mA

2.20      62.220mA            42.370mA            79.440mA

2.30      62.300mA            42.440mA            79.560mA

2.40      62.380mA            42.490mA            79.660mA

2.50      62.450mA            42.550mA            79.750mA

2.60      62.520mA            42.600mA            79.830mA

2.70      62.580mA            42.650mA            79.910mA

2.80      62.640mA            42.770mA            79.980mA

2.90      62.730mA            43.190mA            80.060mA

3.00      63.110mA            44.290mA            80.160mA

3.10      64.340mA            46.130mA            80.580mA

3.20      66.610mA            48.580mA            82.050mA

3.30      69.730mA            51.490mA            84.800mA

3.40      73.520mA            54.790mA            88.570mA

3.50      77.880mA            58.430mA            93.160mA

3.60      82.710mA            62.350mA            98.450mA

3.70      87.980mA            66.520mA             0.104A

3.80      93.620mA            70.930mA             0.111A

4.00       0.106A             80.390mA             0.125A

4.20       0.120A             90.710mA             0.142A

4.40       0.136A              0.102A              0.161A

4.60       0.153A              0.114A              0.184A

4.80       0.174A              0.128A              0.211A

5.00       0.198A              0.144A              0.243A

|

[Pullup]

| voltage     I(typ)              I(min)              I(max)

|

-2.50       0.127A              0.107A              0.142A

-2.30       0.126A              0.105A              0.142A

-2.10       0.124A              0.101A              0.140A

-1.90       0.120A             96.480mA             0.135A

-1.70       0.114A             90.610mA             0.129A

-1.50       0.107A             84.100mA             0.122A

-1.30      99.090mA            76.620mA             0.113A

-1.10      89.310mA            68.240mA             0.103A

-0.90      77.880mA            58.752mA            90.050mA

-0.70      64.193mA            47.867mA            74.874mA

-0.60      56.265mA            41.708mA            66.020mA

-0.50      47.428mA            34.960mA            56.041mA

-0.40      37.889mA            27.784mA            45.021mA

-0.30      28.177mA            20.545mA            33.616mA

-0.20      18.560mA            13.449mA            22.240mA

-0.10       9.145mA             6.584mA            11.000mA

0.00       0.100nA             0.000A              0.000A

0.10      -8.671mA            -6.200mA           -10.510mA

0.20     -16.740mA           -11.970mA           -20.330mA

0.30     -24.210mA           -17.310mA           -29.470mA

0.40     -31.110mA           -22.220mA           -37.950mA

0.50     -37.420mA           -26.720mA           -45.780mA

0.60     -43.180mA           -30.800mA           -52.960mA

0.70     -48.370mA           -34.480mA           -59.510mA

0.80     -53.020mA           -37.750mA           -65.440mA

0.90     -57.140mA           -40.630mA           -70.760mA

1.00     -60.720mA           -43.110mA           -75.480mA

1.10     -63.790mA           -45.190mA           -79.600mA

1.20     -66.340mA           -46.900mA           -83.140mA

1.30     -68.380mA           -48.220mA           -86.100mA

1.40     -69.870mA           -49.180mA           -88.440mA

1.50     -70.870mA           -49.840mA           -90.080mA

1.60     -71.560mA           -50.320mA           -91.160mA

1.70     -72.070mA           -50.690mA           -91.930mA

1.80     -72.470mA           -50.980mA           -92.510mA

1.90     -72.790mA           -51.220mA           -92.970mA

2.00     -73.060mA           -51.430mA           -93.340mA

2.10     -73.290mA           -51.600mA           -93.650mA

2.20     -73.490mA           -51.750mA           -93.920mA

2.30     -73.660mA           -51.890mA           -94.150mA

2.40     -73.820mA           -52.010mA           -94.350mA

2.50     -73.960mA           -52.120mA           -94.540mA

2.60     -74.090mA           -52.220mA           -94.700mA

2.70     -74.210mA           -52.330mA           -94.860mA

2.80     -74.330mA           -52.580mA           -95.000mA

2.90     -74.500mA           -53.660mA           -95.140mA

3.00     -75.270mA           -56.650mA           -95.310mA

3.10     -78.390mA           -61.590mA           -96.060mA

3.20     -84.550mA           -67.990mA           -99.540mA

3.30     -93.000mA           -75.500mA            -0.107A

3.40      -0.103A            -83.880mA            -0.117A

3.50      -0.114A            -92.990mA            -0.129A

3.60      -0.127A             -0.103A             -0.143A

3.70      -0.140A             -0.113A             -0.157A

3.80      -0.153A             -0.123A             -0.173A

4.00      -0.182A             -0.146A             -0.206A

4.20      -0.212A             -0.169A             -0.240A

4.40      -0.244A             -0.194A             -0.276A

4.60      -0.276A             -0.219A             -0.314A

4.80      -0.310A             -0.246A             -0.353A

5.00      -0.344A             -0.273A             -0.392A

|

[GND_clamp]

| voltage     I(typ)              I(min)              I(max)

|

-2.50      -0.256A             -0.196A             -0.298A

-2.40      -0.241A             -0.184A             -0.281A

-2.30      -0.225A             -0.172A             -0.264A

-2.20      -0.210A             -0.159A             -0.247A

-2.10      -0.195A             -0.147A             -0.230A

-2.00      -0.180A             -0.135A             -0.213A

-1.90      -0.165A             -0.124A             -0.196A

-1.80      -0.150A             -0.112A             -0.179A

-1.70      -0.135A             -0.101A             -0.162A

-1.60      -0.120A            -89.330mA            -0.145A

-1.50      -0.106A            -78.310mA            -0.129A

-1.40     -91.800mA           -67.570mA            -0.112A

-1.30     -77.920mA           -57.180mA           -96.090mA

-1.20     -64.460mA           -47.200mA           -80.220mA

-1.10     -51.570mA           -37.730mA           -64.880mA

-1.00     -39.400mA           -28.870mA           -50.230mA

-0.90     -28.170mA           -20.770mA           -36.540mA

-0.80     -18.180mA           -13.580mA           -24.150mA

-0.70      -9.823mA            -7.577mA           -13.540mA

-0.60      -3.759mA            -3.157mA            -5.506mA

-0.50      -0.740mA            -0.782mA            -1.150mA

-0.40     -64.570uA            -0.103mA           -96.710uA

-0.30      -2.842uA            -8.572uA            -3.594uA

-0.20     -89.550nA            -0.503uA           -82.610nA

-0.10     -11.260nA           -30.580nA           -10.770nA

0.00     -42.420pA            -0.758nA           -26.070pA

0.10       9.978nA            10.010nA             9.982nA

0.20      19.980nA            20.110nA            19.980nA

0.30      29.980nA            30.160nA            29.980nA

0.40      39.980nA            40.190nA            39.990nA

0.50      49.990nA            50.230nA            49.990nA

0.60      59.990nA            60.260nA            59.990nA

0.70      69.990nA            70.300nA            69.990nA

0.80      79.990nA            80.330nA            79.990nA

0.90      89.990nA            90.360nA            89.990nA

1.00      99.990nA             0.100uA            99.990nA

1.10       0.110uA             0.110uA             0.110uA

1.20       0.120uA             0.120uA             0.120uA

1.30       0.130uA             0.130uA             0.130uA

1.40       0.140uA             0.140uA             0.140uA

1.50       0.150uA             0.151uA             0.150uA

1.60       0.160uA             0.161uA             0.160uA

1.70       0.170uA             0.171uA             0.170uA

1.80       0.180uA             0.181uA             0.180uA

1.90       0.190uA             0.191uA             0.190uA

2.00       0.200uA             0.201uA             0.200uA

2.10       0.210uA             0.211uA             0.210uA

2.20       0.220uA             0.221uA             0.220uA

2.30       0.230uA             0.231uA             0.230uA

2.40       0.240uA             0.242uA             0.240uA

2.50       0.250uA             0.281uA             0.250uA

|

[POWER_clamp]

| voltage     I(typ)              I(min)              I(max)

|

-2.50       0.100A             78.580mA             0.117A

-2.40      94.350mA            73.670mA             0.110A

-2.30      88.360mA            68.760mA             0.104A

-2.20      82.350mA            63.860mA            96.870mA

-2.10      76.310mA            58.980mA            90.080mA

-2.00      70.270mA            54.130mA            83.250mA

-1.90      64.240mA            49.320mA            76.410mA

-1.80      58.240mA            44.570mA            69.560mA

-1.70      52.280mA            39.890mA            62.740mA

-1.60      46.400mA            35.290mA            55.980mA

-1.50      40.620mA            30.800mA            49.290mA

-1.40      34.980mA            26.450mA            42.720mA

-1.30      29.510mA            22.260mA            36.310mA

-1.20      24.260mA            18.260mA            30.120mA

-1.10      19.290mA            14.490mA            24.210mA

-1.00      14.660mA            11.000mA            18.650mA

-0.90      10.450mA             7.838mA            13.550mA

-0.80       6.770mA             5.078mA             9.014mA

-0.70       3.737mA             2.813mA             5.196mA

-0.60       1.535mA             1.182mA             2.300mA

-0.50       0.362mA             0.310mA             0.599mA

-0.40      41.180uA            46.020uA            69.440uA

-0.30       2.773uA             4.670uA             4.275uA

-0.20       0.349uA             0.546uA             0.382uA

-0.10       0.262uA             0.263uA             0.274uA

0.00       0.250uA             0.239uA             0.262uA

|

[Ramp]

| variable       typ                 min                 max

dV/dt_r          1.18/0.30n          1.02/0.49n          1.30/0.22n

dV/dt_f          1.18/0.28n          0.98/0.50n          1.31/0.21n

R_load = 50.00

|

[Rising Waveform]

R_fixture = 50.00

V_fixture = 0.000

V_fixture_min = 0.000

V_fixture_max = 0.000

C_fixture = 0.000F

R_dut = 68.00m

L_dut = 2.31nH

C_dut = 0.65pF

| time           V(typ)              V(min)              V(max)

|

0.000S         0.000V              0.000V              0.000V

1.000e-10S       0.000V             -2.224uV             0.507uV

0.200nS       -0.665mV           -19.140uV           -10.080mV

0.300nS      -45.680mV            -3.202mV            -0.111V

0.400nS       -0.121V            -43.620mV           -25.430mV

0.500nS       -8.692mV           -91.970mV             0.351V

0.600nS        0.270V            -62.670mV             0.923V

0.700nS        0.627V             26.440mV             1.537V

0.800nS        1.032V              0.157V              1.917V

0.900nS        1.424V              0.343V              2.074V

1.000nS        1.689V              0.528V              2.095V

1.100nS        1.819V              0.763V              2.119V

1.200nS        1.896V              0.988V              2.135V

1.300nS        1.917V              1.192V              2.141V

1.400nS        1.931V              1.357V              2.148V

1.500nS        1.945V              1.482V              2.154V

1.600nS        1.954V              1.551V              2.156V

1.700nS        1.958V              1.593V              2.156V

1.800nS        1.960V              1.618V              2.156V

1.900nS        1.962V              1.637V              2.156V

2.000nS        1.963V              1.656V              2.156V

2.100nS        1.964V              1.665V              2.156V

2.200nS        1.965V              1.671V              2.156V

2.300nS        1.966V              1.676V              2.156V

2.400nS        1.965V              1.678V              2.156V

2.500nS        1.965V              1.681V              2.156V

2.600nS        1.965V              1.684V              2.156V

2.700nS        1.965V              1.687V              2.156V

2.800nS        1.965V              1.689V              2.156V

2.900nS        1.965V              1.689V              2.156V

3.000nS        1.965V              1.690V              2.156V

3.100nS        1.965V              1.690V              2.156V

3.200nS        1.965V              1.690V              2.156V

3.300nS        1.965V              1.691V              2.156V

3.400nS        1.965V              1.691V              2.156V

3.500nS        1.965V              1.691V              2.156V

3.600nS        1.965V              1.691V              2.156V

3.700nS        1.965V              1.691V              2.156V

3.800nS        1.965V              1.691V              2.156V

3.900nS        1.965V              1.691V              2.156V

4.000nS        1.965V              1.691V              2.156V

4.100nS        1.965V              1.691V              2.156V

4.200nS        1.965V              1.691V              2.156V

4.300nS        1.965V              1.691V              2.156V

4.400nS        1.965V              1.691V              2.156V

4.500nS        1.965V              1.691V              2.156V

4.600nS        1.965V              1.691V              2.156V

4.700nS        1.965V              1.691V              2.156V

4.800nS        1.965V              1.691V              2.156V

4.900nS        1.965V              1.691V              2.156V

5.000nS        1.965V              1.691V              2.156V

5.100nS        1.965V              1.691V              2.156V

5.200nS        1.965V              1.691V              2.156V

5.300nS        1.965V              1.691V              2.156V

5.400nS        1.965V              1.691V              2.156V

5.500nS        1.965V              1.691V              2.156V

5.600nS        1.965V              1.691V              2.156V

5.700nS        1.965V              1.691V              2.156V

5.800nS        1.965V              1.691V              2.156V

5.900nS        1.965V              1.691V              2.156V

6.000nS        1.965V              1.691V              2.156V

6.100nS        1.965V              1.691V              2.156V

6.200nS        1.965V              1.691V              2.156V

6.300nS        1.965V              1.691V              2.156V

6.400nS        1.965V              1.691V              2.156V

6.500nS        1.965V              1.691V              2.156V

6.600nS        1.965V              1.691V              2.156V

6.700nS        1.965V              1.691V              2.156V

6.800nS        1.965V              1.691V              2.156V

6.900nS        1.965V              1.691V              2.156V

7.000nS        1.965V              1.691V              2.156V

7.100nS        1.965V              1.691V              2.156V

7.200nS        1.965V              1.691V              2.156V

7.300nS        1.965V              1.691V              2.156V

7.400nS        1.965V              1.691V              2.156V

7.500nS        1.965V              1.691V              2.156V

7.600nS        1.965V              1.691V              2.156V

7.700nS        1.965V              1.691V              2.156V

7.800nS        1.965V              1.691V              2.156V

7.900nS        1.965V              1.691V              2.156V

8.000nS        1.965V              1.691V              2.156V

8.100nS        1.965V              1.691V              2.156V

8.200nS        1.965V              1.691V              2.156V

8.300nS        1.965V              1.691V              2.156V

8.400nS        1.965V              1.691V              2.156V

8.500nS        1.965V              1.691V              2.156V

8.600nS        1.965V              1.691V              2.156V

8.700nS        1.965V              1.691V              2.156V

8.800nS        1.965V              1.691V              2.156V

8.900nS        1.965V              1.691V              2.156V

9.000nS        1.965V              1.691V              2.156V

9.100nS        1.965V              1.691V              2.156V

9.200nS        1.965V              1.691V              2.156V

9.300nS        1.965V              1.691V              2.156V

9.400nS        1.965V              1.691V              2.156V

9.500nS        1.965V              1.691V              2.156V

9.600nS        1.965V              1.691V              2.156V

9.700nS        1.965V              1.691V              2.156V

9.800nS        1.965V              1.691V              2.156V

9.900nS        1.965V              1.691V              2.156V

|

[Rising Waveform]

R_fixture = 50.00

V_fixture = 2.50

V_fixture_min = 2.37

V_fixture_max = 2.62

C_fixture = 0.000F

R_dut = 68.00m

L_dut = 2.31nH

C_dut = 0.65pF

| time           V(typ)              V(min)              V(max)

|

0.000S         0.530V              0.744V              0.450V

1.000e-10S       0.530V              0.744V              0.450V

0.200nS        0.530V              0.744V              0.453V

0.300nS        0.777V              0.750V              1.343V

0.400nS        2.017V              1.090V              2.547V

0.500nS        2.397V              1.859V              2.552V

0.600nS        2.407V              2.239V              2.557V

0.700nS        2.440V              2.298V              2.583V

0.800nS        2.460V              2.318V              2.595V

0.900nS        2.471V              2.328V              2.602V

1.000nS        2.475V              2.338V              2.608V

1.100nS        2.478V              2.342V              2.612V

1.200nS        2.482V              2.345V              2.616V

1.300nS        2.485V              2.349V              2.620V

1.400nS        2.489V              2.352V              2.623V

1.500nS        2.494V              2.355V              2.624V

1.600nS        2.496V              2.359V              2.624V

1.700nS        2.497V              2.363V              2.624V

1.800nS        2.498V              2.366V              2.625V

1.900nS        2.499V              2.370V              2.625V

2.000nS        2.499V              2.373V              2.625V

2.100nS        2.500V              2.373V              2.625V

2.200nS        2.500V              2.373V              2.625V

2.300nS        2.500V              2.373V              2.625V

2.400nS        2.500V              2.373V              2.624V

2.500nS        2.499V              2.373V              2.624V

2.600nS        2.499V              2.374V              2.625V

2.700nS        2.499V              2.374V              2.625V

2.800nS        2.499V              2.374V              2.625V

2.900nS        2.500V              2.374V              2.625V

3.000nS        2.500V              2.374V              2.625V

3.100nS        2.500V              2.375V              2.625V

3.200nS        2.500V              2.375V              2.625V

3.300nS        2.500V              2.375V              2.625V

3.400nS        2.500V              2.375V              2.624V

3.500nS        2.499V              2.375V              2.625V

3.600nS        2.499V              2.374V              2.625V

3.700nS        2.499V              2.374V              2.625V

3.800nS        2.499V              2.374V              2.625V

3.900nS        2.500V              2.374V              2.625V

4.000nS        2.500V              2.375V              2.625V

4.100nS        2.500V              2.375V              2.625V

4.200nS        2.500V              2.375V              2.625V

4.300nS        2.500V              2.375V              2.625V

4.400nS        2.500V              2.375V              2.625V

4.500nS        2.499V              2.375V              2.625V

4.600nS        2.499V              2.375V              2.625V

4.700nS        2.499V              2.374V              2.625V

4.800nS        2.500V              2.374V              2.625V

4.900nS        2.500V              2.375V              2.625V

5.000nS        2.500V              2.375V              2.625V

5.100nS        2.500V              2.375V              2.625V

5.200nS        2.500V              2.375V              2.625V

5.300nS        2.500V              2.375V              2.625V

5.400nS        2.500V              2.375V              2.625V

5.500nS        2.500V              2.375V              2.625V

5.600nS        2.499V              2.375V              2.625V

5.700nS        2.500V              2.375V              2.625V

5.800nS        2.500V              2.375V              2.625V

5.900nS        2.500V              2.375V              2.625V

6.000nS        2.500V              2.375V              2.625V

6.100nS        2.500V              2.375V              2.625V

6.200nS        2.500V              2.375V              2.625V

6.300nS        2.500V              2.375V              2.625V

6.400nS        2.500V              2.375V              2.625V

6.500nS        2.500V              2.375V              2.625V

6.600nS        2.500V              2.375V              2.625V

6.700nS        2.500V              2.375V              2.625V

6.800nS        2.500V              2.375V              2.625V

6.900nS        2.500V              2.375V              2.625V

7.000nS        2.500V              2.375V              2.625V

7.100nS        2.500V              2.375V              2.625V

7.200nS        2.500V              2.375V              2.625V

7.300nS        2.500V              2.375V              2.625V

7.400nS        2.500V              2.375V              2.625V

7.500nS        2.500V              2.375V              2.625V

7.600nS        2.500V              2.375V              2.625V

7.700nS        2.500V              2.375V              2.625V

7.800nS        2.500V              2.375V              2.625V

7.900nS        2.500V              2.375V              2.625V

8.000nS        2.500V              2.375V              2.625V

8.100nS        2.500V              2.375V              2.625V

8.200nS        2.500V              2.375V              2.625V

8.300nS        2.500V              2.375V              2.625V

8.400nS        2.500V              2.375V              2.625V

8.500nS        2.500V              2.375V              2.625V

8.600nS        2.500V              2.375V              2.625V

8.700nS        2.500V              2.375V              2.625V

8.800nS        2.500V              2.375V              2.625V

8.900nS        2.500V              2.375V              2.625V

9.000nS        2.500V              2.375V              2.625V

9.100nS        2.500V              2.375V              2.625V

9.200nS        2.500V              2.375V              2.625V

9.300nS        2.500V              2.375V              2.625V

9.400nS        2.500V              2.375V              2.625V

9.500nS        2.500V              2.375V              2.625V

9.600nS        2.500V              2.375V              2.625V

9.700nS        2.500V              2.375V              2.625V

9.800nS        2.500V              2.375V              2.625V

9.900nS        2.500V              2.375V              2.625V

|

[Falling Waveform]

R_fixture = 50.00

V_fixture = 0.000

V_fixture_min = 0.000

V_fixture_max = 0.000

C_fixture = 0.000F

R_dut = 68.00m

L_dut = 2.31nH

C_dut = 0.65pF

| time           V(typ)              V(min)              V(max)

|

0.000S         1.965V              1.691V              2.156V

1.000e-10S       1.965V              1.691V              2.156V

0.200nS        1.965V              1.691V              2.156V

0.300nS        1.966V              1.691V              2.116V

0.400nS        1.694V              1.689V              0.939V

0.500nS        0.556V              1.541V             41.850mV

0.600nS       40.070mV             0.844V              1.621mV

0.700nS       25.660mV             0.246V             17.700mV

0.800nS       17.720mV            84.790mV            10.160mV

0.900nS       10.790mV            38.050mV             5.693mV

1.000nS        6.480mV            15.890mV             3.136mV

1.100nS        4.388mV            11.180mV             0.579mV

1.200nS        3.324mV             8.730mV             0.236mV

1.300nS        2.261mV             6.276mV             0.472mV

1.400nS        1.197mV             4.494mV             0.417mV

1.500nS        0.683mV             3.866mV             0.324mV

1.600nS        0.539mV             3.238mV             0.231mV

1.700nS        0.379mV             2.609mV             0.137mV

1.800nS        0.298mV             1.981mV            44.140uV

1.900nS        0.218mV             1.379mV           -27.800uV

2.000nS        0.137mV             0.860mV           -26.870uV

2.100nS       56.660uV             0.636mV           -25.950uV

2.200nS      -16.130uV             0.518mV           -25.030uV

2.300nS      -10.830uV             0.415mV           -24.110uV

2.400nS       -5.533uV             0.313mV           -20.910uV

2.500nS       -0.236uV             0.210mV            -9.925uV

2.600nS        5.061uV             0.107mV             1.057uV

2.700nS        9.569uV            82.720uV            12.040uV

2.800nS        6.171uV            69.900uV            23.020uV

2.900nS        2.773uV            57.070uV            28.650uV

3.000nS       -0.625uV            44.250uV            15.990uV

3.100nS       -4.023uV            31.420uV             3.320uV

3.200nS       -6.917uV            23.430uV            -9.347uV

3.300nS       -4.760uV            16.160uV           -22.010uV

3.400nS       -2.603uV             8.899uV           -29.020uV

3.500nS       -0.446uV             1.636uV           -16.670uV

3.600nS        1.711uV            -5.627uV            -4.325uV

3.700nS        3.581uV            -3.397uV             8.023uV

3.800nS        2.566uV             0.273uV            20.370uV

3.900nS        1.551uV             3.942uV            27.380uV

4.000nS        0.536uV             7.612uV            16.150uV

4.100nS       -0.479uV            11.280uV             4.915uV

4.200nS       -1.390uV             8.460uV            -6.319uV

4.300nS       -1.250uV             4.656uV           -17.550uV

4.400nS       -1.110uV             0.852uV           -24.060uV

4.500nS       -0.970uV            -2.951uV           -14.390uV

4.600nS       -0.830uV            -6.755uV            -4.718uV

4.700nS       -0.658uV            -4.467uV             4.953uV

4.800nS       -0.171uV            -1.256uV            14.620uV

4.900nS        0.315uV             1.955uV            20.320uV

5.000nS        0.802uV             5.167uV            12.420uV

5.100nS        1.289uV             8.378uV             4.522uV

5.200nS        1.651uV             6.393uV            -3.377uV

5.300nS        0.755uV             3.620uV           -11.280uV

5.400nS       -0.142uV             0.848uV           -16.010uV

5.500nS       -1.038uV            -1.925uV            -9.903uV

5.600nS       -1.934uV            -4.698uV            -3.800uV

5.700nS       -2.648uV            -3.078uV             2.303uV

5.800nS       -1.523uV            -0.793uV             8.407uV

5.900nS       -0.398uV             1.492uV            12.130uV

6.000nS        0.726uV             3.777uV             7.714uV

6.100nS        1.851uV             6.062uV             3.298uV

6.200nS        2.764uV             4.804uV            -1.118uV

6.300nS        1.554uV             3.009uV            -5.534uV

6.400nS        0.345uV             1.215uV            -8.290uV

6.500nS       -0.865uV            -0.580uV            -5.368uV

6.600nS       -2.075uV            -2.375uV            -2.446uV

6.700nS       -3.067uV            -1.453uV             0.476uV

6.800nS       -1.879uV            -0.119uV             3.398uV

6.900nS       -0.691uV             1.214uV             5.282uV

7.000nS        0.496uV             2.548uV             3.614uV

7.100nS        1.684uV             3.881uV             1.947uV

7.200nS        2.666uV             3.256uV             0.279uV

7.300nS        1.573uV             2.334uV            -1.389uV

7.400nS        0.481uV             1.412uV            -2.528uV

7.500nS       -0.611uV             0.490uV            -1.859uV

7.600nS       -1.703uV            -0.432uV            -1.190uV

7.700nS       -2.610uV           -57.000nV            -0.521uV

7.800nS       -1.659uV             0.515uV             0.148uV

7.900nS       -0.708uV             1.087uV             0.684uV

8.000nS        0.242uV             1.659uV             0.764uV

8.100nS        1.193uV             2.231uV             0.845uV

8.200nS        1.987uV             2.056uV             0.925uV

8.300nS        1.200uV             1.769uV             1.005uV

8.400nS        0.413uV             1.482uV             0.931uV

8.500nS       -0.374uV             1.194uV             0.329uV

8.600nS       -1.161uV             0.907uV            -0.273uV

8.700nS       -1.820uV             0.928uV            -0.875uV

8.800nS       -1.202uV             0.995uV            -1.477uV

8.900nS       -0.584uV             1.062uV            -1.733uV

9.000nS       33.500nV             1.129uV            -0.806uV

9.100nS        0.651uV             1.196uV             0.122uV

9.200nS        1.172uV             1.286uV             1.050uV

9.300nS        0.715uV             1.380uV             1.977uV

9.400nS        0.258uV             1.473uV             2.448uV

9.500nS       -0.199uV             1.566uV             1.355uV

9.600nS       -0.656uV             1.660uV             0.263uV

9.700nS       -1.040uV             1.472uV            -0.829uV

9.800nS       -0.694uV             1.241uV            -1.922uV

9.900nS       -0.347uV             1.011uV            -1.870uV

|

[Falling Waveform]

R_fixture = 50.00

V_fixture = 2.50

V_fixture_min = 2.37

V_fixture_max = 2.62

C_fixture = 0.000F

R_dut = 68.00m

L_dut = 2.31nH

C_dut = 0.65pF

| time           V(typ)              V(min)              V(max)

|

0.000S         2.500V              2.375V              2.625V

1.000e-10S       2.500V              2.375V              2.626V

0.200nS        2.498V              2.376V              2.598V

0.300nS        2.476V              2.372V              2.628V

0.400nS        2.520V              2.368V              2.623V

0.500nS        2.481V              2.379V              2.190V

0.600nS        2.246V              2.381V              1.503V

0.700nS        1.808V              2.354V              0.867V

0.800nS        1.349V              2.225V              0.580V

0.900nS        0.977V              2.072V              0.516V

1.000nS        0.730V              1.830V              0.492V

1.100nS        0.634V              1.601V              0.478V

1.200nS        0.593V              1.395V              0.465V

1.300nS        0.573V              1.214V              0.458V

1.400nS        0.557V              1.068V              0.456V

1.500nS        0.551V              0.961V              0.454V

1.600nS        0.546V              0.887V              0.454V

1.700nS        0.540V              0.847V              0.453V

1.800nS        0.534V              0.819V              0.452V

1.900nS        0.530V              0.796V              0.451V

2.000nS        0.530V              0.781V              0.450V

2.100nS        0.530V              0.773V              0.450V

2.200nS        0.530V              0.767V              0.450V

2.300nS        0.531V              0.762V              0.450V

2.400nS        0.531V              0.757V              0.450V

2.500nS        0.530V              0.752V              0.450V

2.600nS        0.530V              0.750V              0.450V

2.700nS        0.530V              0.749V              0.450V

2.800nS        0.530V              0.748V              0.450V

2.900nS        0.530V              0.746V              0.451V

3.000nS        0.530V              0.745V              0.451V

3.100nS        0.530V              0.745V              0.451V

3.200nS        0.530V              0.745V              0.450V

3.300nS        0.530V              0.744V              0.450V

3.400nS        0.530V              0.744V              0.450V

3.500nS        0.530V              0.744V              0.450V

3.600nS        0.530V              0.744V              0.450V

3.700nS        0.530V              0.744V              0.450V

3.800nS        0.530V              0.744V              0.450V

3.900nS        0.530V              0.744V              0.450V

4.000nS        0.530V              0.744V              0.450V

4.100nS        0.530V              0.744V              0.450V

4.200nS        0.530V              0.744V              0.450V

4.300nS        0.530V              0.744V              0.450V

4.400nS        0.530V              0.744V              0.450V

4.500nS        0.530V              0.744V              0.450V

4.600nS        0.530V              0.744V              0.450V

4.700nS        0.530V              0.744V              0.450V

4.800nS        0.530V              0.744V              0.450V

4.900nS        0.530V              0.744V              0.450V

5.000nS        0.530V              0.744V              0.450V

5.100nS        0.530V              0.744V              0.450V

5.200nS        0.530V              0.744V              0.450V

5.300nS        0.530V              0.744V              0.450V

5.400nS        0.530V              0.744V              0.450V

5.500nS        0.530V              0.744V              0.450V

5.600nS        0.530V              0.744V              0.450V

5.700nS        0.530V              0.744V              0.450V

5.800nS        0.530V              0.744V              0.450V

5.900nS        0.530V              0.744V              0.450V

6.000nS        0.530V              0.744V              0.450V

6.100nS        0.530V              0.744V              0.450V

6.200nS        0.530V              0.744V              0.450V

6.300nS        0.530V              0.744V              0.450V

6.400nS        0.530V              0.744V              0.450V

6.500nS        0.530V              0.744V              0.450V

6.600nS        0.530V              0.744V              0.450V

6.700nS        0.530V              0.744V              0.450V

6.800nS        0.530V              0.744V              0.450V

6.900nS        0.530V              0.744V              0.450V

7.000nS        0.530V              0.744V              0.450V

7.100nS        0.530V              0.744V              0.450V

7.200nS        0.530V              0.744V              0.450V

7.300nS        0.530V              0.744V              0.450V

7.400nS        0.530V              0.744V              0.450V

7.500nS        0.530V              0.744V              0.450V

7.600nS        0.530V              0.744V              0.450V

7.700nS        0.530V              0.744V              0.450V

7.800nS        0.530V              0.744V              0.450V

7.900nS        0.530V              0.744V              0.450V

8.000nS        0.530V              0.744V              0.450V

8.100nS        0.530V              0.744V              0.450V

8.200nS        0.530V              0.744V              0.450V

8.300nS        0.530V              0.744V              0.450V

8.400nS        0.530V              0.744V              0.450V

8.500nS        0.530V              0.744V              0.450V

8.600nS        0.530V              0.744V              0.450V

8.700nS        0.530V              0.744V              0.450V

8.800nS        0.530V              0.744V              0.450V

8.900nS        0.530V              0.744V              0.450V

9.000nS        0.530V              0.744V              0.450V

9.100nS        0.530V              0.744V              0.450V

9.200nS        0.530V              0.744V              0.450V

9.300nS        0.530V              0.744V              0.450V

9.400nS        0.530V              0.744V              0.450V

9.500nS        0.530V              0.744V              0.450V

9.600nS        0.530V              0.744V              0.450V

9.700nS        0.530V              0.744V              0.450V

9.800nS        0.530V              0.744V              0.450V

9.900nS        0.530V              0.744V              0.450V

|

| End [Model] IBIS_IO

|

|************************************************************************

|                             Model IBIS_OUT

|************************************************************************

|

[Model]          IBIS_OUT

Model_type       Output

Polarity         Non-Inverting

Enable           Active-Low

Vmeas =  1.250V

Cref =  0.000F

Rref = 50.000

Vref =  1.250V

C_comp           3.00pF              2.00pF              4.00pF

|

|

[Temperature Range]       25.00               100.00              0.000

[Voltage Range]           2.50V               2.37V               2.62V

[Pulldown]

| voltage     I(typ)              I(min)              I(max)

|

-2.50      -0.117A            -89.700mA            -0.133A

-2.30      -0.113A            -85.600mA            -0.128A

-2.10      -0.108A            -81.500mA            -0.124A

-1.90      -0.104A            -77.500mA            -0.119A

-1.70     -99.400mA           -73.200mA            -0.114A

-1.50     -94.200mA           -68.590mA            -0.109A

-1.30     -88.180mA           -63.450mA            -0.103A

-1.10     -81.070mA           -57.450mA           -94.860mA

-0.90     -72.120mA           -50.410mA           -85.110mA

-0.70     -60.885mA           -41.931mA           -72.520mA

-0.60     -54.073mA           -36.985mA           -64.793mA

-0.50     -46.264mA           -31.435mA           -55.774mA

-0.40     -37.639mA           -25.394mA           -45.609mA

-0.30     -28.607mA           -19.132mA           -34.817mA

-0.20     -19.300mA           -12.780mA           -23.600mA

-0.10      -9.754mA            -6.392mA           -11.980mA

0.00      20.940pA             0.000A             93.150pA

0.10       9.359mA             6.091mA            11.580mA

0.20      17.790mA            11.620mA            22.040mA

0.30      25.330mA            16.620mA            31.450mA

0.40      32.030mA            21.090mA            39.840mA

0.50      37.910mA            25.050mA            47.270mA

0.60      43.030mA            28.520mA            53.780mA

0.70      47.410mA            31.520mA            59.410mA

0.80      51.070mA            34.050mA            64.180mA

0.90      54.030mA            36.140mA            68.110mA

1.00      56.330mA            37.800mA            71.230mA

1.10      58.020mA            39.060mA            73.580mA

1.20      59.210mA            39.980mA            75.260mA

1.30      60.020mA            40.630mA            76.420mA

1.40      60.590mA            41.080mA            77.220mA

1.50      61.000mA            41.410mA            77.800mA

1.60      61.300mA            41.650mA            78.220mA

1.70      61.540mA            41.840mA            78.540mA

1.80      61.720mA            41.990mA            78.790mA

1.90      61.880mA            42.110mA            79.000mA

2.00      62.010mA            42.210mA            79.170mA

2.10      62.120mA            42.290mA            79.320mA

2.20      62.220mA            42.370mA            79.440mA

2.30      62.300mA            42.440mA            79.560mA

2.40      62.380mA            42.490mA            79.660mA

2.50      62.450mA            42.550mA            79.750mA

2.60      62.520mA            42.600mA            79.830mA

2.70      62.580mA            42.650mA            79.910mA

2.80      62.640mA            42.770mA            79.980mA

2.90      62.730mA            43.190mA            80.060mA

3.00      63.110mA            44.290mA            80.160mA

3.10      64.340mA            46.130mA            80.580mA

3.20      66.610mA            48.580mA            82.050mA

3.30      69.730mA            51.490mA            84.800mA

3.40      73.520mA            54.790mA            88.570mA

3.50      77.880mA            58.430mA            93.160mA

3.60      82.710mA            62.350mA            98.450mA

3.70      87.980mA            66.520mA             0.104A

3.80      93.620mA            70.930mA             0.111A

4.00       0.106A             80.390mA             0.125A

4.20       0.120A             90.710mA             0.142A

4.40       0.136A              0.102A              0.161A

4.60       0.153A              0.114A              0.184A

4.80       0.174A              0.128A              0.211A

5.00       0.198A              0.144A              0.243A

|

[Pullup]

| voltage     I(typ)              I(min)              I(max)

|

-2.50       0.127A              0.107A              0.142A

-2.30       0.126A              0.105A              0.142A

-2.10       0.124A              0.101A              0.140A

-1.90       0.120A             96.480mA             0.135A

-1.70       0.114A             90.610mA             0.129A

-1.50       0.107A             84.100mA             0.122A

-1.30      99.090mA            76.620mA             0.113A

-1.10      89.310mA            68.240mA             0.103A

-0.90      77.880mA            58.752mA            90.050mA

-0.70      64.193mA            47.867mA            74.874mA

-0.60      56.265mA            41.708mA            66.020mA

-0.50      47.428mA            34.960mA            56.041mA

-0.40      37.889mA            27.784mA            45.021mA

-0.30      28.177mA            20.545mA            33.616mA

-0.20      18.560mA            13.449mA            22.240mA

-0.10       9.145mA             6.584mA            11.000mA

0.00       0.100nA             0.100nA             0.000A

0.10      -8.671mA            -6.200mA           -10.510mA

0.20     -16.740mA           -11.970mA           -20.330mA

0.30     -24.210mA           -17.310mA           -29.470mA

0.40     -31.110mA           -22.220mA           -37.950mA

0.50     -37.420mA           -26.720mA           -45.780mA

0.60     -43.180mA           -30.800mA           -52.960mA

0.70     -48.370mA           -34.480mA           -59.510mA

0.80     -53.020mA           -37.750mA           -65.440mA

0.90     -57.140mA           -40.630mA           -70.760mA

1.00     -60.720mA           -43.110mA           -75.480mA

1.10     -63.790mA           -45.190mA           -79.600mA

1.20     -66.340mA           -46.900mA           -83.140mA

1.30     -68.380mA           -48.220mA           -86.100mA

1.40     -69.870mA           -49.180mA           -88.440mA

1.50     -70.870mA           -49.840mA           -90.080mA

1.60     -71.560mA           -50.320mA           -91.160mA

1.70     -72.070mA           -50.690mA           -91.930mA

1.80     -72.470mA           -50.980mA           -92.510mA

1.90     -72.790mA           -51.220mA           -92.970mA

2.00     -73.060mA           -51.430mA           -93.340mA

2.10     -73.290mA           -51.600mA           -93.650mA

2.20     -73.490mA           -51.750mA           -93.920mA

2.30     -73.660mA           -51.890mA           -94.150mA

2.40     -73.820mA           -52.010mA           -94.350mA

2.50     -73.960mA           -52.120mA           -94.540mA

2.60     -74.090mA           -52.220mA           -94.700mA

2.70     -74.210mA           -52.330mA           -94.860mA

2.80     -74.330mA           -52.570mA           -95.000mA

2.90     -74.490mA           -53.600mA           -95.140mA

3.00     -75.230mA           -56.430mA           -95.310mA

3.10     -78.200mA           -61.150mA           -96.020mA

3.20     -84.100mA           -67.280mA           -99.340mA

3.30     -92.230mA           -74.490mA            -0.106A

3.40      -0.102A            -82.540mA            -0.116A

3.50      -0.113A            -91.300mA            -0.128A

3.60      -0.125A             -0.101A             -0.141A

3.70      -0.137A             -0.110A             -0.155A

3.80      -0.151A             -0.121A             -0.170A

4.00      -0.178A             -0.142A             -0.202A

4.20      -0.208A             -0.165A             -0.236A

4.40      -0.238A             -0.188A             -0.271A

4.60      -0.270A             -0.213A             -0.307A

4.80      -0.303A             -0.239A             -0.345A

5.00      -0.337A             -0.266A             -0.385A

|

[Ramp]

| variable       typ                 min                 max

dV/dt_r          1.18/0.74n          1.02/1.05n          1.30/0.61n

dV/dt_f          1.18/0.66n          0.98/1.11n          1.31/0.53n

R_load = 50.00

|

[Rising Waveform]

R_fixture = 50.00

V_fixture = 0.000

V_fixture_min = 0.000

V_fixture_max = 0.000

C_fixture = 0.000F

R_dut = 68.00m

L_dut = 2.31nH

C_dut = 0.65pF

| time           V(typ)              V(min)              V(max)

|

0.000S         0.000V              0.000V              0.000V

1.000e-10S       0.000V             -2.474uV             1.816uV

0.200nS      -32.060uV           -15.290uV            -2.075mV

0.300nS      -13.130mV            -0.547mV           -29.690mV

0.400nS      -38.850mV           -11.990mV           -64.030mV

0.500nS      -49.940mV           -29.740mV           -45.800mV

0.600nS      -33.790mV           -46.470mV           -12.430mV

0.700nS      -13.420mV           -42.230mV            -9.130mV

0.800nS       -8.913mV           -24.840mV           -10.260mV

0.900nS       -8.624mV            -8.352mV            38.260mV

1.000nS       -1.217mV            -3.234mV             0.173V

1.100nS        6.189mV            -4.095mV             0.309V

1.200nS       50.630mV            -5.933mV             0.518V

1.300nS        0.130V             -7.771mV             0.791V

1.400nS        0.242V             -8.698mV             1.068V

1.500nS        0.388V             -6.863mV             1.254V

1.600nS        0.534V              3.684mV             1.439V

1.700nS        0.719V             38.730mV             1.625V

1.800nS        0.918V             77.270mV             1.811V

1.900nS        1.101V              0.116V              1.988V

2.000nS        1.240V              0.159V              2.032V

2.100nS        1.378V              0.239V              2.062V

2.200nS        1.516V              0.327V              2.082V

2.300nS        1.654V              0.424V              2.095V

2.400nS        1.771V              0.526V              2.108V

2.500nS        1.818V              0.628V              2.121V

2.600nS        1.848V              0.729V              2.134V

2.700nS        1.868V              0.831V              2.141V

2.800nS        1.884V              0.928V              2.144V

2.900nS        1.900V              1.022V              2.146V

3.000nS        1.917V              1.117V              2.148V

3.100nS        1.933V              1.212V              2.150V

3.200nS        1.939V              1.307V              2.152V

3.300nS        1.943V              1.370V              2.153V

3.400nS        1.946V              1.414V              2.154V

3.500nS        1.950V              1.458V              2.156V

3.600nS        1.953V              1.502V              2.157V

3.700nS        1.956V              1.546V              2.157V

3.800nS        1.958V              1.571V              2.157V

3.900nS        1.960V              1.583V              2.156V

4.000nS        1.962V              1.596V              2.156V

4.100nS        1.964V              1.609V              2.155V

4.200nS        1.964V              1.621V              2.155V

4.300nS        1.964V              1.631V              2.156V

4.400nS        1.964V              1.639V              2.156V

4.500nS        1.963V              1.647V              2.157V

4.600nS        1.963V              1.655V              2.157V

4.700nS        1.964V              1.663V              2.157V

4.800nS        1.964V              1.668V              2.157V

4.900nS        1.965V              1.670V              2.156V

5.000nS        1.965V              1.672V              2.156V

5.100nS        1.966V              1.674V              2.156V

5.200nS        1.966V              1.676V              2.156V

5.300nS        1.965V              1.678V              2.156V

5.400nS        1.965V              1.679V              2.156V

5.500nS        1.965V              1.681V              2.157V

5.600nS        1.965V              1.683V              2.157V

5.700nS        1.965V              1.685V              2.157V

5.800nS        1.965V              1.686V              2.157V

5.900nS        1.965V              1.687V              2.156V

6.000nS        1.965V              1.687V              2.156V

6.100nS        1.965V              1.687V              2.156V

6.200nS        1.965V              1.688V              2.156V

6.300nS        1.965V              1.688V              2.156V

6.400nS        1.965V              1.688V              2.156V

6.500nS        1.965V              1.689V              2.156V

6.600nS        1.965V              1.689V              2.157V

6.700nS        1.965V              1.689V              2.157V

6.800nS        1.965V              1.690V              2.156V

6.900nS        1.965V              1.690V              2.156V

7.000nS        1.965V              1.690V              2.156V

7.100nS        1.965V              1.690V              2.156V

7.200nS        1.965V              1.691V              2.156V

7.300nS        1.965V              1.691V              2.156V

7.400nS        1.965V              1.691V              2.156V

7.500nS        1.965V              1.690V              2.156V

7.600nS        1.965V              1.690V              2.156V

7.700nS        1.965V              1.690V              2.156V

7.800nS        1.965V              1.690V              2.156V

7.900nS        1.965V              1.690V              2.156V

8.000nS        1.965V              1.691V              2.156V

8.100nS        1.965V              1.691V              2.156V

8.200nS        1.965V              1.691V              2.156V

8.300nS        1.965V              1.691V              2.156V

8.400nS        1.965V              1.691V              2.156V

8.500nS        1.965V              1.691V              2.156V

8.600nS        1.965V              1.690V              2.156V

8.700nS        1.965V              1.690V              2.156V

8.800nS        1.965V              1.690V              2.156V

8.900nS        1.965V              1.691V              2.156V

9.000nS        1.965V              1.691V              2.156V

9.100nS        1.965V              1.691V              2.157V

9.200nS        1.965V              1.691V              2.156V

9.300nS        1.965V              1.691V              2.156V

9.400nS        1.965V              1.691V              2.156V

9.500nS        1.965V              1.691V              2.156V

9.600nS        1.965V              1.691V              2.156V

9.700nS        1.965V              1.690V              2.156V

9.800nS        1.965V              1.690V              2.156V

9.900nS        1.965V              1.691V              2.156V

|

[Rising Waveform]

R_fixture = 50.00

V_fixture = 2.50

V_fixture_min = 2.37

V_fixture_max = 2.62

C_fixture = 0.000F

R_dut = 68.00m

L_dut = 2.31nH

C_dut = 0.65pF

| time           V(typ)              V(min)              V(max)

|

0.000S         0.530V              0.744V              0.450V

1.000e-10S       0.530V              0.744V              0.450V

0.200nS        0.530V              0.744V              0.450V

0.300nS        0.576V              0.745V              0.642V

0.400nS        1.192V              0.821V              1.837V

0.500nS        2.047V              1.285V              2.562V

0.600nS        2.415V              1.865V              2.569V

0.700nS        2.468V              2.182V              2.577V

0.800nS        2.474V              2.359V              2.594V

0.900nS        2.482V              2.359V              2.597V

1.000nS        2.482V              2.363V              2.599V

1.100nS        2.483V              2.366V              2.601V

1.200nS        2.483V              2.368V              2.602V

1.300nS        2.483V              2.366V              2.602V

1.400nS        2.477V              2.364V              2.604V

1.500nS        2.477V              2.362V              2.606V

1.600nS        2.478V              2.360V              2.608V

1.700nS        2.480V              2.359V              2.610V

1.800nS        2.482V              2.357V              2.613V

1.900nS        2.483V              2.355V              2.615V

2.000nS        2.485V              2.354V              2.616V

2.100nS        2.487V              2.352V              2.617V

2.200nS        2.488V              2.351V              2.618V

2.300nS        2.489V              2.355V              2.619V

2.400nS        2.491V              2.357V              2.620V

2.500nS        2.492V              2.359V              2.621V

2.600nS        2.493V              2.360V              2.622V

2.700nS        2.494V              2.361V              2.622V

2.800nS        2.495V              2.362V              2.623V

2.900nS        2.495V              2.364V              2.624V

3.000nS        2.496V              2.365V              2.624V

3.100nS        2.497V              2.365V              2.624V

3.200nS        2.497V              2.366V              2.624V

3.300nS        2.498V              2.367V              2.624V

3.400nS        2.498V              2.367V              2.624V

3.500nS        2.499V              2.368V              2.624V

3.600nS        2.499V              2.369V              2.624V

3.700nS        2.499V              2.369V              2.624V

3.800nS        2.499V              2.370V              2.624V

3.900nS        2.499V              2.371V              2.624V

4.000nS        2.499V              2.371V              2.625V

4.100nS        2.499V              2.371V              2.625V

4.200nS        2.499V              2.372V              2.625V

4.300nS        2.499V              2.372V              2.625V

4.400nS        2.500V              2.372V              2.625V

4.500nS        2.500V              2.373V              2.625V

4.600nS        2.500V              2.373V              2.625V

4.700nS        2.500V              2.373V              2.625V

4.800nS        2.499V              2.373V              2.625V

4.900nS        2.499V              2.374V              2.625V

5.000nS        2.499V              2.374V              2.625V

5.100nS        2.499V              2.374V              2.625V

5.200nS        2.499V              2.374V              2.625V

5.300nS        2.500V              2.374V              2.625V

5.400nS        2.500V              2.374V              2.625V

5.500nS        2.500V              2.374V              2.625V

5.600nS        2.500V              2.374V              2.625V

5.700nS        2.500V              2.374V              2.625V

5.800nS        2.500V              2.374V              2.625V

5.900nS        2.499V              2.374V              2.625V

6.000nS        2.499V              2.374V              2.625V

6.100nS        2.499V              2.374V              2.625V

6.200nS        2.499V              2.374V              2.625V

6.300nS        2.500V              2.374V              2.625V

6.400nS        2.500V              2.374V              2.625V

6.500nS        2.500V              2.374V              2.625V

6.600nS        2.500V              2.374V              2.625V

6.700nS        2.500V              2.374V              2.625V

6.800nS        2.500V              2.374V              2.625V

6.900nS        2.500V              2.374V              2.625V

7.000nS        2.499V              2.374V              2.625V

7.100nS        2.499V              2.375V              2.625V

7.200nS        2.500V              2.375V              2.625V

7.300nS        2.500V              2.375V              2.625V

7.400nS        2.500V              2.375V              2.625V

7.500nS        2.500V              2.375V              2.625V

7.600nS        2.500V              2.375V              2.625V

7.700nS        2.500V              2.375V              2.625V

7.800nS        2.500V              2.375V              2.625V

7.900nS        2.500V              2.375V              2.625V

8.000nS        2.500V              2.375V              2.625V

8.100nS        2.500V              2.375V              2.625V

8.200nS        2.500V              2.375V              2.625V

8.300nS        2.500V              2.375V              2.625V

8.400nS        2.500V              2.375V              2.625V

8.500nS        2.500V              2.375V              2.625V

8.600nS        2.500V              2.375V              2.625V

8.700nS        2.500V              2.375V              2.625V

8.800nS        2.500V              2.375V              2.625V

8.900nS        2.500V              2.375V              2.625V

9.000nS        2.500V              2.375V              2.625V

9.100nS        2.500V              2.375V              2.625V

9.200nS        2.500V              2.375V              2.625V

9.300nS        2.500V              2.375V              2.625V

9.400nS        2.500V              2.375V              2.625V

9.500nS        2.500V              2.375V              2.625V

9.600nS        2.500V              2.375V              2.625V

9.700nS        2.500V              2.375V              2.625V

9.800nS        2.500V              2.375V              2.625V

9.900nS        2.500V              2.375V              2.625V

|

[Falling Waveform]

R_fixture = 50.00

V_fixture = 0.000

V_fixture_min = 0.000

V_fixture_max = 0.000

C_fixture = 0.000F

R_dut = 68.00m

L_dut = 2.31nH

C_dut = 0.65pF

| time           V(typ)              V(min)              V(max)

|

0.000S         1.965V              1.691V              2.156V

1.000e-10S       1.965V              1.691V              2.156V

0.200nS        1.965V              1.691V              2.159V

0.300nS        1.948V              1.690V              2.004V

0.400nS        1.593V              1.644V              1.130V

0.500nS        0.790V              1.388V              0.203V

0.600nS        0.144V              0.875V             45.080mV

0.700nS       58.180mV             0.420V             62.050mV

0.800nS       44.520mV             0.179V             25.370mV

0.900nS       21.300mV            29.520mV            12.940mV

1.000nS       12.610mV            23.540mV             5.742mV

1.100nS        5.851mV            18.640mV            -1.461mV

1.200nS        4.238mV            14.240mV            -0.270mV

1.300nS        4.744mV             9.842mV             1.809mV

1.400nS        5.251mV             8.502mV             3.888mV

1.500nS        5.758mV             7.605mV             5.967mV

1.600nS        6.265mV             6.849mV             8.046mV

1.700nS        5.350mV             6.218mV             5.949mV

1.800nS        3.848mV             5.587mV             3.410mV

1.900nS        2.347mV             4.955mV             0.871mV

2.000nS        0.845mV             4.324mV            -1.668mV

2.100nS       -0.657mV             3.811mV            -4.207mV

2.200nS       -0.587mV             3.476mV            -2.913mV

2.300nS        0.130mV             3.140mV            -1.214mV

2.400nS        0.848mV             2.804mV             0.485mV

2.500nS        1.566mV             2.468mV             2.184mV

2.600nS        2.283mV             2.187mV             3.883mV

2.700nS        1.985mV             1.986mV             2.832mV

2.800nS        1.268mV             1.785mV             1.489mV

2.900nS        0.550mV             1.584mV             0.146mV

3.000nS       -0.167mV             1.383mV            -1.197mV

3.100nS       -0.884mV             1.215mV            -2.539mV

3.200nS       -0.814mV             1.099mV            -1.872mV

3.300nS       -0.419mV             0.982mV            -0.992mV

3.400nS      -23.550uV             0.866mV            -0.111mV

3.500nS        0.372mV             0.749mV             0.769mV

3.600nS        0.767mV             0.653mV             1.649mV

3.700nS        0.709mV             0.590mV             1.260mV

3.800nS        0.465mV             0.526mV             0.737mV

3.900nS        0.221mV             0.462mV             0.214mV

4.000nS      -22.860uV             0.398mV            -0.309mV

4.100nS       -0.267mV             0.346mV            -0.833mV

4.200nS       -0.283mV             0.311mV            -0.680mV

4.300nS       -0.206mV             0.276mV            -0.456mV

4.400nS       -0.128mV             0.242mV            -0.232mV

4.500nS      -51.020uV             0.207mV            -8.215uV

4.600nS       26.440uV             0.179mV             0.216mV

4.700nS       71.970uV             0.161mV             0.239mV

4.800nS        0.104mV             0.143mV             0.241mV

4.900nS        0.137mV             0.126mV             0.243mV

5.000nS        0.169mV             0.108mV             0.245mV

5.100nS        0.201mV            93.050uV             0.247mV

5.200nS        0.130mV            82.030uV            98.430uV

5.300nS       15.060uV            71.010uV           -66.050uV

5.400nS      -99.560uV            59.990uV            -0.230mV

5.500nS       -0.214mV            48.960uV            -0.395mV

5.600nS       -0.329mV            41.220uV            -0.559mV

5.700nS       -0.246mV            38.380uV            -0.332mV

5.800nS      -81.220uV            35.540uV           -62.380uV

5.900nS       83.330uV            32.700uV             0.207mV

6.000nS        0.248mV            29.860uV             0.476mV

6.100nS        0.412mV            26.870uV             0.746mV

6.200nS        0.325mV            23.650uV             0.476mV

6.300nS        0.133mV            20.430uV             0.150mV

6.400nS      -58.560uV            17.210uV            -0.176mV

6.500nS       -0.250mV            14.000uV            -0.502mV

6.600nS       -0.442mV            11.730uV            -0.828mV

6.700nS       -0.357mV            10.900uV            -0.547mV

6.800nS       -0.159mV            10.060uV            -0.203mV

6.900nS       39.040uV             9.225uV             0.142mV

7.000nS        0.237mV             8.390uV             0.487mV

7.100nS        0.436mV             7.672uV             0.832mV

7.200nS        0.359mV             7.129uV             0.562mV

7.300nS        0.169mV             6.586uV             0.227mV

7.400nS      -20.940uV             6.044uV            -0.107mV

7.500nS       -0.211mV             5.501uV            -0.442mV

7.600nS       -0.401mV             4.902uV            -0.777mV

7.700nS       -0.335mV             4.219uV            -0.533mV

7.800nS       -0.164mV             3.536uV            -0.229mV

7.900nS        7.351uV             2.853uV            76.200uV

8.000nS        0.179mV             2.169uV             0.381mV

8.100nS        0.350mV             1.915uV             0.686mV

8.200nS        0.296mV             2.303uV             0.477mV

8.300nS        0.150mV             2.690uV             0.215mV

8.400nS        3.864uV             3.078uV           -47.710uV

8.500nS       -0.142mV             3.466uV            -0.310mV

8.600nS       -0.288mV             3.402uV            -0.573mV

8.700nS       -0.247mV             2.662uV            -0.404mV

8.800nS       -0.129mV             1.923uV            -0.190mV

8.900nS      -11.050uV             1.183uV            24.790uV

9.000nS        0.107mV             0.443uV             0.239mV

9.100nS        0.225mV             0.177uV             0.454mV

9.200nS        0.196mV             0.620uV             0.325mV

9.300nS        0.106mV             1.062uV             0.159mV

9.400nS       16.280uV             1.505uV            -5.797uV

9.500nS      -73.710uV             1.948uV            -0.171mV

9.600nS       -0.164mV             2.012uV            -0.336mV

9.700nS       -0.133mV             1.509uV            -0.221mV

9.800nS      -53.290uV             1.006uV           -76.710uV

9.900nS       26.720uV             0.503uV            67.940uV

|

[Falling Waveform]

R_fixture = 50.00

V_fixture = 2.50

V_fixture_min = 2.37

V_fixture_max = 2.62

C_fixture = 0.000F

R_dut = 68.00m

L_dut = 2.31nH

C_dut = 0.65pF

| time           V(typ)              V(min)              V(max)

|

0.000S         2.500V              2.375V              2.625V

1.000e-10S       2.500V              2.375V              2.625V

0.200nS        2.499V              2.375V              2.629V

0.300nS        2.528V              2.376V              2.698V

0.400nS        2.585V              2.401V              2.765V

0.500nS        2.634V              2.438V              2.785V

0.600nS        2.652V              2.466V              2.648V

0.700nS        2.573V              2.491V              2.434V

0.800nS        2.462V              2.487V              2.137V

0.900nS        2.320V              2.448V              1.823V

1.000nS        2.169V              2.403V              1.568V

1.100nS        1.982V              2.358V              1.345V

1.200nS        1.778V              2.313V              1.123V

1.300nS        1.573V              2.253V              0.901V

1.400nS        1.368V              2.173V              0.678V

1.500nS        1.163V              2.079V              0.574V

1.600nS        1.029V              1.986V              0.542V

1.700nS        0.932V              1.892V              0.522V

1.800nS        0.836V              1.798V              0.511V

1.900nS        0.739V              1.699V              0.500V

2.000nS        0.642V              1.596V              0.489V

2.100nS        0.609V              1.493V              0.477V

2.200nS        0.604V              1.391V              0.468V

2.300nS        0.594V              1.288V              0.465V

2.400nS        0.585V              1.210V              0.463V

2.500nS        0.577V              1.152V              0.461V

2.600nS        0.569V              1.094V              0.459V

2.700nS        0.560V              1.036V              0.457V

2.800nS        0.552V              0.979V              0.456V

2.900nS        0.550V              0.939V              0.455V

3.000nS        0.547V              0.913V              0.453V

3.100nS        0.544V              0.888V              0.452V

3.200nS        0.541V              0.862V              0.451V

3.300nS        0.539V              0.837V              0.451V

3.400nS        0.538V              0.821V              0.451V

3.500nS        0.537V              0.813V              0.451V

3.600nS        0.535V              0.806V              0.452V

3.700nS        0.534V              0.798V              0.452V

3.800nS        0.534V              0.790V              0.451V

3.900nS        0.533V              0.784V              0.451V

4.000nS        0.532V              0.779V              0.451V

4.100nS        0.532V              0.774V              0.450V

4.200nS        0.531V              0.770V              0.450V

4.300nS        0.531V              0.765V              0.450V

4.400nS        0.531V              0.762V              0.450V

4.500nS        0.531V              0.760V              0.451V

4.600nS        0.531V              0.758V              0.451V

4.700nS        0.531V              0.756V              0.451V

4.800nS        0.531V              0.754V              0.451V

4.900nS        0.531V              0.753V              0.451V

5.000nS        0.531V              0.752V              0.450V

5.100nS        0.530V              0.751V              0.450V

5.200nS        0.530V              0.751V              0.450V

5.300nS        0.530V              0.750V              0.450V

5.400nS        0.530V              0.749V              0.450V

5.500nS        0.530V              0.748V              0.450V

5.600nS        0.530V              0.747V              0.451V

5.700nS        0.530V              0.746V              0.451V

5.800nS        0.531V              0.745V              0.451V

5.900nS        0.530V              0.745V              0.451V

6.000nS        0.530V              0.745V              0.450V

6.100nS        0.530V              0.746V              0.450V

6.200nS        0.530V              0.746V              0.450V

6.300nS        0.530V              0.746V              0.450V

6.400nS        0.530V              0.746V              0.450V

6.500nS        0.530V              0.745V              0.450V

6.600nS        0.530V              0.745V              0.450V

6.700nS        0.530V              0.744V              0.450V

6.800nS        0.530V              0.743V              0.450V

6.900nS        0.530V              0.743V              0.450V

7.000nS        0.530V              0.744V              0.450V

7.100nS        0.530V              0.744V              0.451V

7.200nS        0.530V              0.745V              0.451V

7.300nS        0.530V              0.745V              0.450V

7.400nS        0.530V              0.745V              0.450V

7.500nS        0.530V              0.745V              0.450V

7.600nS        0.530V              0.744V              0.450V

7.700nS        0.530V              0.743V              0.450V

7.800nS        0.530V              0.743V              0.450V

7.900nS        0.530V              0.743V              0.450V

8.000nS        0.530V              0.743V              0.450V

8.100nS        0.530V              0.744V              0.451V

8.200nS        0.530V              0.744V              0.451V

8.300nS        0.530V              0.745V              0.451V

8.400nS        0.530V              0.745V              0.450V

8.500nS        0.530V              0.744V              0.450V

8.600nS        0.530V              0.744V              0.450V

8.700nS        0.530V              0.744V              0.450V

8.800nS        0.530V              0.743V              0.450V

8.900nS        0.530V              0.743V              0.450V

9.000nS        0.530V              0.743V              0.450V

9.100nS        0.530V              0.744V              0.451V

9.200nS        0.530V              0.744V              0.451V

9.300nS        0.530V              0.744V              0.451V

9.400nS        0.530V              0.744V              0.450V

9.500nS        0.530V              0.744V              0.450V

9.600nS        0.530V              0.744V              0.450V

9.700nS        0.530V              0.744V              0.450V

9.800nS        0.530V              0.744V              0.450V

9.900nS        0.530V              0.743V              0.450V

|

| End [Model] IBIS_OUT

|

|************************************************************************

|                            Model IBIS_INBUF

|************************************************************************

|

[Model]          IBIS_INBUF

Model_type       Input

Vinl =  0.700V

Vinh =  1.700V

Vmeas =  1.250V

C_comp           1.00pF              0.80pF              1.20pF

|

|

[Temperature Range]       25.00               100.00              0.000

[Voltage Range]           2.50V               2.37V               2.62V

[GND_clamp]

| voltage     I(typ)              I(min)              I(max)

|

-2.50      -8.520mA            -7.269mA            -9.344mA

-2.40      -8.028mA            -6.837mA            -8.819mA

-2.30      -7.537mA            -6.408mA            -8.294mA

-2.20      -7.048mA            -5.981mA            -7.771mA

-2.10      -6.560mA            -5.557mA            -7.249mA

-2.00      -6.076mA            -5.136mA            -6.729mA

-1.90      -5.594mA            -4.718mA            -6.212mA

-1.80      -5.115mA            -4.305mA            -5.697mA

-1.70      -4.641mA            -3.896mA            -5.185mA

-1.60      -4.171mA            -3.493mA            -4.678mA

-1.50      -3.707mA            -3.096mA            -4.175mA

-1.40      -3.249mA            -2.707mA            -3.679mA

-1.30      -2.800mA            -2.326mA            -3.189mA

-1.20      -2.361mA            -1.956mA            -2.709mA

-1.10      -1.934mA            -1.599mA            -2.240mA

-1.00      -1.523mA            -1.257mA            -1.785mA

-0.90      -1.133mA            -0.935mA            -1.349mA

-0.80      -0.770mA            -0.639mA            -0.939mA

-0.70      -0.448mA            -0.378mA            -0.567mA

-0.60      -0.192mA            -0.171mA            -0.259mA

-0.50     -43.530uA           -47.010uA           -64.090uA

-0.40      -4.090uA            -6.592uA            -5.956uA

-0.30      -0.207uA            -0.579uA            -0.248uA

-0.20     -24.350nA           -50.960nA           -23.800nA

-0.10     -10.080nA           -11.270nA           -10.040nA

0.00     6.723e-25A          2.838e-21A          1.319e-25A

0.10      10.000nA            10.050nA            10.000nA

0.20      20.000nA            20.050nA            20.000nA

0.30      30.000nA            30.060nA            30.000nA

0.40      40.000nA            40.060nA            40.000nA

0.50      50.000nA            50.060nA            50.000nA

0.60      60.000nA            60.060nA            60.000nA

0.70      70.000nA            70.060nA            70.000nA

0.80      80.000nA            80.060nA            80.000nA

0.90      90.000nA            90.070nA            90.000nA

1.00     100.000nA             0.100uA           100.000nA

1.10       0.110uA             0.110uA             0.110uA

1.20       0.120uA             0.120uA             0.120uA

1.30       0.130uA             0.130uA             0.130uA

1.40       0.140uA             0.140uA             0.140uA

1.50       0.150uA             0.150uA             0.150uA

1.60       0.160uA             0.160uA             0.160uA

1.70       0.170uA             0.170uA             0.170uA

1.80       0.180uA             0.180uA             0.180uA

1.90       0.190uA             0.190uA             0.190uA

2.00       0.200uA             0.200uA             0.200uA

2.10       0.210uA             0.210uA             0.210uA

2.20       0.220uA             0.220uA             0.220uA

2.30       0.230uA             0.230uA             0.230uA

2.40       0.240uA             0.240uA             0.240uA

2.50       0.250uA             0.250uA             0.250uA

|

[POWER_clamp]

| voltage     I(typ)              I(min)              I(max)

|

-2.50       0.500uA             0.488uA             0.512uA

-2.40       0.490uA             0.478uA             0.502uA

-2.30       0.480uA             0.468uA             0.492uA

-2.20       0.470uA             0.458uA             0.482uA

-2.10       0.460uA             0.448uA             0.472uA

-2.00       0.450uA             0.438uA             0.462uA

-1.90       0.440uA             0.428uA             0.452uA

-1.80       0.430uA             0.418uA             0.442uA

-1.70       0.420uA             0.408uA             0.432uA

-1.60       0.410uA             0.398uA             0.422uA

-1.50       0.400uA             0.388uA             0.412uA

-1.40       0.390uA             0.378uA             0.402uA

-1.30       0.380uA             0.368uA             0.392uA

-1.20       0.370uA             0.358uA             0.382uA

-1.10       0.360uA             0.348uA             0.372uA

-1.00       0.350uA             0.338uA             0.362uA

-0.90       0.340uA             0.328uA             0.352uA

-0.80       0.330uA             0.318uA             0.342uA

-0.70       0.320uA             0.308uA             0.332uA

-0.60       0.310uA             0.298uA             0.322uA

-0.50       0.300uA             0.288uA             0.312uA

-0.40       0.290uA             0.278uA             0.302uA

-0.30       0.280uA             0.268uA             0.292uA

-0.20       0.270uA             0.258uA             0.282uA

-0.10       0.260uA             0.248uA             0.272uA

0.00       0.250uA             0.238uA             0.262uA

|

| End [Model] IBIS_INBUF

|

| End [Component] AS8C163631

|

[End]