AS8C40362511111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111/*******************************************************************************

*   Copyright 2010 Alliance Memory, Inc.

*   All right reserved.

*

*   This program is proprietary and confidential information of

*   Alliance Memory, Inc Corp. and may be used and disclosed only as       authorized

*   in a license agreement controlling such use and disclosure.

*   Alliance Memory, Inc reserves the right to make any changes to

*   the product herein to improve function or design.

*   Alliance Memory, Inc does not assume any liability arising out of

*   the application or use of the product herein.

*

*   WARNING: The unlicensed shipping, mailing, or carring of this

*   technical data outside the United States, or the unlicensed

*   disclosure, by whatever means, through visits abroad, or the

*   unlicensed disclosure to foreign national in the United States,

*   may violate the United States criminal law.

*

*   File Name                 : AS8C403625

*   Function                  : 128Kx36 flow-through burst Static RAM

*   Simulation Tool/Version   : Verilog-XL 2.5

*

******************************************************************************/

/*******************************************************************************

* Module Name: AS8C403625

*

* Notes  : This model is believed to be functionally accurate.

*******************************************************************************/

`timescale 1ns/10ps

module idt71v3577s75(A, D, DP, oe_, ce_, cs0, cs1_, lbo_,

gw_, bwe_, bw4_, bw3_, bw2_, bw1_,

adsp_, adsc_, adv_, clk);

initial

begin

$write("\n********************************************************\n");

$write("   AS8C403625                                         \n");

$write("   Rev: 00 July '99                                       \n");

$write("              \n");

$write("********************************************************\n\n");

end

parameter addr_msb = 16;

parameter mem_top = 131071;

parameter regdelay = 1;

inout [31:0] D;

inout [3:0] DP;

input [addr_msb:0] A;

input oe_, ce_, cs0, cs1_, lbo_, gw_, bwe_, bw4_, bw3_, bw2_, bw1_,

adsp_, adsc_, adv_, clk;

//internal registers for data, address, burst counter

reg [31:0] din;

reg [3:0] dpin;

reg [addr_msb:0] reg_addr;

reg [1:0] brst_cnt;

wire[addr_msb:0] m_ad;

wire[31:0] data_out;

wire[3:0] dp_out;

reg wr_b1_, wr_b2_, wr_b3_, wr_b4_, deselr;

wire check_data =  (~adsc_ & adsp_ & ~ce_ & cs0 & ~cs1_

& (~gw_ | ~bwe_ & (~bw1_ | ~bw2_ | ~bw3_ | ~bw4_)))

| (~deselr & adsc_ & (adsp_ | ce_)

& (~gw_ | ~bwe_ & (~bw1_ | ~bw2_ | ~bw3_ | ~bw4_)));

wire check_addr =   (~adsp_ & ~ce_ & cs0 & ~cs1_)

| ( adsp_ & ~adsc_ & ~ce_ & cs0 & ~cs1_);

specify

specparam

//Clock Parameters

tCYC  = 8.5,   //clock cycle time

tCH   = 3.0,   //clock high time

tCL   = 3.0,   //clock low time

//Output Parameters

tCD   = 7.5,   //clk to data

tCDC  = 2.0,   //output hold from clock

tCLZ  = 0.0,   //CLK to output Low-Z

tCHZ  = 3.5,   //CLK to output Hi-Z

tOE   = 3.5,   //OE to output valid

tOLZ  = 0.0,   //OE to output Hi-Z

tOHZ  = 3.5,   //OE to output Hi-Z

//Set up times

tSA   = 1.5, //address set-up

tSS   = 1.5, //address status set-up

tSD   = 1.5, //data set-up

tSW   = 1.5, //write set-up

tSAV  = 1.5, //address advance set-up

tSC   = 1.5, //chip enable and chip select set-up

//Hold times

tHA   = 0.5, //Address hold

tHS   = 0.5, //address status hold

tHD   = 0.5, //data hold

tHW   = 0.5, //write hold

tHAV  = 0.5, //address advance hold

tHC   = 0.5; //chip enable and chip select hold

(oe_ *> D) = (tOE,tOE,tOHZ,tOLZ,tOHZ,tOLZ); //(01,10,0z,z1,1z,z0)

(clk *> D) = (tCD,tCD,tCHZ,tCLZ,tCHZ,tCLZ); //(01,10,0z,z1,1z,z0)

(oe_ *> DP) = (tOE,tOE,tOHZ,tOLZ,tOHZ,tOLZ); //(01,10,0z,z1,1z,z0)

(clk *> DP) = (tCD,tCD,tCHZ,tCLZ,tCHZ,tCLZ); //(01,10,0z,z1,1z,z0)

//timing checks

$period(posedge clk, tCYC );

$width (posedge clk, tCH );

$width (negedge clk, tCL );

$setuphold(posedge clk, adsp_, tSS, tHS);

$setuphold(posedge clk, adsc_, tSS, tHS);

$setuphold(posedge clk, adv_, tSAV, tHAV);

$setuphold(posedge clk, gw_, tSW, tHW);

$setuphold(posedge clk, bwe_, tSW, tHW);

$setuphold(posedge clk, bw1_, tSW, tHW);

$setuphold(posedge clk, bw2_, tSW, tHW);

$setuphold(posedge clk, bw3_, tSW, tHW);

$setuphold(posedge clk, bw4_, tSW, tHW);

$setuphold(posedge clk, ce_, tSC, tHC);

$setuphold(posedge clk, cs0, tSC, tHC);

$setuphold(posedge clk, cs1_, tSC, tHC);

$setuphold(posedge clk &&& check_addr, A, tSA, tHA);

$setuphold(posedge clk &&& check_data, D, tSD, tHD);

$setuphold(posedge clk &&& check_data, DP, tSD, tHD);

endspecify

/////////////memory array//////////////////////////////////////////////

reg [7:0] memb1[0:mem_top], memb2[0:mem_top], memb3[0:mem_top], memb4[0:mem_top];

reg memb1p[0:mem_top], memb2p[0:mem_top], memb3p[0:mem_top], memb4p[0:mem_top];

wire doe, baddr1, baddr0, dsel;

///////////////////////////////////////////////////////////////////////

//

//Output buffers: using a bufif1 has the same effect as...

//

//	assign D = doe ? data_out : 32'hzzzzzzzz;

//

//It was coded this way to support SPECIFY delays in the specparam section.

//

///////////////////////////////////////////////////////////////////////

bufif1 (D[0],data_out[0],doe);

bufif1 (D[1],data_out[1],doe);

bufif1 (D[2],data_out[2],doe);

bufif1 (D[3],data_out[3],doe);

bufif1 (D[4],data_out[4],doe);

bufif1 (D[5],data_out[5],doe);

bufif1 (D[6],data_out[6],doe);

bufif1 (D[7],data_out[7],doe);

bufif1 (DP[0], dp_out[0],doe);

bufif1 (D[8],data_out[8],doe);

bufif1 (D[9],data_out[9],doe);

bufif1 (D[10],data_out[10],doe);

bufif1 (D[11],data_out[11],doe);

bufif1 (D[12],data_out[12],doe);

bufif1 (D[13],data_out[13],doe);

bufif1 (D[14],data_out[14],doe);

bufif1 (D[15],data_out[15],doe);

bufif1 (DP[1], dp_out[1],doe);

bufif1 (D[16],data_out[16],doe);

bufif1 (D[17],data_out[17],doe);

bufif1 (D[18],data_out[18],doe);

bufif1 (D[19],data_out[19],doe);

bufif1 (D[20],data_out[20],doe);

bufif1 (D[21],data_out[21],doe);

bufif1 (D[22],data_out[22],doe);

bufif1 (D[23],data_out[23],doe);

bufif1 (DP[2], dp_out[2],doe);

bufif1 (D[24],data_out[24],doe);

bufif1 (D[25],data_out[25],doe);

bufif1 (D[26],data_out[26],doe);

bufif1 (D[27],data_out[27],doe);

bufif1 (D[28],data_out[28],doe);

bufif1 (D[29],data_out[29],doe);

bufif1 (D[30],data_out[30],doe);

bufif1 (D[31],data_out[31],doe);

bufif1 (DP[3], dp_out[3],doe);

assign doe = ~deselr & ~oe_ & wr_b1_ & wr_b2_ & wr_b3_ & wr_b4_;

assign dsel = (ce_ | ~cs0 | cs1_);

always @(posedge clk)

begin

if ( ~adsc_ || ( ~adsp_ && ~ce_ ))

deselr <= dsel;

end

//////////////////////////////////////////////////////////////

//

//write enable generation

//

//////////////////////////////////////////////////////////////

always @(posedge clk)

begin

if (  (~adsc_ & adsp_ & ~ce_ & cs0 & ~cs1_ & (~gw_ | ~bwe_ & ~bw1_))

| (~deselr & adsc_ & (adsp_ | ce_) & (~gw_ | ~bwe_ & ~bw1_)))

wr_b1_ <= 0;

else wr_b1_ <= 1;

if (  (~adsc_ & adsp_ & ~ce_ & cs0 & ~cs1_ & (~gw_ | ~bwe_ & ~bw2_))

| (~deselr & adsc_ & (adsp_ | ce_) & (~gw_ | ~bwe_ & ~bw2_)))

wr_b2_ <= 0;

else wr_b2_ <= 1;

if (  (~adsc_ & adsp_ & ~ce_ & cs0 & ~cs1_ & (~gw_ | ~bwe_ & ~bw3_))

| (~deselr & adsc_ & (adsp_ | ce_) & (~gw_ | ~bwe_ & ~bw3_)))

wr_b3_ <= 0;

else wr_b3_ <= 1;

if (  (~adsc_ & adsp_ & ~ce_ & cs0 & ~cs1_ & (~gw_ | ~bwe_ & ~bw4_))

| (~deselr & adsc_ & (adsp_ | ce_) & (~gw_ | ~bwe_ & ~bw4_)))

wr_b4_ <= 0;

else wr_b4_ <= 1;

end

/////////////////////////////////////////////////////////////////

//

//input address register

//

/////////////////////////////////////////////////////////////////

always @(posedge clk)

begin

if (  (~adsp_ & ~ce_ & cs0 & ~cs1_)

| ( adsp_ & ~adsc_ & ~ce_ & cs0 & ~cs1_)) reg_addr[addr_msb:0] <= A[addr_msb:0];

end

//////////////////////////////////////////////////////////////////

//

// burst counter

//

//////////////////////////////////////////////////////////////////

always @(posedge clk)

begin

if (lbo_ & (  (~adsp_ & ~ce_ & cs0 & ~cs1_)

| ( adsp_ & ~adsc_ & ~ce_ & cs0 & ~cs1_))) brst_cnt <= 0;

else if (~lbo_ & (  (~adsp_ & ~ce_ & cs0 & ~cs1_)

| ( adsp_ & ~adsc_ & ~ce_ & cs0 & ~cs1_))) brst_cnt <= A[1:0];

else if ((adsp_ | ce_) & adsc_ & ~adv_) brst_cnt <= brst_cnt + 1;

end

//////////////////////////////////////////////////////////////////////

//

//determine the memory address

//

//////////////////////////////////////////////////////////////////////

assign baddr1 = lbo_ ? (brst_cnt[1] ^ reg_addr[1]) : brst_cnt[1];

assign baddr0 = lbo_ ? (brst_cnt[0] ^ reg_addr[0]) : brst_cnt[0];

assign #regdelay m_ad[addr_msb:0] = {reg_addr[addr_msb:2], baddr1, baddr0};

/////////////////////////////////////////////////////////////////////////

//

//data output

//

/////////////////////////////////////////////////////////////////////////

assign data_out = {memb4[m_ad],memb3[m_ad],memb2[m_ad],memb1[m_ad]};

assign dp_out   = {memb4p[m_ad], memb3p[m_ad], memb2p[m_ad], memb1p[m_ad]};

//////////////////////////////////////////////////////////////////////

//

//data input register

//

//////////////////////////////////////////////////////////////////////

always @(posedge clk)

begin

din <= #regdelay D;

dpin <= #regdelay DP;

end

///////////////////////////////////////////////////////////////////////

//

// write to ram

//

///////////////////////////////////////////////////////////////////////

wire #1 wrb1 = ~wr_b1_ & ~clk;

wire #1 wrb2 = ~wr_b2_ & ~clk;

wire #1 wrb3 = ~wr_b3_ & ~clk;

wire #1 wrb4 = ~wr_b4_ & ~clk;

always @(clk)

begin

if (wrb1) begin

memb1[m_ad] = din[7:0];

memb1p[m_ad] = dpin[0];

end

if (wrb2) begin

memb2[m_ad] = din[15:8];

memb2p[m_ad] = dpin[1];

end

if (wrb3) begin

memb3[m_ad] = din[23:16];

memb3p[m_ad] = dpin[2];

end

if (wrb4) begin

memb4[m_ad] = din[31:24];

memb4p[m_ad] = dpin[3];

end

end

endmodule

/

wire #1 wrb1 = ~wr_b1_ & ~clk;

wire #1 wrb2 = ~wr_b2_ & ~clk;

wire #1 wrb3 = ~wr_b3_ & ~clk;

wire #1 wrb4 = ~wr_b4_ & ~clk;

always @(clk)

begin

if (wrb1) begin

memb1[m_ad] = din[7:0];

memb1p[m_ad] = dpin[0];

end

if (wrb2) begin

memb2[m_ad] = din[15:8];

memb2p[m_ad] = dpin[1];

end

777777777777777777777777777777777777777777777777777777777777777777777777777777777777777777777777777777777777777777777777777777777777777777777777777777777777777777777777/*******************************************************************************

*   Copyright 2010 Alliance Memory, Inc.

*   All right reserved.

*

*   This program is proprietary and confidential information of

*   Alliance Memory, Inc. and may be used and disclosed only as authorized

*   in a license agreement controlling such use and disclosure.

*

*   Alliance Memory, Inc reserves the right to make any changes to

*   the product herein to improve function or design.

*   Alliance Memory, Inc. does not assume any liability arising out of

*   the application or use of the product herein.

*

*   WARNING: The unlicensed shipping, mailing, or carring of this

*   technical data outside the United States, or the unlicensed

*   disclosure, by whatever means, through visits abroad, or the

*   unlicensed disclosure to foreign national in the United States,

*   may violate the United States criminal law.

*

*   File Name                 : AS8C403625

*   Function                  : 128Kx36 flow-through burst Static RAM

*   Simulation Tool/Version   : Verilog-XL 2.5

*

******************************************************************************/

*******************************************************************************/

`timescale 1ns/10ps

module AS8C403625(A, D, DP, oe_, ce_, cs0, cs1_, lbo_,

gw_, bwe_, bw4_, bw3_, bw2_, bw1_,

adsp_, adsc_, adv_, clk);

initial

begin

$write("\n********************************************************\n");

$write("   AS8C403625                                          \n");

$write("   Rev: 00 July '99                                       \n");

$write("   copyright 1997,1998,1999 by Alliance Memory, Inc.                  \n");

$write("********************************************************\n\n");

end

parameter addr_msb = 16;

parameter mem_top = 131071;

parameter regdelay = 1;

inout [31:0] D;

inout [3:0] DP;

input [addr_msb:0] A;

input oe_, ce_, cs0, cs1_, lbo_, gw_, bwe_, bw4_, bw3_, bw2_, bw1_,

adsp_, adsc_, adv_, clk;

//internal registers for data, address, burst counter

reg [31:0] din;

reg [3:0] dpin;

reg [addr_msb:0] reg_addr;

reg [1:0] brst_cnt;

wire[addr_msb:0] m_ad;

wire[31:0] data_out;

wire[3:0] dp_out;

reg wr_b1_, wr_b2_, wr_b3_, wr_b4_, deselr;

wire check_data =  (~adsc_ & adsp_ & ~ce_ & cs0 & ~cs1_

& (~gw_ | ~bwe_ & (~bw1_ | ~bw2_ | ~bw3_ | ~bw4_)))

| (~deselr & adsc_ & (adsp_ | ce_)

& (~gw_ | ~bwe_ & (~bw1_ | ~bw2_ | ~bw3_ | ~bw4_)));

wire check_addr =   (~adsp_ & ~ce_ & cs0 & ~cs1_)

| ( adsp_ & ~adsc_ & ~ce_ & cs0 & ~cs1_);

specify

specparam

//Clock Parameters

tCYC  = 10.0,  //clock cycle time

tCH   = 4.0,   //clock high time

tCL   = 4.0,   //clock low time

//Output Parameters

tCD   = 8.0,   //clk to data

tCDC  = 2.0,   //output hold from clock

tCLZ  = 0.0,   //CLK to output Low-Z

tCHZ  = 3.5,   //CLK to output Hi-Z

tOE   = 3.5,   //OE to output valid

tOLZ  = 0.0,   //OE to output Hi-Z

tOHZ  = 3.5,   //OE to output Hi-Z

//Set up times

tSA   = 2.0, //address set-up

tSS   = 2.0, //address status set-up

tSD   = 2.0, //data set-up

tSW   = 2.0, //write set-up

tSAV  = 2.0, //address advance set-up

tSC   = 2.0, //chip enable and chip select set-up

//Hold times

tHA   = 0.5, //Address hold

tHS   = 0.5, //address status hold

tHD   = 0.5, //data hold

tHW   = 0.5, //write hold

tHAV  = 0.5, //address advance hold

tHC   = 0.5; //chip enable and chip select hold

(oe_ *> D) = (tOE,tOE,tOHZ,tOLZ,tOHZ,tOLZ); //(01,10,0z,z1,1z,z0)

(clk *> D) = (tCD,tCD,tCHZ,tCLZ,tCHZ,tCLZ); //(01,10,0z,z1,1z,z0)

(oe_ *> DP) = (tOE,tOE,tOHZ,tOLZ,tOHZ,tOLZ); //(01,10,0z,z1,1z,z0)

(clk *> DP) = (tCD,tCD,tCHZ,tCLZ,tCHZ,tCLZ); //(01,10,0z,z1,1z,z0)

//timing checks

$period(posedge clk, tCYC );

$width (posedge clk, tCH );

$width (negedge clk, tCL );

$setuphold(posedge clk, adsp_, tSS, tHS);

$setuphold(posedge clk, adsc_, tSS, tHS);

$setuphold(posedge clk, adv_, tSAV, tHAV);

$setuphold(posedge clk, gw_, tSW, tHW);

$setuphold(posedge clk, bwe_, tSW, tHW);

$setuphold(posedge clk, bw1_, tSW, tHW);

$setuphold(posedge clk, bw2_, tSW, tHW);

$setuphold(posedge clk, bw3_, tSW, tHW);

$setuphold(posedge clk, bw4_, tSW, tHW);

$setuphold(posedge clk, ce_, tSC, tHC);

$setuphold(posedge clk, cs0, tSC, tHC);

$setuphold(posedge clk, cs1_, tSC, tHC);

$setuphold(posedge clk &&& check_addr, A, tSA, tHA);

$setuphold(posedge clk &&& check_data, D, tSD, tHD);

$setuphold(posedge clk &&& check_data, DP, tSD, tHD);

endspecify

/////////////memory array//////////////////////////////////////////////

reg [7:0] memb1[0:mem_top], memb2[0:mem_top], memb3[0:mem_top], memb4[0:mem_top];

reg memb1p[0:mem_top], memb2p[0:mem_top], memb3p[0:mem_top], memb4p[0:mem_top];

wire doe, baddr1, baddr0, dsel;

///////////////////////////////////////////////////////////////////////

//

//Output buffers: using a bufif1 has the same effect as...

//

//	assign D = doe ? data_out : 32'hzzzzzzzz;

//

//It was coded this way to support SPECIFY delays in the specparam section.

//

///////////////////////////////////////////////////////////////////////

bufif1 (D[0],data_out[0],doe);

bufif1 (D[1],data_out[1],doe);

bufif1 (D[2],data_out[2],doe);

bufif1 (D[3],data_out[3],doe);

bufif1 (D[4],data_out[4],doe);

bufif1 (D[5],data_out[5],doe);

bufif1 (D[6],data_out[6],doe);

bufif1 (D[7],data_out[7],doe);

bufif1 (DP[0], dp_out[0],doe);

bufif1 (D[8],data_out[8],doe);

bufif1 (D[9],data_out[9],doe);

bufif1 (D[10],data_out[10],doe);

bufif1 (D[11],data_out[11],doe);

bufif1 (D[12],data_out[12],doe);

bufif1 (D[13],data_out[13],doe);

bufif1 (D[14],data_out[14],doe);

bufif1 (D[15],data_out[15],doe);

bufif1 (DP[1], dp_out[1],doe);

bufif1 (D[16],data_out[16],doe);

bufif1 (D[17],data_out[17],doe);

bufif1 (D[18],data_out[18],doe);

bufif1 (D[19],data_out[19],doe);

bufif1 (D[20],data_out[20],doe);

bufif1 (D[21],data_out[21],doe);

bufif1 (D[22],data_out[22],doe);

bufif1 (D[23],data_out[23],doe);

bufif1 (DP[2], dp_out[2],doe);

bufif1 (D[24],data_out[24],doe);

bufif1 (D[25],data_out[25],doe);

bufif1 (D[26],data_out[26],doe);

bufif1 (D[27],data_out[27],doe);

bufif1 (D[28],data_out[28],doe);

bufif1 (D[29],data_out[29],doe);

bufif1 (D[30],data_out[30],doe);

bufif1 (D[31],data_out[31],doe);

bufif1 (DP[3], dp_out[3],doe);

assign doe = ~deselr & ~oe_ & wr_b1_ & wr_b2_ & wr_b3_ & wr_b4_;

assign dsel = (ce_ | ~cs0 | cs1_);

always @(posedge clk)

begin

if ( ~adsc_ || ( ~adsp_ && ~ce_ ))

deselr <= dsel;

end

//////////////////////////////////////////////////////////////

//

//write enable generation

//

//////////////////////////////////////////////////////////////

always @(posedge clk)

begin

if (  (~adsc_ & adsp_ & ~ce_ & cs0 & ~cs1_ & (~gw_ | ~bwe_ & ~bw1_))

| (~deselr & adsc_ & (adsp_ | ce_) & (~gw_ | ~bwe_ & ~bw1_)))

wr_b1_ <= 0;

else wr_b1_ <= 1;

if (  (~adsc_ & adsp_ & ~ce_ & cs0 & ~cs1_ & (~gw_ | ~bwe_ & ~bw2_))

| (~deselr & adsc_ & (adsp_ | ce_) & (~gw_ | ~bwe_ & ~bw2_)))

wr_b2_ <= 0;

else wr_b2_ <= 1;

if (  (~adsc_ & adsp_ & ~ce_ & cs0 & ~cs1_ & (~gw_ | ~bwe_ & ~bw3_))

| (~deselr & adsc_ & (adsp_ | ce_) & (~gw_ | ~bwe_ & ~bw3_)))

wr_b3_ <= 0;

else wr_b3_ <= 1;

if (  (~adsc_ & adsp_ & ~ce_ & cs0 & ~cs1_ & (~gw_ | ~bwe_ & ~bw4_))

| (~deselr & adsc_ & (adsp_ | ce_) & (~gw_ | ~bwe_ & ~bw4_)))

wr_b4_ <= 0;

else wr_b4_ <= 1;

end

/////////////////////////////////////////////////////////////////

//

//input address register

//

/////////////////////////////////////////////////////////////////

always @(posedge clk)

begin

if (  (~adsp_ & ~ce_ & cs0 & ~cs1_)

| ( adsp_ & ~adsc_ & ~ce_ & cs0 & ~cs1_)) reg_addr[addr_msb:0] <= A[addr_msb:0];

end

//////////////////////////////////////////////////////////////////

//

// burst counter

//

//////////////////////////////////////////////////////////////////

always @(posedge clk)

begin

if (lbo_ & (  (~adsp_ & ~ce_ & cs0 & ~cs1_)

| ( adsp_ & ~adsc_ & ~ce_ & cs0 & ~cs1_))) brst_cnt <= 0;

else if (~lbo_ & (  (~adsp_ & ~ce_ & cs0 & ~cs1_)

| ( adsp_ & ~adsc_ & ~ce_ & cs0 & ~cs1_))) brst_cnt <= A[1:0];

else if ((adsp_ | ce_) & adsc_ & ~adv_) brst_cnt <= brst_cnt + 1;

end

//////////////////////////////////////////////////////////////////////

//

//determine the memory address

//

//////////////////////////////////////////////////////////////////////

assign baddr1 = lbo_ ? (brst_cnt[1] ^ reg_addr[1]) : brst_cnt[1];

assign baddr0 = lbo_ ? (brst_cnt[0] ^ reg_addr[0]) : brst_cnt[0];

assign #regdelay m_ad[addr_msb:0] = {reg_addr[addr_msb:2], baddr1, baddr0};

/////////////////////////////////////////////////////////////////////////

//

//data output

//

/////////////////////////////////////////////////////////////////////////

assign data_out = {memb4[m_ad],memb3[m_ad],memb2[m_ad],memb1[m_ad]};

assign dp_out   = {memb4p[m_ad], memb3p[m_ad], memb2p[m_ad], memb1p[m_ad]};

//////////////////////////////////////////////////////////////////////

//

//data input register

//

//////////////////////////////////////////////////////////////////////

always @(posedge clk)

begin

din <= #regdelay D;

dpin <= #regdelay DP;

end

///////////////////////////////////////////////////////////////////////

//

// write to ram

//

///////////////////////////////////////////////////////////////////////

wire #1 wrb1 = ~wr_b1_ & ~clk;

wire #1 wrb2 = ~wr_b2_ & ~clk;

wire #1 wrb3 = ~wr_b3_ & ~clk;

wire #1 wrb4 = ~wr_b4_ & ~clk;

always @(clk)

begin

if (wrb1) begin

memb1[m_ad] = din[7:0];

memb1p[m_ad] = dpin[0];

end

if (wrb2) begin

memb2[m_ad] = din[15:8];

memb2p[m_ad] = dpin[1];

end

if (wrb3) begin

memb3[m_ad] = din[23:16];

memb3p[m_ad] = dpin[2];

end

if (wrb4) begin

memb4[m_ad] = din[31:24];

memb4p[m_ad] = dpin[3];

end

end

endmodule

/

wire #1 wrb1 = ~wr_b1_ & ~clk;

wire #1 wrb2 = ~wr_b2_ & ~clk;

wire #1 wrb3 = ~wr_b3_ & ~clk;

wire #1 wrb4 = ~wr_b4_ & ~clk;

always @(clk)

begin

if (wrb1) begin

memb1[m_ad] = din[7:0];

memb1p[m_ad] = dpin[0];

end

if (wrb2) begin

memb2[m_ad] = din[15:8];

memb2p[m_ad] = dpin[1];

end

AS8C4036251414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141400006401400003221400000241400000024266140674473720514001506614014141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414ustar1400arjun141414141414141414141414141414141414141414141414141414users1414141414141414141414141414141414141414141414141414140000000140000000141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414141414/*******************************************************************************

******************************************************************************/

`timescale 1ns/10ps

module AS8C403625(A, D, DP, oe_, ce_, cs0, cs1_, lbo_,

gw_, bwe_, bw4_, bw3_, bw2_, bw1_,

adsp_, adsc_, adv_, clk);

initial

begin

$write("\n********************************************************\n");

$write("   AS8C403625                                        \n");

$write("   Rev: 00 July '99                                       \n");

$write("   copyright 1997,1998,1999 by Alliance Memory, Inc.                  \n");

$write("********************************************************\n\n");

end

parameter addr_msb = 16;

parameter mem_top = 131071;

parameter regdelay = 1;

inout [31:0] D;

inout [3:0] DP;

input [addr_msb:0] A;

input oe_, ce_, cs0, cs1_, lbo_, gw_, bwe_, bw4_, bw3_, bw2_, bw1_,

adsp_, adsc_, adv_, clk;

//internal registers for data, address, burst counter

reg [31:0] din;

reg [3:0] dpin;

reg [addr_msb:0] reg_addr;

reg [1:0] brst_cnt;

wire[addr_msb:0] m_ad;

wire[31:0] data_out;

wire[3:0] dp_out;

reg wr_b1_, wr_b2_, wr_b3_, wr_b4_, deselr;

wire check_data =  (~adsc_ & adsp_ & ~ce_ & cs0 & ~cs1_

& (~gw_ | ~bwe_ & (~bw1_ | ~bw2_ | ~bw3_ | ~bw4_)))

| (~deselr & adsc_ & (adsp_ | ce_)

& (~gw_ | ~bwe_ & (~bw1_ | ~bw2_ | ~bw3_ | ~bw4_)));

wire check_addr =   (~adsp_ & ~ce_ & cs0 & ~cs1_)

| ( adsp_ & ~adsc_ & ~ce_ & cs0 & ~cs1_);

specify

specparam

//Clock Parameters

tCYC  = 11.5,  //clock cycle time

tCH   = 4.5,   //clock high time

tCL   = 4.5,   //clock low time

//Output Parameters

tCD   = 8.5,   //clk to data

tCDC  = 2,   //output hold from clock

tCLZ  = 0,   //CLK to output Low-Z

tCHZ  = 3.5,   //CLK to output Hi-Z

tOE   = 3.5,   //OE to output valid

tOLZ  = 0,   //OE to output Hi-Z

tOHZ  = 3.5,   //OE to output Hi-Z

//Set up times

tSA   = 2.0, //address set-up

tSS   = 2.0, //address status set-up

tSD   = 2.0, //data set-up

tSW   = 2.0, //write set-up

tSAV  = 2.0, //address advance set-up

tSC   = 2.0, //chip enable and chip select set-up

//Hold times

tHA   = 0.5, //Address hold

tHS   = 0.5, //address status hold

tHD   = 0.5, //data hold

tHW   = 0.5, //write hold

tHAV  = 0.5, //address advance hold

tHC   = 0.5; //chip enable and chip select hold

(oe_ *> D) = (tOE,tOE,tOHZ,tOLZ,tOHZ,tOLZ); //(01,10,0z,z1,1z,z0)

(clk *> D) = (tCD,tCD,tCHZ,tCLZ,tCHZ,tCLZ); //(01,10,0z,z1,1z,z0)

(oe_ *> DP) = (tOE,tOE,tOHZ,tOLZ,tOHZ,tOLZ); //(01,10,0z,z1,1z,z0)

(clk *> DP) = (tCD,tCD,tCHZ,tCLZ,tCHZ,tCLZ); //(01,10,0z,z1,1z,z0)

//timing checks

$period(posedge clk, tCYC );

$width (posedge clk, tCH );

$width (negedge clk, tCL );

$setuphold(posedge clk, adsp_, tSS, tHS);

$setuphold(posedge clk, adsc_, tSS, tHS);

$setuphold(posedge clk, adv_, tSAV, tHAV);

$setuphold(posedge clk, gw_, tSW, tHW);

$setuphold(posedge clk, bwe_, tSW, tHW);

$setuphold(posedge clk, bw1_, tSW, tHW);

$setuphold(posedge clk, bw2_, tSW, tHW);

$setuphold(posedge clk, bw3_, tSW, tHW);

$setuphold(posedge clk, bw4_, tSW, tHW);

$setuphold(posedge clk, ce_, tSC, tHC);

$setuphold(posedge clk, cs0, tSC, tHC);

$setuphold(posedge clk, cs1_, tSC, tHC);

$setuphold(posedge clk &&& check_addr, A, tSA, tHA);

$setuphold(posedge clk &&& check_data, D, tSD, tHD);

$setuphold(posedge clk &&& check_data, DP, tSD, tHD);

endspecify

/////////////memory array//////////////////////////////////////////////

reg [7:0] memb1[0:mem_top], memb2[0:mem_top], memb3[0:mem_top], memb4[0:mem_top];

reg memb1p[0:mem_top], memb2p[0:mem_top], memb3p[0:mem_top], memb4p[0:mem_top];

wire doe, baddr1, baddr0, dsel;

///////////////////////////////////////////////////////////////////////

//

//Output buffers: using a bufif1 has the same effect as...

//

//	assign D = doe ? data_out : 32'hzzzzzzzz;

//

//It was coded this way to support SPECIFY delays in the specparam section.

//

///////////////////////////////////////////////////////////////////////

bufif1 (D[0],data_out[0],doe);

bufif1 (D[1],data_out[1],doe);

bufif1 (D[2],data_out[2],doe);

bufif1 (D[3],data_out[3],doe);

bufif1 (D[4],data_out[4],doe);

bufif1 (D[5],data_out[5],doe);

bufif1 (D[6],data_out[6],doe);

bufif1 (D[7],data_out[7],doe);

bufif1 (DP[0], dp_out[0],doe);

bufif1 (D[8],data_out[8],doe);

bufif1 (D[9],data_out[9],doe);

bufif1 (D[10],data_out[10],doe);

bufif1 (D[11],data_out[11],doe);

bufif1 (D[12],data_out[12],doe);

bufif1 (D[13],data_out[13],doe);

bufif1 (D[14],data_out[14],doe);

bufif1 (D[15],data_out[15],doe);

bufif1 (DP[1], dp_out[1],doe);

bufif1 (D[16],data_out[16],doe);

bufif1 (D[17],data_out[17],doe);

bufif1 (D[18],data_out[18],doe);

bufif1 (D[19],data_out[19],doe);

bufif1 (D[20],data_out[20],doe);

bufif1 (D[21],data_out[21],doe);

bufif1 (D[22],data_out[22],doe);

bufif1 (D[23],data_out[23],doe);

bufif1 (DP[2], dp_out[2],doe);

bufif1 (D[24],data_out[24],doe);

bufif1 (D[25],data_out[25],doe);

bufif1 (D[26],data_out[26],doe);

bufif1 (D[27],data_out[27],doe);

bufif1 (D[28],data_out[28],doe);

bufif1 (D[29],data_out[29],doe);

bufif1 (D[30],data_out[30],doe);

bufif1 (D[31],data_out[31],doe);

bufif1 (DP[3], dp_out[3],doe);

assign doe = ~deselr & ~oe_ & wr_b1_ & wr_b2_ & wr_b3_ & wr_b4_;

assign dsel = (ce_ | ~cs0 | cs1_);

always @(posedge clk)

begin

if ( ~adsc_ || ( ~adsp_ && ~ce_ ))

deselr <= dsel;

end

//////////////////////////////////////////////////////////////

//

//write enable generation

//

//////////////////////////////////////////////////////////////

always @(posedge clk)

begin

if (  (~adsc_ & adsp_ & ~ce_ & cs0 & ~cs1_ & (~gw_ | ~bwe_ & ~bw1_))

| (~deselr & adsc_ & (adsp_ | ce_) & (~gw_ | ~bwe_ & ~bw1_)))

wr_b1_ <= 0;

else wr_b1_ <= 1;

if (  (~adsc_ & adsp_ & ~ce_ & cs0 & ~cs1_ & (~gw_ | ~bwe_ & ~bw2_))

| (~deselr & adsc_ & (adsp_ | ce_) & (~gw_ | ~bwe_ & ~bw2_)))

wr_b2_ <= 0;

else wr_b2_ <= 1;

if (  (~adsc_ & adsp_ & ~ce_ & cs0 & ~cs1_ & (~gw_ | ~bwe_ & ~bw3_))

| (~deselr & adsc_ & (adsp_ | ce_) & (~gw_ | ~bwe_ & ~bw3_)))

wr_b3_ <= 0;

else wr_b3_ <= 1;

if (  (~adsc_ & adsp_ & ~ce_ & cs0 & ~cs1_ & (~gw_ | ~bwe_ & ~bw4_))

| (~deselr & adsc_ & (adsp_ | ce_) & (~gw_ | ~bwe_ & ~bw4_)))

wr_b4_ <= 0;

else wr_b4_ <= 1;

end

/////////////////////////////////////////////////////////////////

//

//input address register

//

/////////////////////////////////////////////////////////////////

always @(posedge clk)

begin

if (  (~adsp_ & ~ce_ & cs0 & ~cs1_)

| ( adsp_ & ~adsc_ & ~ce_ & cs0 & ~cs1_)) reg_addr[addr_msb:0] <= A[addr_msb:0];

end

//////////////////////////////////////////////////////////////////

//

// burst counter

//

//////////////////////////////////////////////////////////////////

always @(posedge clk)

begin

if (lbo_ & (  (~adsp_ & ~ce_ & cs0 & ~cs1_)

| ( adsp_ & ~adsc_ & ~ce_ & cs0 & ~cs1_))) brst_cnt <= 0;

else if (~lbo_ & (  (~adsp_ & ~ce_ & cs0 & ~cs1_)

| ( adsp_ & ~adsc_ & ~ce_ & cs0 & ~cs1_))) brst_cnt <= A[1:0];

else if ((adsp_ | ce_) & adsc_ & ~adv_) brst_cnt <= brst_cnt + 1;

end

//////////////////////////////////////////////////////////////////////

//

//determine the memory address

//

//////////////////////////////////////////////////////////////////////

assign baddr1 = lbo_ ? (brst_cnt[1] ^ reg_addr[1]) : brst_cnt[1];

assign baddr0 = lbo_ ? (brst_cnt[0] ^ reg_addr[0]) : brst_cnt[0];

assign #regdelay m_ad[addr_msb:0] = {reg_addr[addr_msb:2], baddr1, baddr0};

/////////////////////////////////////////////////////////////////////////

//

//data output

//

/////////////////////////////////////////////////////////////////////////

assign data_out = {memb4[m_ad],memb3[m_ad],memb2[m_ad],memb1[m_ad]};

assign dp_out   = {memb4p[m_ad], memb3p[m_ad], memb2p[m_ad], memb1p[m_ad]};

//////////////////////////////////////////////////////////////////////

//

//data input register

//

//////////////////////////////////////////////////////////////////////

always @(posedge clk)

begin

din <= #regdelay D;

dpin <= #regdelay DP;

end

///////////////////////////////////////////////////////////////////////

//

// write to ram

//

///////////////////////////////////////////////////////////////////////

wire #1 wrb1 = ~wr_b1_ & ~clk;

wire #1 wrb2 = ~wr_b2_ & ~clk;

wire #1 wrb3 = ~wr_b3_ & ~clk;

wire #1 wrb4 = ~wr_b4_ & ~clk;

always @(clk)

begin

if (wrb1) begin

memb1[m_ad] = din[7:0];

memb1p[m_ad] = dpin[0];

end

if (wrb2) begin

memb2[m_ad] = din[15:8];

memb2p[m_ad] = dpin[1];

end

if (wrb3) begin

memb3[m_ad] = din[23:16];

memb3p[m_ad] = dpin[2];

end

if (wrb4) begin

memb4[m_ad] = din[31:24];

memb4p[m_ad] = dpin[3];

end

end

endmodule

/

wire #1 wrb1 = ~wr_b1_ & ~clk;

wire #1 wrb2 = ~wr_b2_ & ~clk;

wire #1 wrb3 = ~wr_b3_ & ~clk;

wire #1 wrb4 = ~wr_b4_ & ~clk;

always @(clk)

begin

if (wrb1) begin

memb1[m_ad] = din[7:0];

memb1p[m_ad] = dpin[0];

end

if (wrb2) begin

memb2[m_ad] = din[15:8];

memb2p[m_ad] = dpin[1];

end

if (wrAS8C403625_128Kx36_PBSRAM_test.v2020202020202020202020202020202020202020202020202020202020202020202020202020202020202020202020202020202020202020202020202020202000006402000003222000000242100000073602210674473720521001701121021212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121ustar2100arjun212121212121212121212121212121212121212121212121212121users2121212121212121212121212121212121212121212121212121210000000210000000212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121212121///////////////////////////////////////////////////////////

//

//    Test fixture for Alliance Memory 4Meg Synchronous Burst SRAMs

//              (for 128K x 36 configurations)

//

//////////////////////////////////////////////////////////

`timescale 1ns / 10ps

`define Max 16

`define Max1 8

`define Max2 16

module main;

parameter addr_msb = 16;

/////// Remove comments for specific device under test ////

//////////////////////////////////////////////

//

//Pipelined sync burst SRAMs

//

/////////////////////////////////////////////

/////// 3.3v I/O ////////////

//parameter pipe = 1, Tcyc = 7.5, Tsu = 1.5, Tdh = 0.5, Tcd = 4.2, Toe = 4.2;

//

//parameter pipe = 1, Tcyc = 6.7, Tsu = 1.5, Tdh = 0.5, Tcd = 3.8, Toe = 3.8;

//

//parameter pipe = 1, Tcyc = 6.0, Tsu = 1.5, Tdh = 0.5, Tcd = 3.5, Toe = 3.5;

//

//parameter pipe = 1, Tcyc = 5.5, Tsu = 1.5, Tdh = 0.5, Tcd = 3.3, Toe = 3.3;

//

//parameter pipe = 1, Tcyc = 5.0, Tsu = 1.2, Tdh = 0.4, Tcd = 3.1, Toe = 3.1;

//

/////// 2.5v I/O ////////////

//parameter pipe = 1, Tcyc = 7.5, Tsu = 1.5, Tdh = 0.5, Tcd = 4.2, Toe = 4.2;

//

//parameter pipe = 1, Tcyc = 6.7, Tsu = 1.5, Tdh = 0.5, Tcd = 3.8, Toe = 3.8;

//

//parameter pipe = 1, Tcyc = 6.0, Tsu = 1.5, Tdh = 0.5, Tcd = 3.5, Toe = 3.5;

//

//parameter pipe = 1, Tcyc = 5.5, Tsu = 1.5, Tdh = 0.5, Tcd = 3.3, Toe = 3.3;

//

//parameter pipe = 1, Tcyc = 5.0, Tsu = 1.2, Tdh = 0.4, Tcd = 3.1, Toe = 3.1;

//

//////////////////////////////////////////////

//

//Flow-through sync burst SRAMs

//

//////////////////////////////////////////////

/////// 3.3v I/O ////////////

parameter pipe = 0, Tcyc = 11.5, Tsu = 2.0, Tdh = 0.5, Tcd = 8.5, Toe = 3.5;

//parameter pipe = 0, Tcyc = 10.0, Tsu = 2.0, Tdh = 0.5, Tcd = 8.0, Toe = 3.5;

//

//parameter pipe = 0, Tcyc = 8.5,  Tsu = 1.5, Tdh = 0.5, Tcd = 7.5, Toe = 3.5;

//

/////// 2.5v I/O ////////////

//parameter pipe = 0, Tcyc = 11.5, Tsu = 2.0, Tdh = 0.5, Tcd = 8.5, Toe = 3.5;

//

//parameter pipe = 0, Tcyc = 10.0, Tsu = 2.0, Tdh = 0.5, Tcd = 8.0, Toe = 3.5;

//

//parameter pipe = 0, Tcyc = 8.5,  Tsu = 1.5, Tdh = 0.5, Tcd = 7.5, Toe = 3.5;

//

reg   [addr_msb:0] A;

reg          CLK;

reg          ADSP_;

reg          ADV_;

reg          LBO_;

reg          ADSC_;

reg    [4:1] BW_;

reg          BWE_;

reg          GW_;

reg          CE_;

reg          CS0;

reg          CS1_;

reg          OE_;

reg   [35:0] DataOut;

reg   [35:0] TempReg;

reg   [35:0] DQ;

wire  [31:0] DQbus = {DQ[34:27], DQ[25:18], DQ[16:9], DQ[7:0]};

wire  [4:1]  DQPbus = {DQ[35], DQ[26], DQ[17], DQ[8]};

reg   [35:0] Dstore[0:`Max-1];              //temp data store

reg   [35:0] data;

reg   [addr_msb:0] lastaddr;

reg          tempcs1_;

reg          tempcs0;

reg          tempce_;

reg   [35:0] RandomData[0:`Max-1];

reg   [35:0] BurstData[0:`Max-1];

//internal

reg check_data_m1, qual_ads;

reg check_data;

integer   i,j,addrb,counter,

result;

// Output files

initial begin

$recordfile ("idt_sram_3577.trn");

$recordvars;

//  $dumpfile ("idt_sram.vcd");

//  $dumpvars;

result = $fopen("idt_sram.res"); if (result == 0) $finish;

end

always begin

@(posedge CLK)

$fdisplay(result,

"%b",  ADSC_,

"%b",  ADSP_,

"%b",  BWE_,

"%b",  CE_,

"%b",  CS0,

"%b",  CS1_,

"%b",  LBO_,

"%b",  OE_,

"%b",  BW_,  // 4 bits

"%b",  ADV_,

"%b ", GW_,

"%h ", {DQPbus[4], DQbus[31:24], DQPbus[3], DQbus[23:16], DQPbus[2], DQbus[15:8], DQPbus[1], DQbus[7:0]},

"%h ", A,

" ;%d", $stime

);

end

initial begin

ADSC_ = 1;

ADSP_ = 1;

BWE_  = 1;

CE_   = 0;

CS0   = 1;

CS1_  = 0;

LBO_  = 0;

OE_   = 1;

CLK   = 0;

BW_   = 4'hf;

ADV_  = 1;

GW_   = 1;

counter = 0;

for (i=0;i<`Max;i=i+1) begin           // Generate random data for testing

RandomData[i] = $random;

end

//****************

//disable_ce;

//disable_cs0;

//####

init;

$display($time,"(1)  write        adsp_ = 0");

for(i=0; i<`Max1; i=i+1) begin

write(i,i,0,1,0,0,0,1);                  //addr,data,adsp_,adsc_,gw_,ce_,cs1_,cs0

end

$display($time,"     read         adsp_ = 0");

for(i=0; i<`Max1; i=i+1) begin

read(i,0,1,0,0,1);                       //addr,adsp_,adsc_,ce_,cs1_,cs0

end

dummy_cyc(1);

//####

init;

$display($time,"(2)  write        adsc_ = 0");

for(i=0; i<`Max1; i=i+1) begin

write(i,i,1,0,0,0,0,1);                  //addr,data,adsp_,adsc_,gw_,ce_,cs1_,cs0

end

$display($time,"     read         adsc_ = 0");

for(i=0; i<`Max1; i=i+1) begin

read(i,1,0,0,0,1);                       //addr,adsp_,adsc_,ce_,cs1_,cs0

end

//####

init;

$display($time,"(3)  write        adsp_ = 0");

for(i=0; i<`Max1; i=i+1) begin

write(i,i,0,1,0,0,0,1);                  //addr,data,adsp_,adsc_,gw_,ce_,cs1_,cs0

end

$display($time,"     read         adsp_ = 0 cs1_ = 1 - every other cyc");

for(i=0; i<`Max1; i=i+2) begin

read(i,0,1,0,1,1);                       //addr,adsp_,adsc_,ce_,cs1_,cs0

read(i+1,0,1,0,0,1);                     //addr,adsp_,adsc_,ce_,cs1_,cs0

end

//####

init;

$display($time,"(4)  write/read   adsp_ = 0");

for(i=0; i<`Max1; i=i+1) begin

write(i,i,0,1,0,0,0,1);                  //addr,data,adsp_,adsc_,gw_,ce_,cs1_,cs0

read(i,0,1,0,0,1);                       //addr,adsp_,adsc_,ce_,cs1_,cs0

end

//####

init;

$display($time,"(5)  write/read   adsc_ = 0");

for(i=0; i<`Max1; i=i+1) begin

write(i,i,1,0,0,0,0,1);                  //addr,data,adsp_,adsc_,gw_,ce_,cs1_,cs0

read(i,1,0,0,0,1);                       //addr,adsp_,adsc_,ce_,cs1_,cs0

dummy_cyc(1);

end

//####

init;

$display($time,"(6)  burst_write  adsp_ = 0");

for(i=0; i<`Max2; i=i+4) begin

burst_write(i,i,0,1,0,0,0,1,4);          //addr,data,adsp_,adsc_,gw_,ce_,cs1_,cs0,nburst

end

$display($time,"     burst_read   adsp_ = 0");

for(i=0; i<`Max2; i=i+4) begin

burst_read(i,0,1,0,0,1,4);               //addr,adsp_,adsc_,ce_,cs1_,cs0,nburst

end

dummy_cyc(1);

//####

init;

$display($time,"(7)  burst_write  adsc_ = 0");

for(i=0; i<`Max2; i=i+4) begin

burst_write(i,i,1,0,0,0,0,1,4);          //addr,data,adsp_,adsc_,gw_,ce_,cs1_,cs0,nburst

end

$display($time,"     burst_read   adsc_ = 0");

for(i=0; i<`Max2; i=i+4) begin

burst_read(i,1,0,0,0,1,4);               //addr,adsp_,adsc_,ce_,cs1_,cs0,nburst

end

//####

init;

$display($time,"(8)  write        adsp_ = 0 cs1_ = 1 - every other cyc");

for(i=0; i<`Max1; i=i+2) begin

write(i,i,0,1,0,0,0,1);                  //addr,data,adsp_,adsc_,gw_,ce_,cs1_,cs0

write(i+1,9,0,1,0,0,1,1);                //addr,data,adsp_,adsc_,gw_,ce_,cs1_,cs0

end

$display($time,"     read         adsp_ = 0");

for(i=0; i<`Max1; i=i+2) begin

read(i,0,1,0,0,1);                       //addr,adsp_,adsc_,ce_,cs1_,cs0

end

//####

init;

$display($time,"(9)  write        adsp_ = 0 cs0  = 1 - every other cyc");

for(i=0; i<`Max1; i=i+2) begin

write(i,i,0,1,0,0,0,1);                  //addr,data,adsp_,adsc_,gw_,ce_,cs1_,cs0

write(i+1,9,0,1,0,0,0,0);                //addr,data,adsp_,adsc_,gw_,ce_,cs1_,cs0

end

$display($time,"     read         adsc_ = 0");

for(i=0; i<`Max1; i=i+2) begin

read(i,1,0,0,0,1);                       //addr,adsp_,adsc_,ce_,cs1_,cs0

end

//####

init;

$display($time,"(10) write        adsp_ = 0 ce_  = 1 - every other cyc");

for(i=0; i<`Max1; i=i+2) begin

write(i,i,0,1,0,0,0,1);                  //addr,data,adsp_,adsc_,gw_,ce_,cs1_,cs0

write(i+1,i+1,0,1,0,1,0,1);              //addr,data,adsp_,adsc_,gw_,ce_,cs1_,cs0

end

write(i,i,0,1,0,0,0,1);                  //this will write last address to Dstore

$display($time,"     read         adsp_ = 0");

for(i=0; i<`Max1; i=i+2) begin

read(i,0,1,0,0,1);                       //addr,adsp_,adsc_,ce_,cs1_,cs0

end

//####

init;

$display($time,"(11) write        adsc_ = 0 ce_  = 1 - every other cyc");

for(i=0; i<`Max1; i=i+2) begin

write(i,i,1,0,0,0,0,1);                  //addr,data,adsp_,adsc_,gw_,ce_,cs1_,cs0

write(i+1,i+1,1,0,0,1,0,1);              //addr,data,adsp_,adsc_,gw_,ce_,cs1_,cs0

end

write(i,i,1,0,0,0,0,1);                  //this will write last address to Dstore

$display($time,"     read         adsc_ = 0");

for(i=0; i<`Max1; i=i+2) begin

read(i,1,0,0,0,1);                       //addr,adsp_,adsc_,ce_,cs1_,cs0

end

//####

init;

$display($time,"(12) burst_write_adv  adsc_ = 0 adv_ = 1 - 2nd cyc");

for(i=0; i<`Max2; i=i+4) begin

burst_write_adv(i,i,1,0,0,0,0,1,1,0);     //addr,data,adsp_,adsc_,gw_,ce_,cs1_,cs0,adv_,tempcounter

burst_write_adv(i,i,1,0,0,0,0,1,1,1);

burst_write_adv(i+1,i+1,1,0,0,0,0,1,0,2);

burst_write_adv(i+2,i+2,1,0,0,0,0,1,0,3);

burst_write_adv(i+3,i+3,1,0,0,0,0,1,0,4);

end

$display($time,"     burst_read   adsc_ = 0");

for(i=0; i<`Max2; i=i+4) begin

burst_read(i,1,0,0,0,1,4);               //addr,adsp_,adsc_,ce_,cs1_,cs0,nburst

end

//####

init;

$display($time,"(13) burst_write_adv  adsp_ = 0 adv_ = 1 - 2nd cyc");

for(i=0; i<`Max2; i=i+4) begin

burst_write_adv(i,i,0,1,0,0,0,1,1,0);     //addr,data,adsp_,adsc_,gw_,ce_,cs1_,cs0,adv_,tempcounter

burst_write_adv(i,i,0,0,0,1,0,1,1,1);

burst_write_adv(i+1,i+1,0,1,0,0,0,1,0,2);

burst_write_adv(i+2,i+2,0,1,0,0,0,1,0,3);

burst_write_adv(i+3,i+3,0,1,0,0,0,1,0,4);

end

$display($time,"     burst_read   adsc_ = 0");

for(i=0; i<`Max2; i=i+4) begin

burst_read(i,1,0,0,0,1,4);               //addr,adsp_,adsc_,ce_,cs1_,cs0,nburst

end

//####

init;

$display($time,"(14) burst_write  adsp_ = 0");

for(i=0; i<`Max2; i=i+4) begin

burst_write(i,i,0,1,0,0,0,1,4);          //addr,data,adsp_,adsc_,gw_,ce_,cs1_,cs0,nburst

end

$display($time,"     burst_read_adv   adsp_ = 0 adv_ = 1 - 3rd cyc");

for(i=0; i<`Max2; i=i+4) begin

burst_read_adv(i,  0,1,0,0,1,1,0);       //addr,adsp_,adsc_,ce_,cs1_,cs0,adv_,tempcounter

burst_read_adv(i+1,1,1,0,0,1,0,1);

burst_read_adv(i+2,1,1,0,0,1,1,2);

burst_read_adv(i+3,1,1,0,0,1,0,3);

end

//####

init;

$display($time,"(15) burst_write  adsp_ = 0");

for(i=0; i<`Max2; i=i+4) begin

burst_write(i,i,0,1,0,0,0,1,4);          //addr,data,adsp_,adsc_,gw_,ce_,cs1_,cs0,nburst

end

$display($time,"     burst_read_adv   adsp_=1/ce_=0 - 2/3 cyc, adsp = 0/ce_=1 - 4/5 cyc");

for(i=0; i<`Max2; i=i+4) begin

burst_read_adv(i,  0,1,0,0,1,1,0);       //addr,adsp_,adsc_,ce_,cs1_,cs0,adv_,tempcounter

burst_read_adv(i+1,1,1,0,0,1,0,1);

burst_read_adv(i+2,1,1,0,0,1,0,2);

burst_read_adv(i+3,0,1,1,0,1,0,3);

burst_read_adv(i,  0,1,1,0,1,0,4);

end

//####

@( negedge CLK );

@( negedge CLK );

@( negedge CLK );

//*****************

CE_ = 0;

CS0 = 1;

CS1_ = 0;

$display($time,,"Simple read/write test");

for (i=0;i<`Max;i=i+1) begin      // Test straight write/read

write_random(i, RandomData[i]);

$display($time,,"Simple read test");

read_random(i, DataOut, RandomData[i]);

end

$display($time,,"CE_ disable - random data");

read_random(3, DataOut, RandomData[3]);

disable_ce;

read_random(7, DataOut, RandomData[7]);

disable_cs0;

read_random(2, DataOut, RandomData[2]);

for (i=0;i<`Max;i=i+1) begin      // Fill RAM with zero's

write_random(i, 0);

end

$display($time,,"Byte mode read/write test - random data");

//  GW_ = 1;                        // Disable global write

//  BWE_  = 0;                      // Enable byte write

for (i=0;i<`Max;i=i+1) begin    // Test byte write/read

BW_ = $random;

TempReg = RandomData[i];

byte_write_random(i, TempReg);

if ( BW_[1] == 1 ) TempReg[8:0] = 0;

if ( BW_[2] == 1 ) TempReg[17:9] = 0;

if ( BW_[3] == 1 ) TempReg[26:18] = 0;

if ( BW_[4] == 1 ) TempReg[35:27] = 0;

read_random(i, DataOut, TempReg);

end

BWE_  = 1;                      // Disable byte write

// Test burst mode write/read

$display($time,,"Burst mode read/write test - random data");

for (i=0;i<`Max;i=i+1) begin      // Test byte write/read

BurstData[i] = RandomData[i];

end

GW_ = 0;                       // Enable global write

for (i=0;i<`Max;i=i+4) begin   // Write data from BurstData buffer

burst_write_random(i,4);

end

GW_ = 1;                       // Disable global write

for (i=0;j<`Max;i=i+1) begin   // Clear data buffer

BurstData[i] = 0;

end

for (i=0;i<`Max;i=i+4) begin

burst_read_random(i,4);

//    for (j=i;j<i+4;j=j+1) begin      // verify read data

//      if ( BurstData[j] != RandomData[j] )

//          $display("%d  Burst error: Addr %h Exp %h Act %h", $stime, j, RandomData[j], BurstData[j]);

//    end

end

burst_wrap_random(0);

disable_ce;

burst_rd_pipe_random(0,4);

$finish;

end

/////////////////////////////////////////////////////////////////

always @(posedge CLK) begin

if ((~ADSC_ | ~ADSP_) & ~CE_ & CS0 & ~CS1_) qual_ads <= #1 1;

else qual_ads <= #1 0;

check_data_m1 <= #1 ~ADV_;

if (pipe == 0) check_data = #1 (qual_ads | ~ADV_);

else check_data = #1 (qual_ads | check_data_m1);

end

always #(Tcyc/2) CLK = ~CLK;

`device  dut (

.A        (A),

.D        (DQbus),

.DP       (DQPbus),

.oe_      (OE_),

.ce_      (CE_),

.cs0      (CS0),

.cs1_     (CS1_),

.lbo_     (LBO_),

.gw_      (GW_),

.bwe_     (BWE_),

.bw4_     (BW_[4]),

.bw3_     (BW_[3]),

.bw2_     (BW_[2]),

.bw1_     (BW_[1]),

.adsp_    (ADSP_),

.adsc_    (ADSC_),

.adv_     (ADV_),

.clk      (CLK)

);

//================ test bench tasks

task disable_ce;

begin

OE_ = 0;

if (CLK)

@( negedge CLK );

ADSC_ = 0;

CE_   = 1;

@( posedge CLK );

@( negedge CLK );

ADSC_ = 1;

CE_   = 0;

end

endtask

task disable_cs0;

begin

OE_ = 0;

if (CLK)

@( negedge CLK );

ADSP_ = 0;

CS0   = 0;

@( posedge CLK );

@( negedge CLK );

ADSP_ = 1;

CS0   = 1;

end

endtask

task dummy_cyc;

input oe;

begin

@(posedge CLK);

@(negedge CLK);

#Tcd;

OE_ = oe;

end

endtask

task init;

begin

for(i=0; i<`Max2; i=i+1) begin         // fill memory with 0 data

write(i,0,0,1,0,0,0,1);              // addr,data,adsp_,adsc_,gw_,ce_,cs1_,cs0

Dstore[i] = 36'hx;                   // fill temp memory with xx data

end

end

endtask

task read;             // ADSP|ADSP controlled PL - adsp_/adsc_ 2cycle read

input  [16:0] addr;    // ADSP|ADSP controlled FT - adsp_/adsc_ 1cycle read

input  adsp_;

input  adsc_;

input  ce_;

input  cs1_;

input  cs0;

begin

@( negedge CLK );

#(Tcyc/2 - Tsu);

A     = addr;

ADV_  = 1;

GW_   = 1;

BWE_  = 1;

ADSP_ = adsp_;

ADSC_ = adsc_;

CE_   = ce_;

CS1_  = cs1_;

CS0   = cs0;

assign data = {DQPbus[4], DQbus[31:24], DQPbus[3], DQbus[23:16], DQPbus[2],

DQbus[15:8], DQPbus[1], DQbus[7:0]};

@( posedge CLK );           // SRAM latches Address and begins internal read

tempcs0  <= cs0; tempcs1_  <= cs1_; tempce_  <= ce_;

lastaddr <= addr;

A <= #Tdh 17'hz;

ADSP_ <= #Tdh 1;

ADSC_ <= #Tdh 1;

CE_ <= #Tdh ~ce_;

CS1_<= #Tdh ~cs1_;

CS0 <= #Tdh ~cs0;

if(pipe == 1)

OE_ <= #(Tcyc+Tcd-Toe) 0;

else if(pipe == 0)

OE_ <= #(Tcd-Toe) 0;

if(counter != 0)

if ( data !== Dstore[lastaddr] ) begin

if (~tempcs0 & ~tempce_ & tempcs1_)

$display("%d Read error: Addr %h Exp %h Act %h", $stime, lastaddr,

Dstore[lastaddr], data);

end

DQ = 36'hz;

#(Tcyc/2);

counter = counter+1;

end

endtask

task burst_read;       // ADSP|ADSC controlled - adsp/adsc 3-1-1-1 PL read

input  [16:0] addr;    //                        adsp/adsc 2-1-1-1 FT read

input  adsp_;

input  adsc_;

input  ce_;

input  cs1_;

input  cs0;

input  [3:0] nburst;

integer tempaddr,tempcounter;

begin

tempcounter = 0;

for (tempaddr=addr; tempaddr<addr+nburst; tempaddr=tempaddr+1) begin

@( negedge CLK );

if (tempaddr == addr) begin           // 1st address

#(Tcyc/2 - Tsu);

A     = addr;

GW_   = 1;

BWE_  = 1;

ADSP_ = adsp_;

ADSC_ = adsc_;

ADV_  = 1;

CE_   = ce_;

CS1_  = cs1_;

CS0   = cs0;

end

else begin

#(Tcyc/2 - Tsu);                  // after 2nd address

A     = 17'hz;

ADV_ = 0;

end

assign data = {DQPbus[4], DQbus[31:24], DQPbus[3], DQbus[23:16], DQPbus[2],

DQbus[15:8], DQPbus[1], DQbus[7:0]};

@( posedge CLK );      // SRAM latches Address and begins internal read

lastaddr <= #(Tcyc) tempaddr;

if (tempaddr == addr) begin           // 1st address

A <= #Tdh 17'hz;

ADSP_ <= #Tdh 1;

ADSC_ <= #Tdh 1;

CE_   <= #Tdh ~ce_;

CS1_  <= #Tdh ~cs1_;

CS0   <= #Tdh ~cs0;

if(pipe == 1)

OE_ <= #(Tcyc+Tcd-Toe) 0;

if(pipe == 0)

OE_ <= #(Tcd-Toe) 0;

end

else begin                            // after 2nd address

ADV_  <= #Tdh 1;

end

if(pipe == 1)

if(tempcounter > 1 )

if ( data !== Dstore[lastaddr] ) begin

$display("%d Read error: Addr %h Exp %h Act %h", $stime, lastaddr,

Dstore[lastaddr], data);

end

else if(pipe == 0)

if(tempcounter > 0 )

if ( data !== Dstore[lastaddr] ) begin

$display("%d Read error: Addr %h Exp %h Act %h", $stime, lastaddr,

Dstore[lastaddr], data);

end

DQ = 36'hz;

#Tdh;

tempcounter = tempcounter+1;

end

end

endtask

task burst_read_adv;       // ADSP|ADSC controlled - adsp/adsc 3-1-1-1 PL read

input  [16:0] addr;        //                        adsp/adsc 2-1-1-1 FT read

input  adsp_;

input  adsc_;

input  ce_;

input  cs1_;

input  cs0;

input  adv_;

input  [3:0] tempcounter;

begin

@( negedge CLK );

if (tempcounter == 0) begin            // 1st address

#(Tcyc/2 - Tsu);

A     = addr;

GW_   = 1;

BWE_  = 1;

ADSP_ = adsp_;

ADSC_ = adsc_;

ADV_  = adv_;

CE_   = ce_;

CS1_  = cs1_;

CS0   = cs0;

end

else begin

#(Tcyc/2 - Tsu);                  // after 2nd address

A     = 17'hz;

ADSP_ = adsp_;

ADSC_ = adsc_;

CE_   = ce_;

ADV_ = adv_;

end

assign data = {DQPbus[4], DQbus[31:24], DQPbus[3], DQbus[23:16], DQPbus[2],

DQbus[15:8], DQPbus[1], DQbus[7:0]};

@( posedge CLK );      // SRAM latches Address and begins internal read

lastaddr <= #(Tcyc) addr;

if (tempcounter == 0) begin           // 1st address

A <= #Tdh 17'hz;

ADSP_ <= #Tdh ~adsp_;

ADSC_ <= #Tdh ~adsc_;

CE_   <= #Tdh ~ce_;

CS1_  <= #Tdh ~cs1_;

CS0   <= #Tdh ~cs0;

if(pipe == 1)

OE_ <= #(Tcyc+Tcd-Toe) 0;

if(pipe == 0)

OE_ <= #(Tcd-Toe) 0;

end

else begin                            // after 2nd address

ADSP_ <= #Tdh ~adsp_;

ADSC_ <= #Tdh ~adsc_;

CE_   <= #Tdh ~ce_;

ADV_  <= #Tdh ~adv_;

end

if(pipe == 1)

if(tempcounter > 1 )

if ( data !== Dstore[lastaddr] ) begin

$display("%d Read error: Addr %h Exp %h Act %h", $stime, lastaddr,

Dstore[lastaddr], data);

end

else if(pipe == 0)

if(tempcounter > 0 )

if ( data !== Dstore[lastaddr] ) begin

$display("%d Read error: Addr %h Exp %h Act %h", $stime, lastaddr,

Dstore[lastaddr], data);

end

DQ = 36'hz;

end

endtask

task read_random;

input  [16:0] addr;

output [35:0] data;

input  [35:0] exp;

begin

if (CLK )

@( negedge CLK );

//    DQ = 36'hz;

ADV_  = 1;

A = addr;

ADSP_ = 0;

@( posedge CLK );      // SRAM latches Address and begins internal read

@( negedge CLK );

ADSP_ = 1;

OE_   = 0;

if (pipe == 1) @( posedge CLK );      // SRAM begins placing data onto bus

@( posedge CLK );      // Data sampled by reading device

// Hopefully the SRAM has an output hold time

data = {DQPbus[4], DQbus[31:24], DQPbus[3], DQbus[23:16], DQPbus[2], DQbus[15:8], DQPbus[1], DQbus[7:0]};

if ( data !== exp )

$display("%d Read_random error: Addr %h Exp %h Act %h", $stime, addr, exp, data);

@( negedge CLK );

OE_   = 1;

end

endtask

task burst_read_random;

input  [16:0] addr;

input  [35:0] n;

integer       i;

begin

DQ = 36'hz;

if ( CLK )

@( negedge CLK );

#1 A = addr;

ADSP_ = 0;

@( posedge CLK );           // Address latched by SRAM, begins internal read

#(Tcyc/2) ADSP_ = 1;       // SRAM starts driving bus (flow-through)

#1 OE_   = 0;

ADV_  = 0;

if (pipe == 1) @(posedge CLK); //SRAM starts driving bus (pipelined)

for (i=addr;i<addr+n;i=i) begin

@( posedge CLK ) begin

if (check_data == 1)

BurstData[i] = {DQPbus[4], DQbus[31:24], DQPbus[3], DQbus[23:16], DQPbus[2], DQbus[15:8], DQPbus[1], DQbus[7:0]};

if ( BurstData[i] !== RandomData[i] && check_data == 1 )

$display("%d task burst_read_random read error: Addr %h Exp %h Act %h", $stime, i, RandomData[i], BurstData[i]);

end

@( negedge CLK );

if (check_data) i=i+1;

if ( ($random & 3) === 2'b11 ) // suspend burst 25% of the time

ADV_ = 1;

else begin

ADV_ = 0;

end

end

OE_   = 1;

ADV_  = 1;

end

endtask

task burst_wrap_random; //checks burst counter wrap-around

input  [16:0] addr;

integer i,j;

begin

DQ = 36'hz;

if ( CLK )

@( negedge CLK );

#1 A = addr;

ADSP_ = 0;

@(posedge CLK);           // Address latched by SRAM, begins internal read

#(Tcyc/2) ADSP_ = 1;

#1 OE_   = 0;

ADV_  = 0;

if (pipe == 1) @(posedge CLK);

for (i=0;i<2;i=i+1) begin

for (j=0;j<4;j=j+1) begin

@( posedge CLK ) begin

if (check_data == 1)

BurstData[j] = {DQPbus[4], DQbus[31:24], DQPbus[3], DQbus[23:16], DQPbus[2], DQbus[15:8], DQPbus[1], DQbus[7:0]};

if ( BurstData[j] !== RandomData[j] && check_data == 1 )

$display("%d task burst_wrap_random read error: Addr %h Exp %h Act %h", $stime, i, RandomData[i], BurstData[i]);

end

end

end

#1 OE_   = 1;

ADV_  = 1;

end

endtask

task burst_rd_pipe_random;

input  [16:0] addr1;

input  [16:0] addr2;

integer       i;

begin

DQ = 36'hz;

for (i=0;i<12;i=i+1) begin

@(posedge CLK);

if (i == 0 | i == 4) begin

#(Tcyc/2) ADSP_ <= 0;

if (i == 0) A = addr1;

if (i == 4) A = addr2;

end

else #(Tcyc/2) ADSP_ <= 1;

if (i >= 1 && i <=10) OE_ = 0;

else OE_ = 1;

if (i >= 1 && i <= 3 || i >= 5 && i<= 7) ADV_ <= 0;

else ADV_ <= 1;

end

end

endtask

task write;        //ADSP|ADSC controlled PL|FT - adsp 2cycle/adsc 1cycle write

input  [16:0] addr;

input  [35:0] data;

input  adsp_;

input  adsc_;

input  gw_;

input  ce_;

input  cs1_;

input  cs0;

begin

@( negedge CLK );

A <= #(Tcyc/2-Tsu) addr;

ADSP_ <= #(Tcyc/2-Tsu) adsp_;

ADSC_ <= #(Tcyc/2-Tsu) adsc_;

DQ = 36'hz;

ADV_  = 1;

CE_   <= #(Tcyc/2-Tsu) ce_;

CS1_  <= #(Tcyc/2-Tsu) cs1_;

CS0   <= #(Tcyc/2-Tsu) cs0;

OE_   <= #(Tcyc/2-Tsu) 1;

if (adsp_ == 0)                               // if adsp_ controlled

GW_ = ~gw_;

else if (adsp_ == 1 & adsc_ == 0) begin       // if adsc_ controlled

#(Tcyc/2-Tsu)

GW_ = gw_;

DQ  = data;

if (cs1_ == 0 & cs0 == 1 & ce_ == 0)

Dstore[addr] = data;

end

else

DQ  = 36'hz;

@( posedge CLK );

counter = 0;

A <= #Tdh 17'hz;

ADSP_ <= #Tdh 1;

ADSC_ <= #Tdh 1;

//    OE_   <= #Tdh 1;

CE_   <= #Tdh ~ce_;

CS1_  <= #Tdh ~cs1_;

CS0   <= #Tdh ~cs0;

if (adsp_ == 0) begin                         // if adsp controlled

#(Tcyc - Tsu);

GW_ = gw_;

DQ = data;

GW_ <= #(Tsu + Tdh) ~gw_;

DQ  <= #(Tsu + Tdh) 36'hz;

if (cs1_ == 0 & cs0 == 1 & ce_ == 0)

Dstore[addr] = data;

end

else if (adsp_ == 1 & adsc_ == 0) begin       // if adsc_ controlled

GW_ <= #Tdh ~gw_;

DQ  <= #Tdh 36'hz;

end

else

DQ  = 36'hz;

end

endtask

task burst_write;     //ADSP&ADSC controlled PL|FT - adsp_ 2-1-1-1/adsc_ 1-1-1-1 write

input  [16:0] addr;

input  [35:0] data;

input  adsp_;

input  adsc_;

input  gw_;

input  ce_;

input  cs1_;

input  cs0;

input  [3:0] nburst;

integer tempaddr,tempcounter;

begin

tempcounter = 0;

for (tempaddr=addr; tempaddr<addr+nburst; tempaddr=tempaddr+1) begin

@( negedge CLK );

DQ = 36'hz;

if (tempaddr == addr) begin

A <= #(Tcyc/2-Tsu) addr;

ADSP_ <= #(Tcyc/2-Tsu) adsp_;

ADSC_ <= #(Tcyc/2-Tsu) adsc_;

ADV_   = 1;

CE_   <= #(Tcyc/2-Tsu) ce_;

CS1_  <= #(Tcyc/2-Tsu) cs1_;

CS0   <= #(Tcyc/2-Tsu) cs0;

if (adsp_ == 0) begin                        // if adsp_ controlled

ADV_ = 1;

GW_ = ~gw_;

end

else if (adsp_ == 1 & adsc_ == 0) begin       // if adsc_ controlled

#(Tcyc/2-Tsu);

GW_ = gw_;

DQ  = data;

if (cs1_ == 0 & cs0 == 1 & ce_ == 0)

Dstore[tempaddr] = data;

end

else

DQ  = 36'hz;

end

else begin                                       // burst after 2nd cycle

ADSP_ = 1;

ADSC_ = 1;

#(Tcyc/2-Tsu);

GW_ = gw_;

data = data+1;

DQ  = data;

if (cs1_ == 0 & cs0 == 1 & ce_ == 0)

Dstore[tempaddr] = data;

if (tempcounter == 0) ADV_ = 1;

else ADV_ = 0;

end

@( posedge CLK );

counter = 0;

if (tempaddr == addr) begin

A <= #Tdh 17'hz;

ADSP_ <= #Tdh 1;

ADSC_ <= #Tdh 1;

OE_   <= #Tdh 1;

CE_   <= #Tdh ~ce_;

CS1_  <= #Tdh ~cs1_;

CS0   <= #Tdh ~cs0;

if (adsp_ == 0) begin                       // if adsp_ controlled

#(Tcyc - Tsu);

GW_ = gw_;

DQ = data;

ADV_ <= #(Tsu + Tdh) 1;

GW_  <= #(Tsu + Tdh) ~gw_;

DQ   <= #(Tsu + Tdh) 36'hz;

if (cs1_ == 0 & cs0 == 1 & ce_ == 0)

Dstore[tempaddr] = data;

if (tempcounter == 0) ADV_ = 1;

else ADV_ = 0;

end

else if (adsp_ == 1 & adsc_ == 0) begin       // if adsc_ controlled

ADV_ <= #Tdh 1;

GW_  <= #Tdh ~gw_;

DQ   <= #Tdh 36'hz;

end

else

DQ  = 36'hz;

end

else begin                                        // burst after 2nd cycle

ADV_ <= #Tdh 1;

GW_  <= #Tdh ~gw_;

DQ   <= #Tdh 36'hz;

end

tempcounter = tempcounter+1;

end

end

endtask

task burst_write_adv;   //ADSP|ADSC controlled PL|FT - adsp_ 2-1-1-1/adsc_ 1-1-1-1 write

input  [16:0] addr;

input  [35:0] data;

input  adsp_;

input  adsc_;

input  gw_;

input  ce_;

input  cs1_;

input  cs0;

input  adv_;

input  [3:0] tempcounter;

begin

@( negedge CLK );

DQ = 36'hz;

if (tempcounter == 0) begin

A <= #(Tcyc/2-Tsu) addr;

ADSP_ <= #(Tcyc/2-Tsu) adsp_;

ADSC_ <= #(Tcyc/2-Tsu) adsc_;

ADV_   = adv_;

CE_   <= #(Tcyc/2-Tsu) ce_;

CS1_  <= #(Tcyc/2-Tsu) cs1_;

CS0   <= #(Tcyc/2-Tsu) cs0;

if (adsp_ == 0) begin                        // if adsp_ controlled

ADV_ = adv_;

GW_ = ~gw_;

end

else if (adsp_ == 1 & adsc_ == 0) begin       // if adsc_ controlled

#(Tcyc/2-Tsu);

GW_ = gw_;

DQ  = data;

if (cs1_ == 0 & cs0 == 1 & ce_ == 0)

Dstore[addr] = data;

end

else

DQ  = 36'hz;

end

else begin                                       // burst after 2nd cycle

ADSP_ = 1;

ADSC_ = 1;

#(Tcyc/2-Tsu);

GW_ = gw_;

ADV_ = adv_;

DQ  = data;

if (cs1_ == 0 & cs0 == 1 & ce_ == 0)

Dstore[addr] = data;

end

@( posedge CLK );

counter = 0;

if (tempcounter == 0) begin

A <= #Tdh 17'hz;

ADSP_ <= #Tdh 1;

ADSC_ <= #Tdh 1;

OE_   <= #Tdh 1;

CE_   <= #Tdh ~ce_;

CS1_  <= #Tdh ~cs1_;

CS0   <= #Tdh ~cs0;

if (adsp_ == 0) begin                       // if adsp_ controlled

#(Tcyc - Tsu);

GW_ = gw_;

DQ = data;

ADV_ <= #(Tsu + Tdh) ~adv_;

GW_  <= #(Tsu + Tdh) ~gw_;

DQ   <= #(Tsu + Tdh) 36'hz;

if (cs1_ == 0 & cs0 == 1 & ce_ == 0)

Dstore[addr] = data;

ADV_ = adv_;

end

else if (adsp_ == 1 & adsc_ == 0) begin       // if adsc_ controlled

ADV_ <= #Tdh 1;

GW_  <= #Tdh ~gw_;

DQ   <= #Tdh 36'hz;

end

else

DQ  = 36'hz;

end

else begin                                        // burst after 2nd cycle

ADV_ <= #Tdh 1;

GW_  <= #Tdh ~gw_;

DQ   <= #Tdh 36'hz;

end

end

endtask

task write_random;

input  [16:0] addr;

input  [35:0] data;

begin

if ( CLK )

@( negedge CLK );

OE_ = 1;

ADV_  = 1;

A = addr;

ADSP_ = 0;

@( negedge CLK );

ADSP_ = 1;

GW_ = 0;

#(Tcyc/2-Tsu) DQ = data;

@( posedge CLK );

#Tdh

DQ = 36'hz;

@( negedge CLK );

GW_ = 1;

end

endtask

task burst_write_random;

input  [16:0] addr;

input  [35:0] n;

integer       i;

begin

if ( CLK )

@( negedge CLK );

#1 A = addr;

ADSP_ = 0;

for (i=addr;i<addr+n;i=i+1) begin

@( negedge CLK );

ADSP_ = 1;

if (addr!=i) ADV_  = 0;

#(Tcyc/2-Tsu) DQ = BurstData[i];

@( posedge CLK );

end

@( negedge CLK );

ADV_  = 1;

end

endtask

task byte_write_random;

input  [16:0] addr;

input  [35:0] data;

begin

if ( CLK )

@( negedge CLK );

ADV_  = 1;

A = addr;

ADSP_ = 0;

@( negedge CLK );

ADSP_ = 1;

BWE_ = 0;

#(Tcyc/2-Tsu) DQ = data;

@( posedge CLK );

#Tdh

DQ = 36'hz;

@( negedge CLK );

BWE_ = 1;

end

endtask

endmodule

ADSP_ = 1;

if (addr!=i) ADV_  = 0;

#(Tcyc/2-Tsu) DQ = BurstData[i];

@( posedge CLK );

end

@(43434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343434343444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444444rstData[i]);

end

@( negedge CLK );

if (check_data) i=i+1;

if ( ($random & 3) === 2'b11 ) // suspend burst 25% of the time

ADV_ = 1;

else begin

ADV_ = 0;

end

end

OE_   = 1;

ADV_  = 1;

end

endtask

task burst_wrap_random; //checks burst counter wrap-around

input  [16:0] addr;

integer i,j;

begin

DQ = 36'hz;

if ( CLK )

@( negedge CLK );

#1 A = addr;

ADSP_ = 0;

@(posedge CLK);           // Address latched by SRAM, begins internal read

#(Tcyc/2) ADSP_ = 1;

#1 OE_   = 0;

ADV_  = 0;

if (pipe == 1) @(posedge CLK);

for (i=0;i<2;i=i+1) begin

for (j=0;j<4;j=j+1) begin

@( posedge CLK ) begin

if (check_data == 1)

BurstData[j] = {DQPbus[4], DQbus[31:24], DQPbus[3], DQbus[23:16], DQPbus[2], DQbus[15:8], DQPbus[1], DQbus[7:0]};

if ( BurstData[j] !== RandomData[j] && check_data == 1 )

$display("%d task burst_wrap_random read error: Addr %h Exp %h Act %h", $stime, i, RandomData[i], BurstData[i]);

end

end

end

#1 OE_   = 1;

ADV_  = 1;

end

endtask

task burst_rd_pipe_random;

input  [16:0] addr1;

input  [16:0] addr2;

integer       i;

begin

DQ = 36'hz;

for (i=0;i<12;i=i+1) begin

@(posedge CLK);

if (i == 0 | i == 4) begin

#(Tcyc/2) ADSP_ <= 0;

if (i == 0) A = addr1;

if (i == 4) A = addr2;

end

else #(Tcyc/2) ADSP_ <= 1;

if (i >= 1 && i <=10) OE_ = 0;

else OE_ = 1;

if (i >= 1 && i <= 3 || i >= 5 && i<= 7) ADV_ <= 0;

else ADV_ <= 1;

end

end

endtask

task write;        //ADSP|ADSC controlled PL|FT - adsp 2cycle/adsc 1cycle write

input  [16:0] addr;

input  [35:0] data;

input  adsp_;

input  adsc_;

input  gw_;

input  ce_;

input  cs1_;

input  cs0;

begin

@( negedge CLK );

A <= #(Tcyc/2-Tsu) addr;

ADSP_ <= #(Tcyc/2-Tsu) adsp_;

ADSC_ <= #(Tcyc/2-Tsu) adsc_;

DQ = 36'hz;

ADV_  = 1;

CE_   <= #(Tcyc/2-Tsu) ce_;

CS1_  <= #(Tcyc/2-Tsu) cs1_;

CS0   <= #(Tcyc/2-Tsu) cs0;

OE_   <= #(Tcyc/2-Tsu) 1;

if (adsp_ == 0)                               // if adsp_ controlled

GW_ = ~gw_;

else if (adsp_ == 1 & adsc_ == 0) begin       // if adsc_ controlled

#(Tcyc/2-Tsu)

GW_ = gw_;

DQ  = data;

if (cs1_ == 0 & cs0 == 1 & ce_ == 0)

Dstore[addr] = data;

end

else

DQ  = 36'hz;

@( posedge CLK );

counter = 0;

A <= #Tdh 17'hz;

ADSP_ <= #Tdh 1;

ADSC_ <= #Tdh 1;

//    OE_   <= #Tdh 1;

CE_   <= #Tdh ~ce_;

CS1_  <= #Tdh ~cs1_;

CS0   <= #Tdh ~cs0;

if (adsp_ == 0) begin                         // if adsp controlled

#(Tcyc - Tsu);

GW_ = gw_;

DQ = data;

GW_ <= #(Tsu + Tdh) ~gw_;

DQ  <= #(Tsu + Tdh) 36'hz;

if (cs1_ == 0 & cs0 == 1 & ce_ == 0)

Dstore[addr] = data;

end

else if (adsp_ == 1 & adsc_ == 0) begin       // if adsc_ controlled

GW_ <= #Tdh ~gw_;

DQ  <= #Tdh 36'hz;

end

else

DQ  = 36'hz;

end

endtask

task burst_write;     //ADSP&ADSC controlled PL|FT - adsp_ 2-1-1-1/adsc_ 1-1-1-1 write

input  [16:0] addr;

input  [35:0] data;

input  adsp_;

input  adsc_;

input  gw_;

input  ce_;

input  cs1_;

input  cs0;

input  [3:0] nburst;

integer tempaddr,tempcounter;

begin

tempcounter = 0;

for (tempaddr=addr; tempaddr<addr+nburst; tempaddr=tempaddr+1) begin

@( negedge CLK );

DQ = 36'hz;

if (tempaddr == addr) begin

A <= #(Tcyc/2-Tsu) addr;

ADSP_ <= #(Tcyc/2-Tsu) adsp_;

ADSC_ <= #(Tcyc/2-Tsu) adsc_;

ADV_   = 1;

CE_   <= #(Tcyc/2-Tsu) ce_;

CS1_  <= #(Tcyc/2-Tsu) cs1_;

CS0   <= #(Tcyc/2-Tsu) cs0;

if (adsp_ == 0) begin                        // if adsp_ controlled

ADV_ = 1;

GW_ = ~gw_;

end

else if (adsp_ == 1 & adsc_ == 0) begin       // if adsc_ controlled

#(Tcyc/2-Tsu);

GW_ = gw_;

DQ  = data;

if (cs1_ == 0 & cs0 == 1 & ce_ == 0)

Dstore[tempaddr] = data;

end

else

DQ  = 36'hz;

end

else begin                                       // burst after 2nd cycle

ADSP_ = 1;

ADSC_ = 1;

#(Tcyc/2-Tsu);

GW_ = gw_;

data = data+1;

DQ  = data;

if (cs1_ == 0 & cs0 == 1 & ce_ == 0)

Dstore[tempaddr] = data;

if (tempcounter == 0) ADV_ = 1;

else ADV_ = 0;

end

@( posedge CLK );

counter = 0;

if (tempaddr == addr) begin

A <= #Tdh 17'hz;

ADSP_ <= #Tdh 1;

ADSC_ <= #Tdh 1;

OE_   <= #Tdh 1;

CE_   <= #Tdh ~ce_;

CS1_  <= #Tdh ~cs1_;

CS0   <= #Tdh ~cs0;

if (adsp_ == 0) begin                       // if adsp_ controlled

#(Tcyc - Tsu);

GW_ = gw_;

DQ = data;

ADV_ <= #(Tsu + Tdh) 1;

GW_  <= #(Tsu + Tdh) ~gw_;

DQ   <= #(Tsu + Tdh) 36'hz;

if (cs1_ == 0 & cs0 == 1