AS8C8036011111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111/*******************************************************************************

*

*   File Name                 : AS8C8C803601

*   Product                   : AS8C803601

*   Function                  : 256K x 36 pipeline ZBT Static RAM

*   Simulation Tool/Version   : Verilog-XL 2.5

*   Date                      : 12/20/10

*

*   Copyright 2010 Alliance Memory, Inc.

*

*   Revision Notes:   12/20/10  Rev00

*

******************************************************************************/

/***************************************************************

*

*  Alliance Memory, Inc. hereby grants the

*  user of this Verilog/VCS model a non-exclusive, nontransferable

*  license to use this Verilog/VCS model under the following terms.

*  The user is granted this license only to use the Verilog/VCS

*  model and is not granted rights to sell, copy (except as needed

*  to run the IBIS model), rent, lease or sub-license the Verilog/VCS

*  model in whole or in part, or in modified form to anyone. The User

*  may modify the Verilog/VCS model to suit its specific applications,

*  but rights to derivative works and such modifications shall belong

*  to IDT.

*

*  This Verilog/VCS model is provided on an "AS IS" basis and

*  Alliance Memory makes absolutely no warranty with respect to the information

*  contained herein. Alliance Memory DISCLAIMS AND CUSTOMER WAIVES ALL

*  WARRANTIES, EXPRESS AND IMPLIED, INCLUDING WARRANTIES OF

*  MERCHANTABILITY AND FITNESS FOR A PARTICULAR PURPOSE. THE

*  ENTIRE RISK AS TO QUALITY AND PERFORMANCE IS WITH THE

*  USER ACCORDINGLY, IN NO EVENT SHALL IDT BE LIABLE

*   FOR ANY DIRECT OR INDIRECT DAMAGES, WHETHER IN CONTRACT OR

*  TORT, INCLUDING ANY LOST PROFITS OR OTHER INCIDENTAL,

*  CONSEQUENTIAL, EXEMPLARY, OR PUNITIVE DAMAGES ARISING OUT OF

*  THE USE OR APPLICATION OF THE VERILOG/VCS model.  Further,

*  Alliance Memory reserves the right to make changes without notice to any

*  product herein to improve reliability, function, or design.

*  Alliance Memory does not convey any license under patent rights or

*  any other intellectual property rights, including those of

*  third parties. IDT is not obligated to provide maintenance

*  or support for the licensed Verilog/VCS model.

*

***************************************************************/

`timescale 1ns/100ps

module AS8C803601 (A,

adv_ld_,                  // advance (high) / load (low)

bw1_, bw2_, bw3_, bw4_,   // byte write enables (low)

ce1_, ce2, ce2_,          // chip enables

cen_,                     // clock enable (low)

clk,                      // clock

IO, IOP,                  // data bus

lbo_,                     // linear burst order (low)

oe_,                      // output enable (low)

r_w_);                    // read (high) / write (low)

initial

begin

$write("\n********************************************************\n");

$write("   AS8C803601, 256K x 36 Pipelined burst ZBT SRAM    \n");

$write("   Rev: 00   July 2000                                    \n");

$write("   copyright 1997,1998,1999,2000 by Alliance Memory, Inc.             \n");

$write("********************************************************\n\n");

end

input [17:0] A;

inout [31:0] IO;

inout [4:1]  IOP;

input adv_ld_, bw1_, bw2_, bw3_, bw4_, ce1_, ce2, ce2_,

cen_, clk, lbo_, oe_, r_w_;

//internal registers for data, address, etc

reg [8:0] mem1[0:262143];    //memory array

reg [8:0] mem2[0:262143];    //memory array

reg [8:0] mem3[0:262143];    //memory array

reg [8:0] mem4[0:262143];    //memory array

reg [35:0] dout;

reg [17:0] addr_a,

addr_b;

reg  wren_a, wren_b;

reg  cs_a,   cs_b;

reg  bw_a1, bw_b1;

reg  bw_a2, bw_b2;

reg  bw_a3, bw_b3;

reg  bw_a4, bw_b4;

reg  [1:0] brst_cnt;

wire[35:0] data_out;

wire doe;

wire cs = (~ce1_ & ce2 & ~ce2_);

wire baddr0, baddr1;

parameter regdelay = 0.2;

parameter outdly   = 0.2;

specify

specparam

//Clock Parameters

tCYC  = 10,    //clock cycle time

tCH   = 3.2,   //clock high time

tCL   = 3.2,   //clock low time

//Output Parameters

tCD   = 5.0,       //clk to data valid

tCLZ  = 1.5,       //clk to output Low-Z

tCHZ  = 3.3,       //clk to data Hi-Z

tOE   = 5.0,       //OE to output valid

tOLZ  = 0.0,       //OE to output Hi-Z

tOHZ  = 5.0,       //OE to output Hi-Z

//Set up times

tSE   = 2.0,       //clock enable set-up

tSA   = 2.0,       //address set-up

tSD   = 2.0,       //data set-up

tSW   = 2.0,       //Read/Write set-up

tSADV = 2.0,       //Advance/Load set-up

tSC   = 2.0,       //Chip enable set-up

tSB   = 2.0,       //Byte write enable set-up

//Hold times

tHE   = 0.5,       //clock enable hold

tHA   = 0.5,       //address hold

tHD   = 0.5,       //data hold

tHW   = 0.5,       //Read/Write hold

tHADV = 0.5,       //Advance/Load hold

tHC   = 0.5,       //Chip enable hold

tHB   = 0.5;       //Byte write enable hold

(oe_ *> IO) = (tOE,tOE,tOHZ,tOLZ,tOHZ,tOLZ); //(01,10,0z,z1,1z,z0)

(clk *> IO) = (tCD,tCD,tCHZ,tCLZ,tCHZ,tCLZ); //(01,10,0z,z1,1z,z0)

(oe_ *> IOP) = (tOE,tOE,tOHZ,tOLZ,tOHZ,tOLZ);  //(01,10,0z,z1,1z,z0)

(clk *> IOP) = (tCD,tCD,tCHZ,tCLZ,tCHZ,tCLZ);  //(01,10,0z,z1,1z,z0)

//timing checks

$period(posedge clk, tCYC );

$width (posedge clk, tCH );

$width (negedge clk, tCL );

$setuphold(posedge clk, A, tSA, tHA);

$setuphold(posedge clk, IO, tSD, tHD);

$setuphold(posedge clk, IOP, tSD, tHD);

$setuphold(posedge clk, adv_ld_, tSADV, tHADV);

$setuphold(posedge clk, bw1_, tSB, tHB);

$setuphold(posedge clk, bw2_, tSB, tHB);

$setuphold(posedge clk, bw3_, tSB, tHB);

$setuphold(posedge clk, bw4_, tSB, tHB);

$setuphold(posedge clk, ce1_, tSC, tHC);

$setuphold(posedge clk, ce2, tSC, tHC);

$setuphold(posedge clk, ce2_, tSC, tHC);

$setuphold(posedge clk, cen_, tSE, tHE);

$setuphold(posedge clk, r_w_, tSW, tHW);

endspecify

initial begin

cs_a = 0;

cs_b = 0;

end

/////////////////////////////////////////////////////////////////////////

//input registers

//--------------------

always @(posedge clk)

begin

if ( ~cen_ & ~adv_ld_ ) cs_a <= #regdelay cs;

if ( ~cen_ )            cs_b <= #regdelay cs_a;

if ( ~cen_ & ~adv_ld_ ) wren_a <= #regdelay (cs & ~r_w_);

if ( ~cen_ )            wren_b <= #regdelay wren_a;

if ( ~cen_ ) bw_a1 <= #regdelay ~bw1_;

if ( ~cen_ ) bw_a2 <= #regdelay ~bw2_;

if ( ~cen_ ) bw_a3 <= #regdelay ~bw3_;

if ( ~cen_ ) bw_a4 <= #regdelay ~bw4_;

if ( ~cen_ ) bw_b1 <= #regdelay bw_a1;

if ( ~cen_ ) bw_b2 <= #regdelay bw_a2;

if ( ~cen_ ) bw_b3 <= #regdelay bw_a3;

if ( ~cen_ ) bw_b4 <= #regdelay bw_a4;

if ( ~cen_ & ~adv_ld_ ) addr_a[17:0] <= #regdelay A[17:0];

if ( ~cen_ )   addr_b[17:0] <= #regdelay {addr_a[17:2], baddr1, baddr0};

end

/////////////////////////////////////////////////////////////////////////

//burst counter

//--------------------

always @(posedge clk)

begin

if      ( lbo_ & ~cen_ & ~adv_ld_) brst_cnt <= #regdelay 0;

else if (~lbo_ & ~cen_ & ~adv_ld_) brst_cnt <= #regdelay A[1:0];

else if (        ~cen_ &  adv_ld_) brst_cnt <= #regdelay brst_cnt + 1;

end

/////////////////////////////////////////////////////////////////////////

//address logic

//--------------------

assign baddr1 = lbo_ ? (brst_cnt[1] ^ addr_a[1]) : brst_cnt[1];

assign baddr0 = lbo_ ? (brst_cnt[0] ^ addr_a[0]) : brst_cnt[0];

/////////////////////////////////////////////////////////////////////////

//data output register

//--------------------

always @(posedge clk)

begin

#regdelay;

#regdelay;

dout[8:0]     = mem1[addr_b];

dout[17:9]    = mem2[addr_b];

dout[26:18]   = mem3[addr_b];

dout[35:27]   = mem4[addr_b];

end

assign data_out = dout;

/////////////////////////////////////////////////////////////////////////

//Output buffers: using a bufif1 has the same effect as...

//

//	assign D = doe ? data_out : 36'hz;

//

//It was coded this way to support SPECIFY delays in the specparam section.

//--------------------

bufif1 #outdly (IO[0],data_out[0],doe);

bufif1 #outdly (IO[1],data_out[1],doe);

bufif1 #outdly (IO[2],data_out[2],doe);

bufif1 #outdly (IO[3],data_out[3],doe);

bufif1 #outdly (IO[4],data_out[4],doe);

bufif1 #outdly (IO[5],data_out[5],doe);

bufif1 #outdly (IO[6],data_out[6],doe);

bufif1 #outdly (IO[7],data_out[7],doe);

bufif1 #outdly (IOP[1],data_out[8],doe);

bufif1 #outdly (IO[8],data_out[9],doe);

bufif1 #outdly (IO[9],data_out[10],doe);

bufif1 #outdly (IO[10],data_out[11],doe);

bufif1 #outdly (IO[11],data_out[12],doe);

bufif1 #outdly (IO[12],data_out[13],doe);

bufif1 #outdly (IO[13],data_out[14],doe);

bufif1 #outdly (IO[14],data_out[15],doe);

bufif1 #outdly (IO[15],data_out[16],doe);

bufif1 #outdly (IOP[2],data_out[17],doe);

bufif1 #outdly (IO[16],data_out[18],doe);

bufif1 #outdly (IO[17],data_out[19],doe);

bufif1 #outdly (IO[18],data_out[20],doe);

bufif1 #outdly (IO[19],data_out[21],doe);

bufif1 #outdly (IO[20],data_out[22],doe);

bufif1 #outdly (IO[21],data_out[23],doe);

bufif1 #outdly (IO[22],data_out[24],doe);

bufif1 #outdly (IO[23],data_out[25],doe);

bufif1 #outdly (IOP[3],data_out[26],doe);

bufif1 #outdly (IO[24],data_out[27],doe);

bufif1 #outdly (IO[25],data_out[28],doe);

bufif1 #outdly (IO[26],data_out[29],doe);

bufif1 #outdly (IO[27],data_out[30],doe);

bufif1 #outdly (IO[28],data_out[31],doe);

bufif1 #outdly (IO[29],data_out[32],doe);

bufif1 #outdly (IO[30],data_out[33],doe);

bufif1 #outdly (IO[31],data_out[34],doe);

bufif1 #outdly (IOP[4],data_out[35],doe);

assign  doe = cs_b & ~wren_b & ~oe_ ;

/////////////////////////////////////////////////////////////////////////

// write to ram

//-------------

always @(posedge clk)

begin

if (wren_b & bw_b1 & ~cen_) mem1[addr_b] = {IOP[1], IO[7:0]};

if (wren_b & bw_b2 & ~cen_) mem2[addr_b] = {IOP[2], IO[15:8]};

if (wren_b & bw_b3 & ~cen_) mem3[addr_b] = {IOP[3], IO[23:16]};

if (wren_b & bw_b4 & ~cen_) mem4[addr_b] = {IOP[4], IO[31:24]};

end

endmodule

666666666666666666666666666666666666666666666666666666666666666666666666666666666666666666666666666666666666666666666666666666666666666666666666666666666666666666666666/*******************************************************************************

*

***************************************************************/

`timescale 1ns/100ps

module AS8C803601 (A,

adv_ld_,                  // advance (high) / load (low)

bw1_, bw2_, bw3_, bw4_,   // byte write enables (low)

ce1_, ce2, ce2_,          // chip enables

cen_,                     // clock enable (low)

clk,                      // clock

IO, IOP,                  // data bus

lbo_,                     // linear burst order (low)

oe_,                      // output enable (low)

r_w_);                    // read (high) / write (low)

initial

begin

$write("\n********************************************************\n");

$write("   AS8C803601, 256K x 36 Pipelined burst ZBT SRAM    \n");

$write("   Rev: 00   Dec  2010                                    \n");

$write("   copyright 1997,1998,1999,2000 by Alliance Memory, Inc.             \n");

$write("********************************************************\n\n");

end

input [17:0] A;

inout [31:0] IO;

inout [4:1]  IOP;

input adv_ld_, bw1_, bw2_, bw3_, bw4_, ce1_, ce2, ce2_,

cen_, clk, lbo_, oe_, r_w_;

//internal registers for data, address, etc

reg [8:0] mem1[0:262143];    //memory array

reg [8:0] mem2[0:262143];    //memory array

reg [8:0] mem3[0:262143];    //memory array

reg [8:0] mem4[0:262143];    //memory array

reg [35:0] dout;

reg [17:0] addr_a,

addr_b;

reg  wren_a, wren_b;

reg  cs_a,   cs_b;

reg  bw_a1, bw_b1;

reg  bw_a2, bw_b2;

reg  bw_a3, bw_b3;

reg  bw_a4, bw_b4;

reg  [1:0] brst_cnt;

wire[35:0] data_out;

wire doe;

wire cs = (~ce1_ & ce2 & ~ce2_);

wire baddr0, baddr1;

parameter regdelay = 0.2;

parameter outdly   = 0.2;

specify

specparam

//Clock Parameters

tCYC  = 7.5,       //clock cycle time

tCH   = 2.2,       //clock high time

tCL   = 2.2,       //clock low time

//Output Parameters

tCD   = 4.2,       //clk to data valid

tCLZ  = 1.5,       //clk to output Low-Z

tCHZ  = 3.0,       //clk to data Hi-Z

tOE   = 4.2,       //OE to output valid

tOLZ  = 0.0,       //OE to output Hi-Z

tOHZ  = 4.2,       //OE to output Hi-Z

//Set up times

tSE   = 1.7,       //clock enable set-up

tSA   = 1.7,       //address set-up

tSD   = 1.7,       //data set-up

tSW   = 1.7,       //Read/Write set-up

tSADV = 1.7,       //Advance/Load set-up

tSC   = 1.7,       //Chip enable set-up

tSB   = 1.7,       //Byte write enable set-up

//Hold times

tHE   = 0.5,       //clock enable hold

tHA   = 0.5,       //address hold

tHD   = 0.5,       //data hold

tHW   = 0.5,       //Read/Write hold

tHADV = 0.5,       //Advance/Load hold

tHC   = 0.5,       //Chip enable hold

tHB   = 0.5;       //Byte write enable hold

(oe_ *> IO) = (tOE,tOE,tOHZ,tOLZ,tOHZ,tOLZ); //(01,10,0z,z1,1z,z0)

(clk *> IO) = (tCD,tCD,tCHZ,tCLZ,tCHZ,tCLZ); //(01,10,0z,z1,1z,z0)

(oe_ *> IOP) = (tOE,tOE,tOHZ,tOLZ,tOHZ,tOLZ);  //(01,10,0z,z1,1z,z0)

(clk *> IOP) = (tCD,tCD,tCHZ,tCLZ,tCHZ,tCLZ);  //(01,10,0z,z1,1z,z0)

//timing checks

$period(posedge clk, tCYC );

$width (posedge clk, tCH );

$width (negedge clk, tCL );

$setuphold(posedge clk, A, tSA, tHA);

$setuphold(posedge clk, IO, tSD, tHD);

$setuphold(posedge clk, IOP, tSD, tHD);

$setuphold(posedge clk, adv_ld_, tSADV, tHADV);

$setuphold(posedge clk, bw1_, tSB, tHB);

$setuphold(posedge clk, bw2_, tSB, tHB);

$setuphold(posedge clk, bw3_, tSB, tHB);

$setuphold(posedge clk, bw4_, tSB, tHB);

$setuphold(posedge clk, ce1_, tSC, tHC);

$setuphold(posedge clk, ce2, tSC, tHC);

$setuphold(posedge clk, ce2_, tSC, tHC);

$setuphold(posedge clk, cen_, tSE, tHE);

$setuphold(posedge clk, r_w_, tSW, tHW);

endspecify

initial begin

cs_a = 0;

cs_b = 0;

end

/////////////////////////////////////////////////////////////////////////

//input registers

//--------------------

always @(posedge clk)

begin

if ( ~cen_ & ~adv_ld_ ) cs_a <= #regdelay cs;

if ( ~cen_ )            cs_b <= #regdelay cs_a;

if ( ~cen_ & ~adv_ld_ ) wren_a <= #regdelay (cs & ~r_w_);

if ( ~cen_ )            wren_b <= #regdelay wren_a;

if ( ~cen_ ) bw_a1 <= #regdelay ~bw1_;

if ( ~cen_ ) bw_a2 <= #regdelay ~bw2_;

if ( ~cen_ ) bw_a3 <= #regdelay ~bw3_;

if ( ~cen_ ) bw_a4 <= #regdelay ~bw4_;

if ( ~cen_ ) bw_b1 <= #regdelay bw_a1;

if ( ~cen_ ) bw_b2 <= #regdelay bw_a2;

if ( ~cen_ ) bw_b3 <= #regdelay bw_a3;

if ( ~cen_ ) bw_b4 <= #regdelay bw_a4;

if ( ~cen_ & ~adv_ld_ ) addr_a[17:0] <= #regdelay A[17:0];

if ( ~cen_ )   addr_b[17:0] <= #regdelay {addr_a[17:2], baddr1, baddr0};

end

/////////////////////////////////////////////////////////////////////////

//burst counter

//--------------------

always @(posedge clk)

begin

if      ( lbo_ & ~cen_ & ~adv_ld_) brst_cnt <= #regdelay 0;

else if (~lbo_ & ~cen_ & ~adv_ld_) brst_cnt <= #regdelay A[1:0];

else if (        ~cen_ &  adv_ld_) brst_cnt <= #regdelay brst_cnt + 1;

end

/////////////////////////////////////////////////////////////////////////

//address logic

//--------------------

assign baddr1 = lbo_ ? (brst_cnt[1] ^ addr_a[1]) : brst_cnt[1];

assign baddr0 = lbo_ ? (brst_cnt[0] ^ addr_a[0]) : brst_cnt[0];

/////////////////////////////////////////////////////////////////////////

//data output register

//--------------------

always @(posedge clk)

begin

#regdelay;

#regdelay;

dout[8:0]     = mem1[addr_b];

dout[17:9]    = mem2[addr_b];

dout[26:18]   = mem3[addr_b];

dout[35:27]   = mem4[addr_b];

end

assign data_out = dout;

/////////////////////////////////////////////////////////////////////////

//Output buffers: using a bufif1 has the same effect as...

//

//	assign D = doe ? data_out : 36'hz;

//

//It was coded this way to support SPECIFY delays in the specparam section.

//--------------------

bufif1 #outdly (IO[0],data_out[0],doe);

bufif1 #outdly (IO[1],data_out[1],doe);

bufif1 #outdly (IO[2],data_out[2],doe);

bufif1 #outdly (IO[3],data_out[3],doe);

bufif1 #outdly (IO[4],data_out[4],doe);

bufif1 #outdly (IO[5],data_out[5],doe);

bufif1 #outdly (IO[6],data_out[6],doe);

bufif1 #outdly (IO[7],data_out[7],doe);

bufif1 #outdly (IOP[1],data_out[8],doe);

bufif1 #outdly (IO[8],data_out[9],doe);

bufif1 #outdly (IO[9],data_out[10],doe);

bufif1 #outdly (IO[10],data_out[11],doe);

bufif1 #outdly (IO[11],data_out[12],doe);

bufif1 #outdly (IO[12],data_out[13],doe);

bufif1 #outdly (IO[13],data_out[14],doe);

bufif1 #outdly (IO[14],data_out[15],doe);

bufif1 #outdly (IO[15],data_out[16],doe);

bufif1 #outdly (IOP[2],data_out[17],doe);

bufif1 #outdly (IO[16],data_out[18],doe);

bufif1 #outdly (IO[17],data_out[19],doe);

bufif1 #outdly (IO[18],data_out[20],doe);

bufif1 #outdly (IO[19],data_out[21],doe);

bufif1 #outdly (IO[20],data_out[22],doe);

bufif1 #outdly (IO[21],data_out[23],doe);

bufif1 #outdly (IO[22],data_out[24],doe);

bufif1 #outdly (IO[23],data_out[25],doe);

bufif1 #outdly (IOP[3],data_out[26],doe);

bufif1 #outdly (IO[24],data_out[27],doe);

bufif1 #outdly (IO[25],data_out[28],doe);

bufif1 #outdly (IO[26],data_out[29],doe);

bufif1 #outdly (IO[27],data_out[30],doe);

bufif1 #outdly (IO[28],data_out[31],doe);

bufif1 #outdly (IO[29],data_out[32],doe);

bufif1 #outdly (IO[30],data_out[33],doe);

bufif1 #outdly (IO[31],data_out[34],doe);

bufif1 #outdly (IOP[4],data_out[35],doe);

assign  doe = cs_b & ~wren_b & ~oe_ ;

/////////////////////////////////////////////////////////////////////////

// write to ram

//-------------

always @(posedge clk)

begin

if (wren_b & bw_b1 & ~cen_) mem1[addr_b] = {IOP[1], IO[7:0]};

if (wren_b & bw_b2 & ~cen_) mem2[addr_b] = {IOP[2], IO[15:8]};

if (wren_b & bw_b3 & ~cen_) mem3[addr_b] = {IOP[3], IO[23:16]};

if (wren_b & bw_b4 & ~cen_) mem4[addr_b] = {IOP[4], IO[31:24]};

end

endmodule

1111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111111 *

***************************************************************/

`timescale 1ns/100ps

module AS8C803601 (A,

adv_ld_,                  // advance (high) / load (low)

bw1_, bw2_, bw3_, bw4_,   // byte write enables (low)

ce1_, ce2, ce2_,          // chip enables

cen_,                     // clock enable (low)

clk,                      // clock

IO, IOP,                  // data bus

lbo_,                     // linear burst order (low)

oe_,                      // output enable (low)

r_w_);                    // read (high) / write (low)

initial

begin

$write("\n********************************************************\n");

$write("   AS8C803601, 256K x 36 Pipelined burst ZBT SRAM    \n");

$write("   Rev: 00   Dec 2010                                    \n");

$write("   copyright 1997,1998,1999,2000 by Alliance Memory, Inc.             \n");

$write("********************************************************\n\n");

end

input [17:0] A;

inout [31:0] IO;

inout [4:1]  IOP;

input adv_ld_, bw1_, bw2_, bw3_, bw4_, ce1_, ce2, ce2_,

cen_, clk, lbo_, oe_, r_w_;

//internal registers for data, address, etc

reg [8:0] mem1[0:262143];    //memory array

reg [8:0] mem2[0:262143];    //memory array

reg [8:0] mem3[0:262143];    //memory array

reg [8:0] mem4[0:262143];    //memory array

reg [35:0] dout;

reg [17:0] addr_a,

addr_b;

reg  wren_a, wren_b;

reg  cs_a,   cs_b;

reg  bw_a1, bw_b1;

reg  bw_a2, bw_b2;

reg  bw_a3, bw_b3;

reg  bw_a4, bw_b4;

reg  [1:0] brst_cnt;

wire[35:0] data_out;

wire doe;

wire cs = (~ce1_ & ce2 & ~ce2_);

wire baddr0, baddr1;

parameter regdelay = 0.2;

parameter outdly   = 0.2;

specify

specparam

//Clock Parameters

tCYC  = 6.7,       //clock cycle time

tCH   = 2.0,       //clock high time

tCL   = 2.0,       //clock low time

//Output Parameters

tCD   = 3.8,       //clk to data valid

tCLZ  = 1.5,       //clk to output Low-Z

tCHZ  = 3.0,       //clk to data Hi-Z

tOE   = 3.8,       //OE to output valid

tOLZ  = 0.0,       //OE to output Hi-Z

tOHZ  = 3.8,       //OE to output Hi-Z

//Set up times

tSE   = 1.5,       //clock enable set-up

tSA   = 1.5,       //address set-up

tSD   = 1.5,       //data set-up

tSW   = 1.5,       //Read/Write set-up

tSADV = 1.5,       //Advance/Load set-up

tSC   = 1.5,       //Chip enable set-up

tSB   = 1.5,       //Byte write enable set-up

//Hold times

tHE   = 0.5,       //clock enable hold

tHA   = 0.5,       //address hold

tHD   = 0.5,       //data hold

tHW   = 0.5,       //Read/Write hold

tHADV = 0.5,       //Advance/Load hold

tHC   = 0.5,       //Chip enable hold

tHB   = 0.5;       //Byte write enable hold

(oe_ *> IO) = (tOE,tOE,tOHZ,tOLZ,tOHZ,tOLZ); //(01,10,0z,z1,1z,z0)

(clk *> IO) = (tCD,tCD,tCHZ,tCLZ,tCHZ,tCLZ); //(01,10,0z,z1,1z,z0)

(oe_ *> IOP) = (tOE,tOE,tOHZ,tOLZ,tOHZ,tOLZ);  //(01,10,0z,z1,1z,z0)

(clk *> IOP) = (tCD,tCD,tCHZ,tCLZ,tCHZ,tCLZ);  //(01,10,0z,z1,1z,z0)

//timing checks

$period(posedge clk, tCYC );

$width (posedge clk, tCH );

$width (negedge clk, tCL );

$setuphold(posedge clk, A, tSA, tHA);

$setuphold(posedge clk, IO, tSD, tHD);

$setuphold(posedge clk, IOP, tSD, tHD);

$setuphold(posedge clk, adv_ld_, tSADV, tHADV);

$setuphold(posedge clk, bw1_, tSB, tHB);

$setuphold(posedge clk, bw2_, tSB, tHB);

$setuphold(posedge clk, bw3_, tSB, tHB);

$setuphold(posedge clk, bw4_, tSB, tHB);

$setuphold(posedge clk, ce1_, tSC, tHC);

$setuphold(posedge clk, ce2, tSC, tHC);

$setuphold(posedge clk, ce2_, tSC, tHC);

$setuphold(posedge clk, cen_, tSE, tHE);

$setuphold(posedge clk, r_w_, tSW, tHW);

endspecify

initial begin

cs_a = 0;

cs_b = 0;

end

/////////////////////////////////////////////////////////////////////////

//input registers

//--------------------

always @(posedge clk)

begin

if ( ~cen_ & ~adv_ld_ ) cs_a <= #regdelay cs;

if ( ~cen_ )            cs_b <= #regdelay cs_a;

if ( ~cen_ & ~adv_ld_ ) wren_a <= #regdelay (cs & ~r_w_);

if ( ~cen_ )            wren_b <= #regdelay wren_a;

if ( ~cen_ ) bw_a1 <= #regdelay ~bw1_;

if ( ~cen_ ) bw_a2 <= #regdelay ~bw2_;

if ( ~cen_ ) bw_a3 <= #regdelay ~bw3_;

if ( ~cen_ ) bw_a4 <= #regdelay ~bw4_;

if ( ~cen_ ) bw_b1 <= #regdelay bw_a1;

if ( ~cen_ ) bw_b2 <= #regdelay bw_a2;

if ( ~cen_ ) bw_b3 <= #regdelay bw_a3;

if ( ~cen_ ) bw_b4 <= #regdelay bw_a4;

if ( ~cen_ & ~adv_ld_ ) addr_a[17:0] <= #regdelay A[17:0];

if ( ~cen_ )   addr_b[17:0] <= #regdelay {addr_a[17:2], baddr1, baddr0};

end

/////////////////////////////////////////////////////////////////////////

//burst counter

//--------------------

always @(posedge clk)

begin

if      ( lbo_ & ~cen_ & ~adv_ld_) brst_cnt <= #regdelay 0;

else if (~lbo_ & ~cen_ & ~adv_ld_) brst_cnt <= #regdelay A[1:0];

else if (        ~cen_ &  adv_ld_) brst_cnt <= #regdelay brst_cnt + 1;

end

/////////////////////////////////////////////////////////////////////////

//address logic

//--------------------

assign baddr1 = lbo_ ? (brst_cnt[1] ^ addr_a[1]) : brst_cnt[1];

assign baddr0 = lbo_ ? (brst_cnt[0] ^ addr_a[0]) : brst_cnt[0];

/////////////////////////////////////////////////////////////////////////

//data output register

//--------------------

always @(posedge clk)

begin

#regdelay;

#regdelay;

dout[8:0]     = mem1[addr_b];

dout[17:9]    = mem2[addr_b];

dout[26:18]   = mem3[addr_b];

dout[35:27]   = mem4[addr_b];

end

assign data_out = dout;

/////////////////////////////////////////////////////////////////////////

//Output buffers: using a bufif1 has the same effect as...

//

//	assign D = doe ? data_out : 36'hz;

//

//It was coded this way to support SPECIFY delays in the specparam section.

//--------------------

bufif1 #outdly (IO[0],data_out[0],doe);

bufif1 #outdly (IO[1],data_out[1],doe);

bufif1 #outdly (IO[2],data_out[2],doe);

bufif1 #outdly (IO[3],data_out[3],doe);

bufif1 #outdly (IO[4],data_out[4],doe);

bufif1 #outdly (IO[5],data_out[5],doe);

bufif1 #outdly (IO[6],data_out[6],doe);

bufif1 #outdly (IO[7],data_out[7],doe);

bufif1 #outdly (IOP[1],data_out[8],doe);

bufif1 #outdly (IO[8],data_out[9],doe);

bufif1 #outdly (IO[9],data_out[10],doe);

bufif1 #outdly (IO[10],data_out[11],doe);

bufif1 #outdly (IO[11],data_out[12],doe);

bufif1 #outdly (IO[12],data_out[13],doe);

bufif1 #outdly (IO[13],data_out[14],doe);

bufif1 #outdly (IO[14],data_out[15],doe);

bufif1 #outdly (IO[15],data_out[16],doe);

bufif1 #outdly (IOP[2],data_out[17],doe);

bufif1 #outdly (IO[16],data_out[18],doe);

bufif1 #outdly (IO[17],data_out[19],doe);

bufif1 #outdly (IO[18],data_out[20],doe);

bufif1 #outdly (IO[19],data_out[21],doe);

bufif1 #outdly (IO[20],data_out[22],doe);

bufif1 #outdly (IO[21],data_out[23],doe);

bufif1 #outdly (IO[22],data_out[24],doe);

bufif1 #outdly (IO[23],data_out[25],doe);

bufif1 #outdly (IOP[3],data_out[26],doe);

bufif1 #outdly (IO[24],data_out[27],doe);

bufif1 #outdly (IO[25],data_out[28],doe);

bufif1 #outdly (IO[26],data_out[29],doe);

bufif1 #outdly (IO[27],data_out[30],doe);

bufif1 #outdly (IO[28],data_out[31],doe);

bufif1 #outdly (IO[29],data_out[32],doe);

bufif1 #outdly (IO[30],data_out[33],doe);

bufif1 #outdly (IO[31],data_out[34],doe);

bufif1 #outdly (IOP[4],data_out[35],doe);

assign  doe = cs_b & ~wren_b & ~oe_ ;

/////////////////////////////////////////////////////////////////////////

// write to ram

//-------------

always @(posedge clk)

begin

if (wren_b & bw_b1 & ~cen_) mem1[addr_b] = {IOP[1], IO[7:0]};

if (wren_b & bw_b2 & ~cen_) mem2[addr_b] = {IOP[2], IO[15:8]};

if (wren_b & bw_b3 & ~cen_) mem3[addr_b] = {IOP[3], IO[23:16]};

if (wren_b & bw_b4 & ~cen_) mem4[addr_b] = {IOP[4], IO[31:24]};

end

endmodule

1616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616IDT71V65603_V/test71v65603.v16161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161600007501600003221600000241600000014641160713616242216001533116016161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616ustar1600arjun161616161616161616161616161616161616161616161616161616users1616161616161616161616161616161616161616161616161616160000000160000000161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616161616

// Test fixture for AS8C803601, 256K x 36, Piplined,

//     Synchronous ZBT SRAM

`timescale 1ns / 10ps

`define Max  17

module main;

parameter Tdh = 0.5;

//Pipelined ZBT synchronous SRAMs - remove comments for model tested

//parameter Tkc =  10.0, Tds = 2.0; `define device idt71v65603s100

//parameter Tkc =  7.5, Tds = 1.7; `define device idt71v65603s133

parameter Tkc =  6.7, Tds = 1.5; `define device idt71v65603s150

reg   [17:0] A, ad_p, ad_pp;

reg          CLK;

reg          WE_, ADV_LD_;

reg   [4:1]  wr_p, wr_pp;

reg   [4:1]  BWE_;

reg          CS_;

reg          CEN_;

reg          OE_;

reg          LBO_;

reg   [35:0] DQ, DQ_p, DQ_pp;

wire  [31:0] DQbus = {DQ[34:27], DQ[25:18], DQ[16:9], DQ[7:0]};

wire  [4:1]  DQPbus = {DQ[35], DQ[26], DQ[17], DQ[8]};

wire  CE1_ = CS_;

wire  CE2  = ~CS_;

wire  CE2_ = CS_;

reg   [8:0] dupdata1[0:`Max-1];

reg   [8:0] dupdata2[0:`Max-1];

reg   [8:0] dupdata3[0:`Max-1];

reg   [8:0] dupdata4[0:`Max-1];

reg   [35:0] expecdata;

reg   [35:0] data;

reg   samp_pp, samp_p;

integer   i, j;

`device ssram (

A,

ADV_LD_,

BWE_[1],

BWE_[2],

BWE_[3],

BWE_[4],

CE1_,

CE2,

CE2_,

CEN_,

CLK,

DQbus,

DQPbus,

LBO_,

OE_,

WE_

);

initial begin

#200000;

$finish;

end

initial begin

$recordfile ("65603test.trn");  // For Signalscan output

$recordvars;  // For Signalscan output

//  $dumpfile ("65603file.vcd");

//  $dumpvars;

end

initial begin

WE_   = 1;

BWE_   = 0;

ADV_LD_ = 0;

CS_   = 1;

CEN_  = 0;

OE_   = 0;

CLK   = 0;

LBO_  = 0;

for (i=0;i<`Max;i=i+1) begin      // Fill part of SRAM with random data

write(i, $random, 'hf);

end

for (i=30;i<50;i=i+1) begin      // This is the test

@( negedge CLK );

j <= ($random & 'h3);

case (j)

0: b_write_cen  (($random & 'hf), $random); // write   random addr, data, burst size

1: b_read_cen (($random & 'hf), $random);          // read    random addr, burst size

2: deselect;

3: clk_disable;

endcase

end

@( posedge CLK );

@( posedge CLK );

@( posedge CLK );

$finish;

end

always @( negedge CLK ) begin

if (wr_p[1]) dupdata1[ad_p] <= DQ_p[8:0];

if (wr_p[2]) dupdata2[ad_p] <= DQ_p[17:9];

if (wr_p[3]) dupdata3[ad_p] <= DQ_p[26:18];

if (wr_p[4]) dupdata4[ad_p] <= DQ_p[35:27];

end

always @( posedge CLK )

if (~CEN_) begin

DQ_p   <= #1 DQ_pp;

ad_p   <= #1 ad_pp;

wr_p   <= #1 wr_pp;

samp_p <= #1 samp_pp;

end

always @( posedge CLK ) begin

#Tdh;

DQ <= 36'hz;

#(Tkc - Tdh - Tds);

if (wr_p && ~CEN_) DQ = DQ_p;

end

always @( posedge CLK )

if (samp_p) begin

data[8:0]   <= {DQPbus[1], DQbus[7:0]};

data[17:9]  <= {DQPbus[2], DQbus[15:8]};

data[26:18] <= {DQPbus[3], DQbus[23:16]};

data[35:27] <= {DQPbus[4], DQbus[31:24]};

expecdata[8:0]   <= dupdata1[ad_p];

expecdata[17:9]  <= dupdata2[ad_p];

expecdata[26:18] <= dupdata3[ad_p];

expecdata[35:27] <= dupdata4[ad_p];

#1;

if ( expecdata !== data ) begin

$display("Time %0d  Read error: Addr %h  Exp %h  Act %h", $stime, ad_p, expecdata, data);

end

else begin

//      $display("Time %0d  Read        Addr %h  Exp %h  Act %h", $stime, ad_p, expecdata, data);

end

end

always #(Tkc/2) CLK <= ~CLK;

task deselect;

begin

if (CLK)

@( negedge CLK );

ADV_LD_ <= 0;

CS_  <= 1;

CEN_ <= 0;

//    WE_  <= $random;

//    A    <= $random;

@( posedge CLK );

wr_pp   <= #1 0;

samp_pp <= #1 0;

# Tdh;

end

endtask

task clk_disable;

begin

if (CLK)

@( negedge CLK );

CEN_   <= 1;

//    ADV_LD_ <= $random;

@( posedge CLK );

# Tdh;

end

endtask

task b_read_cen;

input [17:0] addr;

input [2:0] b_size;

integer j;

begin

j <= 0;

if ( CLK )

@( negedge CLK );

A <= addr;

ADV_LD_ <= 0;

CS_  <= 0;

CEN_ <= 0;

WE_  <= 1;

@( posedge CLK );

ad_pp   <= #1 addr;

wr_pp   <= #1 0;

samp_pp <= #1 1;

#Tdh;

while (j < b_size) begin

ADV_LD_ <= 1;

CEN_ <= $random;

#1;

if (~CEN_) begin

j <= (j+1);

A <= $random;

CS_  <= $random;

WE_  <= $random;

@( posedge CLK );

ad_pp   <= #1 FBA(addr, j);

end

else @( posedge CLK );

#Tdh;

end

end

endtask

function [17:0] FBA; // Function Burst Address

input [17:0] addr;

input [1:0]  j;

begin

if (~LBO_)

FBA = {addr[16:2], (addr[1:0] + j)};

else

FBA = {addr[16:2], (addr[1] ^ j[1]), (addr[0] ^ j[0])};

end

endfunction

function [35:0] FRD; // Function Random Data

input [4:1] bytes;

begin

if (bytes[1])  FRD[8:0] = $random;

else FRD[8:0] = 'hx;

if (bytes[2])  FRD[17:9] = $random;

else FRD[17:9] = 'hx;

if (bytes[3])  FRD[26:18] = $random;

else FRD[26:18] = 'hx;

if (bytes[4])  FRD[35:27] = $random;

else FRD[35:27] = 'hx;

end

endfunction

task write;

input [17:0] addr;

input [35:0] data;

input [4:1] bytes;

reg   [35:0] mod_data;

begin

if ( CLK )

@( negedge CLK );

if (bytes[1])  mod_data[8:0] <= #1 data[8:0];

else mod_data[8:0] <= #1 'hx;

if (bytes[2])  mod_data[17:9] <= #1 data[17:9];

else mod_data[17:9] <= #1 'hx;

if (bytes[3])  mod_data[26:18] <= #1 data[26:18];

else mod_data[26:18] <= #1 'hx;

if (bytes[4])  mod_data[35:27] <= #1 data[35:27];

else mod_data[35:27] <= #1 'hx;

ADV_LD_ <= 0;

A <= addr;

CS_  <= 0;

CEN_ <= 0;

WE_  <= 0;

BWE_ <= ~bytes;

@( posedge CLK );

DQ_pp   <= #1 mod_data;

ad_pp   <= #1 addr;

wr_pp   <= #1 bytes;

samp_pp <= #1 0;

#Tdh;

end

endtask

task b_write_cen;

input [17:0] addr;

input [2:0] b_size;

integer j;

reg   [4:1] bytes;

begin

j <= 0;

if ( CLK )

@( negedge CLK );

bytes <= $random;

ADV_LD_ <= 0;

A <= addr;

CS_  <= 0;

CEN_ <= 0;

WE_  <= 0;

#1;

BWE_ <= ~bytes;

@( posedge CLK );

DQ_pp   <= #1 FRD(bytes);

ad_pp   <= #1 addr;

wr_pp   <= #1 bytes;

samp_pp <= #1 0;

#Tdh;

while (j < b_size) begin

bytes <= $random;

ADV_LD_ <= 1;

CEN_ <= $random;

#1;

if (~CEN_) begin

j <= (j+1);

A <= $random;

CS_  <= $random;

WE_  <= $random;

#1;

BWE_ <= ~bytes;

@( posedge CLK );

DQ_pp   <= #1 FRD(bytes);

ad_pp   <= #1 FBA(addr, j);

wr_pp   <= #1 bytes;

end

else @( posedge CLK );

#Tdh;

end

end

endtask

endmodule

2323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323232323bufif1 #outdly (IO[1],data_out[1],doe);

bufif1 #outdly (IO[2],data_out[2],doe);

bufif1 #outdly (IO[3],data_out[3],doe);

bufif1 #outdly (IO[4],data_out[4],doe);

bufif1 #outdly (IO[5],data_out[5],doe);

bufif1 #outdly (IO[6],data_out[6],doe);

bufif1 #outdly (IO[7],data_out[7],doe);

bufif1 #outdly (IOP[1],data_out[8],doe);

bufif1 #outdly (IO[8],data_out[9],doe);

bufif1 #outdly (IO[9],data_out[10],doe);

bufif1 #outdly (IO[10],data_out[11],doe);

bufif1 #outdly (IO[11],data_out[12],doe);

bufif1 #outdly (IO[12],data_out[13],doe);

bufif1 #outdly (IO[13],data_out[14],doe);

bufif1 #outdly (IO[14],data_out[15],doe);

bufif1 #outdly (IO[15],data_out[16],doe);

bufif1 #outdly (IOP[2],data_out[17],doe);

bufif1 #outdly (IO[16],data_out[18],doe);

bufif1 #outdly (IO[17],data_out[19],doe);

bufif1 #outdly (IO[18],data_out[20],doe);

bufif1 #outdly (IO[19],data_out[21],doe);

bufif1 #outdly (IO[20],data_out[22],doe);

bufif1 #outdly (IO[21],data_out[23],doe);

bufif1 #outdly (IO[22],data_out[24],doe);

bufif1 #outdly (IO[23],data_out[25],doe);

bufif1 #outdly (IOP[3],data_out[26],doe);

bufif1 #outdly (IO[24],data_out[27],doe);

bufif1 #outdly (IO[25],data_out[28],doe);

bufif1 #outdly (IO[26],data_out[29],doe);

bufif1 #outdly (IO[27],data_out[30],doe);

bufif1 #outdly (IO[28],data_out[31],doe);

bufif1 #outdly (IO[29],data_out[32],doe);

bufif1 #outdly (IO[30],data_out[33],doe);

bufif1 #outdly (IO[31],data_out[34],doe);

bufif1 #outdly (IOP[4],data_out[35],doe);

assign  doe = cs_b & ~wren_b & ~oe_ ;

/////////////////////////////////////////////////////////////////////////

// write to ram

//-------------

always @(posedge clk)

begin

if (wren_b & bw_b1 & ~cen_) mem1[addr_b] = {IOP[1], IO[7:0]};

if (wren_b & bw_b2 & ~cen_) mem2[addr_b] = {IOP[2], IO[15:8]};

if (wren_b & bw_b3 & ~cen_) mem3[addr_b] = {IOP[3], IO[23:16]};

if (wren_b & bw_b4 & ~cen_) mem4[addr_b] = {IOP[4], IO[31:24]};

end

endmodule

2424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424AS8C803601_V/test71v65603.v24242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242400007502400003222400000242400000014641240713616242224001533124024242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424ustar2400arjun242424242424242424242424242424242424242424242424242424users2424242424242424242424242424242424242424242424242424240000000240000000242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424242424

// Test fixture for AS8C803601, 256K x 36, Piplined,

//     Synchronous ZBT SRAM

`timescale 1ns / 10ps

`define Max  17

module main;

parameter Tdh = 0.5;

//Pipelined ZBT synchronous SRAMs - remove comments for model tested

//parameter Tkc =  10.0, Tds = 2.0; `define device idt71v65603s100

//parameter Tkc =  7.5, Tds = 1.7; `define device idt71v65603s133

parameter Tkc =  6.7, Tds = 1.5; `define device idt71v65603s150

reg   [17:0] A, ad_p, ad_pp;

reg          CLK;

reg          WE_, ADV_LD_;

reg   [4:1]  wr_p, wr_pp;

reg   [4:1]  BWE_;

reg          CS_;

reg          CEN_;

reg          OE_;

reg          LBO_;

reg   [35:0] DQ, DQ_p, DQ_pp;

wire  [31:0] DQbus = {DQ[34:27], DQ[25:18], DQ[16:9], DQ[7:0]};

wire  [4:1]  DQPbus = {DQ[35], DQ[26], DQ[17], DQ[8]};

wire  CE1_ = CS_;

wire  CE2  = ~CS_;

wire  CE2_ = CS_;

reg   [8:0] dupdata1[0:`Max-1];

reg   [8:0] dupdata2[0:`Max-1];

reg   [8:0] dupdata3[0:`Max-1];

reg   [8:0] dupdata4[0:`Max-1];

reg   [35:0] expecdata;

reg   [35:0] data;

reg   samp_pp, samp_p;

integer   i, j;

`device ssram (

A,

ADV_LD_,

BWE_[1],

BWE_[2],

BWE_[3],

BWE_[4],

CE1_,

CE2,

CE2_,

CEN_,

CLK,

DQbus,

DQPbus,

LBO_,

OE_,

WE_

);

initial begin

#200000;

$finish;

end

initial begin

$recordfile ("65603test.trn");  // For Signalscan output

$recordvars;  // For Signalscan output

//  $dumpfile ("65603file.vcd");

//  $dumpvars;

end

initial begin

WE_   = 1;

BWE_   = 0;

ADV_LD_ = 0;

CS_   = 1;

CEN_  = 0;

OE_   = 0;

CLK   = 0;

LBO_  = 0;

for (i=0;i<`Max;i=i+1) begin      // Fill part of SRAM with random data

write(i, $random, 'hf);

end

for (i=30;i<50;i=i+1) begin      // This is the test

@( negedge CLK );

j <= ($random & 'h3);

case (j)

0: b_write_cen  (($random & 'hf), $random); // write   random addr, data, burst size

1: b_read_cen (($random & 'hf), $random);          // read    random addr, burst size

2: deselect;

3: clk_disable;

endcase

end

@( posedge CLK );

@( posedge CLK );

@( posedge CLK );

$finish;

end

always @( negedge CLK ) begin

if (wr_p[1]) dupdata1[ad_p] <= DQ_p[8:0];

if (wr_p[2]) dupdata2[ad_p] <= DQ_p[17:9];

if (wr_p[3]) dupdata3[ad_p] <= DQ_p[26:18];

if (wr_p[4]) dupdata4[ad_p] <= DQ_p[35:27];

end

always @( posedge CLK )

if (~CEN_) begin

DQ_p   <= #1 DQ_pp;

ad_p   <= #1 ad_pp;

wr_p   <= #1 wr_pp;

samp_p <= #1 samp_pp;

end

always @( posedge CLK ) begin

#Tdh;

DQ <= 36'hz;

#(Tkc - Tdh - Tds);

if (wr_p && ~CEN_) DQ = DQ_p;

end

always @( posedge CLK )

if (samp_p) begin

data[8:0]   <= {DQPbus[1], DQbus[7:0]};

data[17:9]  <= {DQPbus[2], DQbus[15:8]};

data[26:18] <= {DQPbus[3], DQbus[23:16]};

data[35:27] <= {DQPbus[4], DQbus[31:24]};

expecdata[8:0]   <= dupdata1[ad_p];

expecdata[17:9]  <= dupdata2[ad_p];

expecdata[26:18] <= dupdata3[ad_p];

expecdata[35:27] <= dupdata4[ad_p];

#1;

if ( expecdata !== data ) begin

$display("Time %0d  Read error: Addr %h  Exp %h  Act %h", $stime, ad_p, expecdata, data);

end

else begin

//      $display("Time %0d  Read        Addr %h  Exp %h  Act %h", $stime, ad_p, expecdata, data);

end

end

always #(Tkc/2) CLK <= ~CLK;

task deselect;

begin

if (CLK)

@( negedge CLK );

ADV_LD_ <= 0;

CS_  <= 1;

CEN_ <= 0;

//    WE_  <= $random;

//    A    <= $random;

@( posedge CLK );

wr_pp   <= #1 0;

samp_pp <= #1 0;

# Tdh;

end

endtask

task clk_disable;

begin

if (CLK)

@( negedge CLK );

CEN_   <= 1;

//    ADV_LD_ <= $random;

@( posedge CLK );

# Tdh;

end

endtask

task b_read_cen;

input [17:0] addr;

input [2:0] b_size;

integer j;

begin

j <= 0;

if ( CLK )

@( negedge CLK );

A <= addr;

ADV_LD_ <= 0;

CS_  <= 0;

CEN_ <= 0;

WE_  <= 1;

@( posedge CLK );

ad_pp   <= #1 addr;

wr_pp   <= #1 0;

samp_pp <= #1 1;

#Tdh;

while (j < b_size) begin

ADV_LD_ <= 1;

CEN_ <= $random;

#1;

if (~CEN_) begin

j <= (j+1);

A <= $random;

CS_  <= $random;

WE_  <= $random;

@( posedge CLK );

ad_pp   <= #1 FBA(addr, j);

end

else @( posedge CLK );

#Tdh;

end

end

endtask

function [17:0] FBA; // Function Burst Address

input [17:0] addr;

input [1:0]  j;

begin

if (~LBO_)

FBA = {addr[16:2], (addr[1:0] + j)};

else

FBA = {addr[16:2], (addr[1] ^ j[1]), (addr[0] ^ j[0])};

end

endfunction

function [35:0] FRD; // Function Random Data

input [4:1] bytes;

begin

if (bytes[1])  FRD[8:0] = $random;

else FRD[8:0] = 'hx;

if (bytes[2])  FRD[17:9] = $random;

else FRD[17:9] = 'hx;

if (bytes[3])  FRD[26:18] = $random;

else FRD[26:18] = 'hx;

if (bytes[4])  FRD[35:27] = $random;

else FRD[35:27] = 'hx;

end

endfunction

task write;

input [17:0] addr;

input [35:0] data;

input [4:1] bytes;

reg   [35:0] mod_data;

begin

if ( CLK )

@( negedge CLK );

if (bytes[1])  mod_data[8:0] <= #1 data[8:0];

else mod_data[8:0] <= #1 'hx;

if (bytes[2])  mod_data[17:9] <= #1 data[17:9];

else mod_data[17:9] <= #1 'hx;

if (bytes[3])  mod_data[26:18] <= #1 data[26:18];

else mod_data[26:18] <= #1 'hx;

if (bytes[4])  mod_data[35:27] <= #1 data[35:27];

else mod_data[35:27] <= #1 'hx;

ADV_LD_ <= 0;

A <= addr;

CS_  <= 0;

CEN_ <= 0;

WE_  <= 0;

BWE_ <= ~bytes;

@( posedge CLK );

DQ_pp   <= #1 mod_data;

ad_pp   <= #1 addr;

wr_pp   <= #1 bytes;

samp_pp <= #1 0;

#Tdh;

end

endtask

task b_write_cen;

input [17:0] addr;

input [2:0] b_size;

integer j;

reg   [4:1] bytes;

begin

j <= 0;

if ( CLK )

@( negedge CLK );

bytes <= $random;

ADV_LD_ <= 0;

A <= addr;

CS_  <= 0;

CEN_ <= 0;

WE_  <= 0;

#1;

BWE_ <= ~bytes;

@( posedge CLK );

DQ_pp   <= #1 FRD(bytes);

ad_pp   <= #1 addr;

wr_pp   <= #1 bytes;

samp_pp <= #1 0;

#Tdh;

while (j < b_size) begin

bytes <= $rando